| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0101 |
| --- | --- |

| Title: | 3D-CE5: Segment-wise depth inter mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

Segment-wise depth inter mode coding (E0156) was adopted at Vienna meeting and implemented in 3D-HEVC. It encodes one representative value (RV) for each segment or PU instead of the residual for all pixels within a CU. RV is calculated by average difference (DC) between the original and predicted pixels. However, this method is not very efficient in the partitioned PUs such as 2NxN, Nx2N, NxN, and so on. Since the correlation between the residual values in the partitioned PU is very low, DC cannot represent all the residual values. As follow-up of F0149, the proposed method applies the segment-wise depth inter mode coding to only 2Nx2N. Also, in order to find more accurate RV for all the residual, it checks three candidates, for example, DC, DC-1, and DC+1, or more. The best RV is decided and coded through RDO. The results demonstrate that the proposed method achieves the coding gain of -0.31% and -0.46% in the synthesized view, when three and five RV candidates are used, respectively.

# Proposed Segment-Wise Depth Inter Mode Coding

i) The current segment-wise depth inter mode coding encodes one RV for each PU instead of the residual for all pixels within a CU. RV is calculated by DC between the original and predicted pixels (RV=DC). As shown in Fig. 1, the current method is used in all the PUs such as 2Nx2N, 2NxN, Nx2N, NxN, 2NxnU, 2NxnD, nLx2N, and nRx2N.

# Fig. 1. PUs for the segment-wise depth inter mode coding

Unlike 2Nx2N, the correlation between the residual values in the partitioned PUs such as 2NxN, Nx2N, NxN, 2NxnU, 2NxnD, nLx2N, and nRx2N is very low. Therefore, DC cannot represent all the residual values. Based on this observation, the proposed method restricts the segment-wise coding into 2Nx2N.

ii) To find more accurate RV for all the residual, the proposed method checks three different candidates, which consist of DC, DC-1, and DC+1, and decides the best RV through RDO (RV=k, k={DC-1, DC, DC+1}). Fig. 2 shows three different RV candidates. When CU is coded by the segment-wise depth inter mode, the magnitude of RV is coded and then its sign is signaled.

# Fig. 2. Three RV candidates in 2Nx2N

iii) For the further improvement, the proposed method can test more candidates, for example, DC, DC-1, DC+1, DC-2, and DC+2. Then, the encoder decides the best RV through RDO (RV=k, k={ DC-2, DC-1, DC, DC+1, DC+2}). The number of the RV candidates can be decided in an encoder side.

# Proposed Text

H.7.3.8.5 Coding unit syntax

| if( vps_inter_sdc_flag && DepthFlag && PredMode[ x0 ][ y0 ] ! = MODE_INTRA && !skip_flag[ x0 ][ y0 ] && PartMode == PART_2Nx2N) |  |
| --- | --- |
| inter_sdc_flag | ae(v) |
| if( inter_sdc_flag ) { |  |
| inter_sdc_resi_abs[ x0 ][ y0 ] | ae(v) |
| if( inter_sdc_resi_abs[ x0 ][ y0 ] ) { |  |
| inter_sdc_resi_sign_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |

inter_sdc_flag equal to 1 specifies segment-wise depth coding of residual blocks is used for the current coding unit. inter_sdc_flag equal to 0 specifies segment-wise depth coding of residual blocks is not used for the current coding unit. When not present, inter_sdc_flag is inferred to be equal to 0.

inter_sdc_resi_abs [ x0 ][ y0 ], inter_sdc_resi_sign_flag[ x0 ][ y0 ] are used to derive InterSdcResi[ x0 ][ y0 ] as follows:

  - InterSdcResi[ x0 ][ y0 ] = ( 1 − 2 * inter_sdc_resi_sign_flag[ x0 ][ y0 ] ) * 
		( inter_sdc_resi_abs [ x0 ][ y0 ] )	(Error: Reference source not found-1)

- H.8.5.4 Decoding process for the residual signal of coding units coded in inter prediction mode

- H.8.5.4.1 General

- ...

- Otherwise (inter_sdc_flag is equal to 1), resSampleL[x][y] is set equal to InterSdcResi[ xCb ][ yCb ] for x in the range of 0 to nCbSL − 1 and y in the range of 0 to nCbSL − 1

...

-

- – Specification of the variables xOff, yOff, and interSdcResiIdx[ i ]

# Test Results

Test results of the proposed method are summarized in Tables 1 to 3. When the segment-wise depth inter mode coding is applied to 2Nx2N, the very small coding loss of 0.06% is observed as shown in Table 1. However, when three and five RV candidates are additionally employed in 2Nx2N, Tables 2 and 3 show that the proposed method obtains about -0.31% and -0.46% gains in terms of the synthesized PSNR over the total bit rate, respectively.

Table 1. Performance of the restriction to 2Nx2N

Table 2. Performance of the proposed method with three candidates

Table 3. Performance of the proposed method with five candidates

# Conclusion

By integrating the proposed method on 3D-HTM 9.0r1, it improves the coding performance. Hence, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).