| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0102 |
| --- | --- |

| Title: | 3D-CE5 related: Segment-wise depth intra mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, Byeongdoo Choi, Yongjin Cho, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

Segment-wise DC coding (SDC) is used for the efficient depth intra coding in 3D-HEVC. Currently, SDC mode consists of Planar and DMM1 modes, and its partition size is always 2Nx2N. Instead of transform and quantization processes for coding of residual samples, SDC signals one or two constant values, which are the average difference between the original depth values and the predicted depth values. In order to improve the performance of SDC, the generic SDC was proposed at the last meeting (F0126). It enables SDC to be applied to all the intra modes. The generic SDC provides the high coding gain, but it increases the encoding complexity. To reduce the complexity, the proposed method only uses the most probable modes (MPM) and depth modelling modes (DMM) as SDC modes, instead of all the modes. Compared to the generic SDC (Gxxxx), the results illustrate that the proposed method achieves the less gain than 0.2% in both CTC and all intra condition (AI), but it reduces the encoding time of about 20% in AI by reducing the number of the intra modes to be tested in full RDO from 998 to 423 (about -58%) in CTU.

# Proposed Method

At this meeting, the generic SDC (Gxxxx) is planned to be evaluated as a follow-up of F0126. Most parts including the syntax and the decoding processes in the proposed method are based on Gxxxx and the SW is also implemented on the top of SW released by Qualcomm within CE5. Only difference between the proposed method and Gxxxx is how to construct the SDC modes.

The proposed SDC mode uses three MPMs and two DMMs, as shown in Table 1.

Table 1. SDC mode construction

|  | SDC modes |
| --- | --- |
| Proposed | Three MPMs (The first, second, and third most probable modes) Two DMMs (DMM1 and DMM4) |
| Gxxxx | All the modes to be tested in full RDO |

When SDC is applied to only five modes, the total number of the intra modes to be tested in full RDO can be significantly reduced as displayed in Table 2.

Table 2. Number of full RDO in CTU

| CU size | Number of CU in CTU | Total number of full RDO (RMD+MPM+DMM) |  | Reduction |
| --- | --- | --- | --- | --- |
|  |  | Generic (Gxxxx) | Proposed |  |
| 64x64 | 1 | 1*(3+3+0)=6 | 1*(0+3+0)=3 | 50% |
| 32x32 | 4 | 4*(3+3+2)=32 | 4*(0+3+2)=20 | 38% |
| 16x16 | 16 | 16*(3+3+2)=128 | 16*(0+3+2)=80 | 38% |
| 8x8 | 64 | 64*(8+3+2)=832 | 64*(0+3+2)=320 | 62% |
| Total | 85 | 998 | 423 | 58% |

# Test Results

Tables 3 and 4 illustrate the coding performance of the proposed method in CTC and AI, respectively. The proposed method is implemented on the top of SW released by Qualcomm in CE5.

Table 3. Performance of the proposed method in CTC

Table 4. Performance of the proposed method in AI

# Conclusion

By integrating the proposed method on 3D-HTM 9.0r1, it improves the coding performance. Hence, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).