| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0103 |
| --- | --- |

| Title: | 3D-CE5 related: Depth quantization in intra residual domain |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Young Lee, Min Woo Park, Byeongdoo Choi, Yongjin Cho, and Chanyul Kim | Email: | jinyoung79.lee@samsung.com |
| Source: | Samsung Electronics Co. Ltd. |  |  |

_____________________________

# Abstract

DLT-based residual coding (F0159) was adopted for the efficient depth intra coding at the last meeting. It compresses a difference between indexes of original and predicted pixels for several intra modes. Since it reduces a magnitude of the residual, some improvement is achieved. In order to reduce the magnitude of the residual of the remaining intra modes which the DLT-based residual coding is not applied and further improve the intra coding efficiency, this contribution proposes depth quantization in intra residual domain. The proposed method quantizes a difference between original and predicted pixels in the residual domain with a simple shift operation. Then, the quantized residual is transformed to the frequency domain for the further process. The residual quantization parameter can be signalled in VPS or some proper headers. The results illustrate that the proposed method achieves the coding gain of -0.2% and -0.4% in both CTC and all intra (AI) conditions.

# Proposed Method

The DLT-based residual coding (F0159) was adopted at the last meeting. For DC, vertical, horizontal, and DMM modes, it encodes residual between indexes of original and predicted pixels, as follows:

residual = DLT[original] – DLT[predicted]

For the other modes, the conventional residual coding is performed.

residual = original – predicted

Since the magnitude of the intra residual is relatively high, the DLT-based residual coding can reduce the residual size. It results in the depth bit rate saving.

In order to further improve the intra coding efficiency, the proposed method adds the depth quantization in the residual domain before DCT. It quantizes the difference between the original and predicted pixels in the residual domain with a simple shift operation as follows.

residual = (original – predicted) >> resdiualQp

Then, the quantized residual is transformed to the frequency domain for the further process. The residual quantization parameter (resdiualQp) can be signalled in VPS or some proper headers. At a decoder side, a reverse process is performed. The residual after the inverse DCT is de-quantized with the shift operation as follows.

residual <<= resdiualQp

By quantizing the residual in the image domain, the magnitude of the residual can be drastically reduced and remove high frequency components little bit, which makes the data compaction in the frequency domain well.

In the proposed method, the DLT-based residual coding is only applied to the DC, vertical, and horizontal modes and the residual quantization is applied to the other intra modes.

# Proposed Text

H.7.3.2.1.2 Video parameter set extension 2 syntax

…

| if ( layerId ! = 0 ) { |  |
| --- | --- |
| iv_mv_pred_flag[ layerId ] | u(1) |
| log2_sub_pb_size_minus2[ layerId ] | ue(v) |
| if ( !VpsDepthFlag[ layerId ] ) { |  |
| depth_intra_residual_qp[ layerId ] | ue(v) |
| iv_res_pred_flag[ layerId ] | u(1) |
| depth_refinement_flag[ layerId ] | u(1) |
| view_synthesis_pred_flag[ layerId ] | u(1) |
| } else { |  |

…

depth_intra_residual_qp[ layerId ] specifies the value of the variable LesidualQp[ layerId ] that may be used in the decoding of prediction units coded in intra prediction mode.

H.8.4.4.1 General decoding process for intra blocks

...

If DltFlag[ nuh_layer_id ] is equal to 1 and predModeIntra is equal to INTRA_DC, INTRA_ANGULAR10, or INTRA_ANGULAR26, the following applies, for i in the range of 0 to nTbS − 1, inclusive, and j in the range of 0 to nTbS − 1, inclusive:

idx = DepthValue2Idx[ predSamples[ i ][ j ] ] + resSamples[ i ][ j ]		(Error: Reference source not found-1)

SL[ xTb0 + i ][ yTb0 + j ] = 
	Idx2DepthValue[ clip3( 0, NumDepthValuesInDlt[ nuh_layer_id ] – 1, idx ) ]	(Error: Reference source not found-2)

Otherwise, the following applies:

- If depth_intra_residual_qp[ layered ] is not equal to 0, the following applies, for i in the range of 0 to nTbS − 1, inclusive, and j in the range of 0 to nTbS − 1, inclusive:

resSamplies[ i ][ j ] <<= depth_intra_residual_qp[ layered ]

- The picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause 8.6.5 is invoked with the transform block location ( xTb0, yTb0 ), the transform block size nTbS, the variable cIdx, the (nTbS)x(nTbS) array predSamples, and the (nTbS)x(nTbS) array resSamples as inputs.

...

# Test Results

Tables 1 and 2 illustrate the coding performance of the proposed method in CTC and AI, respectively.

Table 1. Performance of the proposed method in CTC

Table 2. Performance of the proposed method in AI

# Conclusion

By integrating the proposed method on 3D-HTM 9.0r1, it improves the coding performance. Hence, it is recommended to adopt the proposed method to the next 3D-HTM.

# Patent Rights Declaration(s)

Samsung Electronics Co. Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).