| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0104r1 |
| --- | --- |

| Title: | CE3 related: Result on clean ups of VSP merging candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yichen Zhang Lu Yu 38# Zheda Road, Hangzhou, China | Tel: Email: | +86-571-87953107 felixzyc@gmail.com yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

# Abstract

This contribution describes the clean ups of VSP merging candidates. The number of VSP candidates in merge candidate list is constrained to one by three different methods.

# Introduction

In the current design, for every prediction unit (PU), the merge candidate list (MCL) is constructed as Fig. 1 shows:

- A HEVC-based MCL is constructed first (reusing the HEVC v1 MCL construction module)

- Then the 3D-HEVC related candidates are inserted into the MCL, including:

  - MPI candidate (Texture motion inherited for depth coding)

  - Inter-view motion prediction (IvMC, Motion from reference view is copied to current view for temporal prediction, specifically, Sup-PU motion prediction candidate)

  - Inter-view disparity prediction (IvDC, using NBDV as a merging candidate)

  - Default VSP candidate

  - Shifted IvMC and IvDC candidate

For VSP-related information, VSP flags of every spatial neighboring blocks are checked before the 3D-HEVC candidates insertion and inherited to its corresponding merging candidate, as Fig. 2 shows. A spatial neighboring block coded by VSP mode turns its corresponding merge candidate to a VSP candidate (by setting the VSP flag of the candidate to 1).

A VSP simplification proposal [1] was adopted in the last meeting (using NBDV for virtual depth block fetching rather than DV of its corresponding spatial neighboring blocks), so all the VSP modes with the same DV for virtual depth block fetching generate same predicting blocks for current PU. So multiple VSP candidates (with no difference except their corresponding merge index) are redundant for the MCL.

Fig. 1. Current design of merge candidate list (MCL) construction

Fig. 2. VSP-related MCL construction

# Proposed Method

The proposed methods all keep only one VSP candidate in the MCL to reduce the redundancy of current design.

Fig 3. Method 1: Keeps only one VSP candidate in the merge candidate list (inherited or default)

Method 1 (extra VSP converted to DCP):

- When checking the VSP flags of spatial neighboring blocks (A1, B1, and B0), only the first candidate with its corresponding block coded by VSP mode is set to a VSP candidate.

- The VSP flags of spatial neighboring block A0 and B2 are always set to 0 for that the position of default VSP candidate is prior to these two

- Other candidates with VSP flag equaling to 1 are turn to a DCP candidate (by setting VSP flag to 0)

- If no spatial neighboring block is VSP coded, the default VSP candidate is inserted to the MCL (the bottom one in Fig. 3).

Method 2 (VSP pruning)

On top of method 1.2, extra VSP candidates are removed from MCL instead of converting to a DCP candidate.

Method 3 (fixed VSP position in MCL):

- All the spatial VSP candidate is converted to a DCP candidate;

- Default VSP candidate is not inserted to MCL;

- A VSP candidate is inserted to a fixed position in MCL (only when merge index equals to a certain value N, i.e., N = 3).

The benefit of method 3 is:

- The determination of using VSP mode for a PU or not could be simplified, i.e., a PU with merge index equal to N (the fixed position of VSP) is coded by VSP mode.

- The VSP flag storage could be totally removed.

Fig. 4. Method 3: Setting all the VSP flags to 0 before processing all the 3D-related candidates

Fig. 5. Method 3: Insert a VSP candidate after all the candidates are processed (fixed position = 3)

# Simulation results

The proposed method was implemented into HTM 9.0r1. Simulations are conducted according to the common test conditions as defined in [2].

The simulation results is summarized and listed in Table 1. More details of the performance can be found in Table 2-4.

Table 1. Summary of description and performance of the proposed methods

| Method | First VSP Candidate | A0/B2 VSP Candidate | Other VSP Candidate(s) | Default VSP | Fixed VSP Position | Video PSNR/ Video Bitrate | Video PSNR/ Total Bitrate | Synth PSNR/ Total Bitrate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | VSP | DCP | DCP |  | N/A | -0.02% | -0.03% | -0.01% |
| 2 | VSP | Remove | Remove |  | N/A | -0.03% | -0.03% | -0.01% |
| 3 | DCP | DCP | DCP | N/A | 3 | 0.02% | 0.01% | 0.03% |

Note: In method 1 and 2, default VSP candidate will still be inserted into MCL when no spatial VSP candidate is available.

Table 2. Performance of method 1 (extra VSP converted to DCP) vs HTM9.0r1

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.03% | 0.06% | 0.00% | -0.01% | -0.03% | 100.4% | 97.2% |
| Kendo | 0.00% | -0.10% | -0.16% | -0.05% | -0.04% | -0.06% | 95.2% | 111.7% |
| Newspaper_CC | 0.00% | -0.02% | 0.04% | 0.00% | -0.02% | 0.03% | 97.1% | 96.0% |
| GT_Fly | 0.00% | -0.12% | -0.12% | -0.03% | -0.03% | 0.00% | 101.9% | 98.0% |
| Poznan_Hall2 | 0.00% | -0.10% | -0.01% | -0.01% | -0.02% | 0.03% | 100.2% | 100.5% |
| Poznan_Street | 0.00% | -0.04% | -0.14% | -0.03% | -0.02% | -0.02% | 99.6% | 108.4% |
| Undo_Dancer | 0.00% | -0.16% | -0.17% | -0.05% | -0.05% | -0.03% | 102.4% | 95.2% |
| Shark | 0.00% | -0.17% | -0.15% | -0.04% | -0.04% | -0.02% | 97.4% | 89.4% |
| 1024x768 | 0.00% | -0.05% | -0.02% | -0.02% | -0.02% | -0.02% | 97.5% | 101.6% |
| 1920x1088 | 0.00% | -0.12% | -0.12% | -0.03% | -0.03% | -0.01% | 100.3% | 98.3% |
| average | 0.00% | -0.09% | -0.08% | -0.02% | -0.03% | -0.01% | 99.26% | 99.56% |

Table 3. Performance of method 2 (VSP pruning) vs HTM9.0r1

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.08% | -0.04% | -0.02% | -0.03% | -0.06% | 98.4% | 106.5% |
| Kendo | 0.00% | -0.13% | -0.17% | -0.06% | -0.05% | -0.08% | 100.0% | 96.5% |
| Newspaper_CC | 0.00% | 0.03% | 0.02% | 0.01% | -0.01% | 0.01% | 97.9% | 99.4% |
| GT_Fly | 0.00% | -0.11% | -0.07% | -0.03% | -0.03% | -0.06% | 104.1% | 98.1% |
| Poznan_Hall2 | 0.00% | -0.04% | -0.05% | -0.02% | -0.04% | 0.07% | 102.3% | 96.0% |
| Poznan_Street | 0.00% | -0.08% | -0.08% | -0.01% | -0.02% | 0.00% | 103.5% | 103.1% |
| Undo_Dancer | 0.00% | -0.12% | -0.10% | -0.04% | -0.04% | 0.03% | 107.4% | 98.9% |
| Shark | 0.00% | -0.10% | -0.08% | -0.03% | -0.03% | -0.03% | 102.4% | 92.4% |
| 1024x768 | 0.00% | -0.06% | -0.06% | -0.03% | -0.03% | -0.04% | 98.8% | 100.8% |
| 1920x1088 | 0.00% | -0.09% | -0.08% | -0.02% | -0.03% | 0.00% | 103.9% | 97.7% |
| average | 0.00% | -0.08% | -0.07% | -0.03% | -0.03% | -0.01% | 102.00% | 98.87% |

Table 4. performance of method 3 (fixed VSP position, 3) vs HTM9.0r1

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.05% | 0.02% | -0.01% | -0.02% | -0.04% | 97.3% | 96.1% |
| Kendo | 0.00% | -0.11% | -0.36% | -0.08% | -0.06% | -0.14% | 95.6% | 100.7% |
| Newspaper_CC | 0.00% | -0.10% | 0.07% | -0.01% | -0.01% | 0.07% | 96.5% | 98.3% |
| GT_Fly | 0.00% | 1.11% | 0.99% | 0.16% | 0.14% | 0.15% | 96.2% | 94.5% |
| Poznan_Hall2 | 0.00% | -0.05% | -0.01% | -0.02% | -0.04% | 0.05% | 95.7% | 99.2% |
| Poznan_Street | 0.00% | 0.04% | -0.04% | -0.02% | -0.01% | -0.01% | 97.1% | 103.5% |
| Undo_Dancer | 0.00% | 0.38% | 0.55% | 0.09% | 0.07% | 0.14% | 98.2% | 100.3% |
| Shark | 0.00% | 0.32% | 0.16% | 0.02% | -0.01% | 0.02% | 96.0% | 89.3% |
| 1024x768 | 0.00% | -0.09% | -0.09% | -0.04% | -0.03% | -0.04% | 96.5% | 98.4% |
| 1920x1088 | 0.00% | 0.36% | 0.33% | 0.05% | 0.03% | 0.07% | 96.7% | 97.3% |
| average | 0.00% | 0.19% | 0.17% | 0.02% | 0.01% | 0.03% | 96.59% | 97.74% |

# Conclusion

In this contribution, three methods are proposed to reduce the number of VSP candidates in merge candidate list. The redundancy of VSP candidate in MCL is removed and the simulation results shows about 0.1% gain on coded views.

# References

- Y.-W. Chen, J.-L. Lin, N. Zhang, J. An, K. Zhang, S. Lei, “3D-CE1: Simplified view synthesis prediction”, JCT3V-F0111, Geneva, CH, Oct. 2013.

- D. Rusanovskyy, K. Muller, A. Vetro, “Common Test Conditions of 3DV Core Experiments”, JCT3V-F1100, Geneva, CH, Oct. 2013.

# Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).