| - Joint Collaborative Team on 3D Video Coding Extensions - of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 - 7th Meeting: San Jose, US, 11 Jan. – 17 Jan. 2014 | Document: JCT3V-G0107r5 M31965r5 |
| --- | --- |

| Title: | Technically improved texture and depth view packing SEI message in 3D-AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takanori Senoh Koki Wakunami Yasuyuki Ichihashi Hisayuki Sasaki Kenji Yamamoto Masayuki Tanimoto Ying Chen | Tel: Email: | senoh@nict.go.jp k.wakunami@nict.go.jp y-ichihashi@nict.go.jp sasaki@nict.go.jp k.yamamoto@nict.go.jp tanimototentative3@yahoo.co.jp cheny@qti.qualcomm.com |
| Source: | NICT, NISRI and Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document provides technically improved texture and depth view packing SEI message in 3D-AVC. The document is improved based on the editor’s comments at the 6th JCT-3V meeting.

# Relevant WD text changes in 3D-AVC

In Table D.1, add the following after “constrained_depth_parameter_set_identifier ( payloadSize )”:

| else if( payloadType = = 55 ) |  |  |
| --- | --- | --- |
| texture_and_depth_view_packing( payloadSize ) /* specified in Annex J */ | 5 |  |

In J.13.1 SEI message syntax, add the following after J.13.1.1

      - Texture and depth view packing SEI message syntax

| texture_and_depth_view_packing( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| depth_type | 5 | ue(v) |
| if( depth_type = = 0 ) { |  |  |
| num_residual_texture_views_minus1 | 5 | ue(v) |
| residual_depth_flag | 5 | u(1) |
| view_synthesis_parameter( ) | 5 |  |
| } |  |  |
| } |  |  |

        - View synthesis parameter syntax

[Reviewer (JR O): syntax should follow the existing one.]

| View_synthesis_parameter( ) { |  | C | Descriptor |  |
| --- | --- | --- | --- | --- |
| num_views = num_residual_texture_views_minus1 + 2 |  |  |  |  |
| z_flag |  | 5 | u(1) |  |
| intrinsic_param_flag |  | 5 | u(1) |  |
| rotation_flag |  | 5 | u(1) |  |
| translation_flag |  | 5 | u(1) |  |
| if( z_flag ) { |  |  |  |  |
| for( i=0; i<num_views; i++ ) { |  |  |  |  |
| sign_z_near[ i ] |  | 5 | u(1) |  |
| exponent_z_near[ i ] |  | 5 | u(7) |  |
| mantissa_len_minus1_z_near[ i ] |  | 5 | u(5) |  |
| mantissa_z_near[ i ] |  | 5 | u(v) |  |
| sign_z_far[ i ] |  | 5 | u(1) |  |
| exponent_z_far[ i ] |  | 5 | u(7) |  |
| mantissa_len_minus1_z_far[ i ] |  | 5 | u(5) |  |
| mantissa_z_far[ i ] |  | 5 | u(v) |  |
| } |  |  |  |  |
| } |  |  |  |  |
| if( intrinsic_param_flag ) { |  |  |  |  |
| prec_focal_length | 5 |  |  | ue(v) |
| prec_principal_point | 5 |  |  | ue(v) |
| } |  |  |  |  |
| if( rotation_flag ) |  |  |  |  |
| prec_rotation_param |  | 5 | ue(v) |  |
| If ( translation_flag ) |  |  |  |  |
| prec_translation_param |  | 5 | ue(v) |  |
| for( i=0; i<num_views; i++ ) { |  |  |  |  |
| if( intrinsic_param_flag ) { |  |  |  |  |
| sign_focal_length_x[ i ] |  | 5 | u(1) |  |
| exponent_focal_length_x[ i ] |  | 5 | u(6) |  |
| mantissa_focal_length_x[ i ] |  | 5 | u(v) |  |
| sign_focal_length_y[ i ] |  | 5 | u(1) |  |
| exponent_focal_length_y[ i ] |  | 5 | u(6) |  |
| mantissa_focal_length_y[ i ] |  | 5 | u(v) |  |
| sign_principal_point_x[ i ] |  | 5 | u(1) |  |
| exponent_principal_point_x[ i ] |  | 5 | u(6) |  |
| mantissa_principal_point_x[ i ] |  | 5 | u(v) |  |
| sign_principal_point_y[ i ] |  | 5 | u(1) |  |
| exponent_principal_point_y[ i ] |  | 5 | u(6) |  |
| mantissa_principal_point_y[ i ] |  | 5 | u(v) |  |
| } |  |  |  |  |
| if( rotation_flag ) { |  |  |  |  |
| For ( j=1;j<=3;j++ ) { /* row */ |  |  |  |  |
| for( k=1;k<=3;k++ ) { /* column */ |  |  |  |  |
| sign_r[ i ][ j ][ k ] |  | 5 | u(1) |  |
| exponent_r[ i ][ j ][ k ] |  | 5 | u(6) |  |
| mantissa_r[ i ][ j ][ k ] |  | 5 | u(v) |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| If ( translation_flag ) { |  |  |  |  |
| sign_t_x[ i ] |  | 5 | u(1) |  |
| exponent_t_x[ i ] |  | 5 | u(6) |  |
| mantissa_t_x[ i ] |  | 5 | u(v) |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

In J.13.2 SEI message semantics, add the following after J.13.2.1

      - Texture and depth view packing SEI message semantics

The texture and depth view packing SEI message informs the decoder that some view components of the output views contain samples of multiple distinct spatially packed constituent residual pictures. The output views are not suitable to be displayed directly and may be converted to the natural views that can be appropriate for direct display as indicated by the depth_type syntax element. This information can be used after the decoder output to appropriately rearrange the samples to produce natural views that are appropriate for display or to server for other purposes (which are outside the scope of this Recommendation | International Standard).

The depth view information SEI message, when present, shall be associated with an IDR access unit. The information signaled in the SEI message applies to all the access units in the coded video sequence. This SEI message conveys information associated with the coded video sequence that outputs views that may be further processed to generate the natural views that are to be displayed.

In such a coded video sequence, there are at least two output views, wherein the base view is a natural view and a non-base view is a residual view. A residual view is not a natural view, instead it contains a sequence of difference pictures which are decoded view components of the residual view. Each difference picture is created by frame packing up to four constituent residual pictures.

Each constituent residual picture represents a picture generated by 2:1 decimation from the residual picture horizontally and vertically, starting from the top-left most sample.

Each constituent residual picture is considered as produced by pictures of two natural views, wherein one belongs to the view that is coded as the base view and used to predict the other picture in the other view of view A. The residual picture of such a process is considered as the residual picture associated with view A. The residual picture consists of the pixels which are not predictable by the base view, namely occluded pixels and out-of-picture pixels in the base picture. After decimation from the residual picture associated with view A, a constituent residual picture associated with view A is generated.

depth_type specifies the arrangement how distinct spatially packed constituent residual pictures are packed into view components. The values of depth_type greater than 0 are reserved for future use.

num_residual_texture_views_minus1 + 1 specifies the number of constituent residual pictures packed to composite a difference picture which is a non-base texture view component. num_residual_texture_views_minus1 shall be in the range of 0 to 3, inclusive.

residual_depth_flag equal to 1 specifies that the number of constituent residual pictures packed to composite a difference picture which is a depth view component of a non-base view is equal to num_residual_texture_views_minus1 + 1. residual_depth_flag equal to 0 specifies there is only one depth view.

When depth_type is equal to 0, num_anchor_refs_l0[ i ], num_anchor_refs_l1[ i ], num_non_anchor_refs_l0[ i ] and num_non_anchor_refs_l1[ i ] shall be equal to 0 in the active SPS RBSP for the coded video sequence and avc_3d_extension_flag shall be 0 in all VCL NAL units of the coded video sequence.

NOTE: These constraints disable tools for inter-view and inter-component prediction.

When depth_type is equal to 0, the coded video sequence shall contain two texture views, wherein the non-base texture view is the only residual view containing texture information and is composited by spatially packed residual pictures for texture.

When depth_type is equal to 0, the number of coded depth views in coded video sequence shall be equal to 1 + residual_depth_flag.

The order of view synthesis parameters for ( i=0, i< num_views) is base view, residual picture1, residual picture2, ….

The constituent residual pictures are converted to horizontally and vertically half-size constituent residual pictures and packed in the non-base texture view from top-left, to bottom-left, top-right and bottom-right order. Their top-left co-ordinates of the constituent residual pictures in the difference picture of the residual view of width=W and height=H is shown in Table J-1 where W and H shall be even number. Each constituent residual picture represents an occluded area or out-of-picture area of the base texture view when it is projected to the corresponding input texture views. Process to reconstruct natural views from constituent residual pictures is summarized in the following paragraph.

- Table J-1 – Top-left corner co-ordinates of constituent residual pictures (i=1, i<5) packed in a residual view of width=W and height=H.

| i=1 | (0, 0) |
| --- | --- |
| i=2 | (H/2, 0) |
| i=3 | (0, W/2) |
| i=4 | (H/2, W/2) |

Base depth view (i=0) is a sample-wise median of the same coordinates of all input depth views and is associated with the base texture view (i=0s) when num_view is odd number. The other input depth views are converted to the constituent residual pictures (i=1,…4) for depth of the non-base depth view component as specified in the next paragraph. They are packed in the non-base depth view in the same positions as the constituent residual texture pictures shown in Table J-1. Each constituent residual picture of the non-base depth view component associates with a constituent residual picture of the non-base texture view component in the same relative location.

When residual_depth_flag is equal to 0 and num_views is even number, the position of base depth view (i=0) is the center of the first view and the second view( i=0 and 1).

Each constituent residual picture for depth represents the occluded area or the out-of-pictures area of the base depth view when it is projected to the other input depth views.

Within an access unit, given a decoded view component of the base view and the decoded view component of the non-base view, which is a difference picture, the picture of the natural view, view A of the same time instance is generated as follows.

- A constituent residual picture associated with view A from a difference picture is firstly extracted out and 1:2 upsampled horizontally and vertically to a residual picture associated with view A.

- The decoded view component of the base view is used to produce the prediction picture associated with view A.

- A residual picture and the prediction picture of view A are composited to produce the picture of the view A.

[Ed. (MH/CY): camera parameters to be associated with the constituent residual pictures. The current camera parameters are associated ONLY for the coded views. The current design might need camera parameters to be transmitted in other metadata that is not supported in the current specification.]

      - View Synthesis Parameter semantics

The syntax structure specifies parameters of view synthesis parameters which are used to synthesize views from the decoded base view and constituent residual pictures.

z_flag equal to 1 indicates the presence of the nearest depth value (z_near) and the farthest depth value (z_far). z_flag equal to 0 indicates the absence of the nearest and the farthest depth values.

intrinsic_param_flag equal to 1 indicates the presence of intrinsic camera parameters. intrinsic_param_flag equal to 0 indicates the absence of intrinsic camera parameters.

rotation_flag equal to 1 indicates the presence of rotation camera parameters. rotation_flag equal to 0 indicates the absence of rotation camera parameters. If rotation_flag is 0, a default rotation camera parameter of unit matrix is used unless provided before.

translation_flag equal to 1 indicates the presence of horizontal translation camera parameters. translation_flag equal to 0 indicates the absence of horizontal translation camera parameters.

sign_z_near[ i ] equal to 0 indicates that the sign of the nearest depth value of the i-th camera is positive. sign_z_near [ i ] equal to 1 indicates that the sign is negative.

exponent_z_near[ i ] specifies the exponent part of the nearest depth value of the i-th camera. The value of exponent_z_near[ i ] shall be in the range of 0 to 27-2, inclusive. The value 27-1 is reserved for future use by ITU T | ISO/IEC. Decoders shall treat the value 27-1 as indicating an unspecified z near[ i ].

mantissa_len_minus1_z_near[ i ] + 1 specifies the number of bits in the mantissa of the nearest depth value of the i-th camera. The value of mantissa_len_minus1_z_near[ i ] shall be in the range of 0 to 31, inclusive.

mantissa_z_near[ i ] specifies the mantissa part of the nearest depth value of the i-th camera.

sign_z_far[ i ] equal to 0 indicates that the sign of the farthest depth value of the i-th camera is positive. sign_z_far[ i ] equal to 1 indicates that the sign is negative.

exponent_z_far[ i ] specifies the exponent part of the farthest depth value of the i-th camera. The value of exponent_z_far[ i ] shall be in the range of 0 to 27-2, inclusive. The value 27-1 is reserved for future use by ITU T | ISO/IEC. Decoders shall treat the value 27-1 as indicating an unspecified z far[ i ].

mantissa_len_minus1_z_far[ i ] + 1 specifies the number of bits in the mantissa of the farthest depth value of the i-th camera. The value of mantissa_len_minus1_z_far[ i ] shall be in the range of 0 to 31, inclusive.

mantissa_z_far[ i ] specifies the mantissa part of the farthest depth value of the i-th camera.

prec_focal_length specifies the exponent of the maximum allowable truncation error for focal_length_x[ i ] and focal_length_y[ i ] as given by 2−prec_focal_length. The value of prec_focal_length shall be in the range of 0 to 31, inclusive.

prec_principal_point specifies the exponent of the maximum allowable truncation error for principal_point_x[ i ] and principal_point_y[ i ] as given by 2−prec_principal_point. The value of prec_principal_point shall be in the range of 0 to 31, inclusive.

prec_rotation_param specifies the exponent of the maximum allowable truncation error for r[ i ][ j ][ k ] as given by 2−prec_rotation_param. The value of prec_rotation_param shall be in the range of 0 to 31, inclusive.

prec_translation_param specifies the exponent of the maximum allowable truncation error for t[ i ][ j ] as given by 2−prec_translation_param. The value of prec_ translation_param shall be in the range of 0 to 31, inclusive.

sign_focal_length_x[ i ] equal to 0 indicates that the sign of the focal length of the i-th camera in the horizontal direction is positive. sign_focal_length_x [ i ] equal to 1 indicates that the sign is negative.

exponent_focal_length_x[ i ] specifies the exponent part of the focal length of the i-th camera in the horizontal direction. The value of exponent_focal_length_x[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified focal length x[ i ].

mantissa_focal_length_x[ i ] specifies the mantissa part of the focal length of the i-th camera in the horizontal direction. The length of the mantissa_focal_length_x[ i ] syntax element is variable and determined as follows:

–	If exponent_focal_length_x[ i ] = = 0, the length is Max( 0, prec_focal_length − 30 ).

–	Otherwise (0 < exponent_focal_length_x[ i ] < 63), the length is Max( 0, exponent_focal_length_x[ i ] + prec_focal_length − 31 ).

sign_focal_length_y[ i ] equal to 0 indicates that the sign of the focal length of the i-th camera in the vertical direction is positive. sign_focal_length_y [ i ] equal to 1 indicates that the sign is negative.

exponent_focal_length_y[ i ] specifies the exponent part of the focal length of the i-th camera in the vertical direction. The value of exponent_focal_length_y[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified focal length y[ i ].

mantissa_focal_length_y[ i ] specifies the mantissa part of the focal length of the i-th camera in the vertical direction.

The length of the mantissa_focal_length_y[ i ] syntax element is variable and determined as follows:

–	If exponent_focal_length_y[ i ] = = 0, the length is Max( 0, prec_focal_length − 30 ).

–	Otherwise (0 < exponent_focal_length_y[ i ] < 63), the length is Max( 0, exponent_focal_length_y[ i ] + prec_focal_length − 31 ).

sign_principal_point_x[ i ] equal to 0 indicates that the sign of the principal point of the i-th camera in the horizontal direction is positive. sign_ principal_point_x [ i ] equal to 1 indicates that the sign is negative.

exponent_principal_point_x[ i ] specifies the exponent part of the principal point of the i-th camera in the horizontal direction. The value of exponent_ principal_point_x[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified principal point x[ i ].

mantissa_principal_point_x[ i ] specifies the mantissa part of the principal point of the i-th camera in the horizontal direction The length of the mantissa_principal_point_x[ i ] syntax element in units of bits is variable and is determined as follows:

–	If exponent_principal_point_x[ i ] = = 0, the length is Max( 0, prec_principal_point − 30 ).

–	Otherwise (0 < exponent_principal_point_x[ i ] < 63), the length is Max( 0, exponent_principal_point_x[ i ] + prec_principal_point − 31 ).

sign_principal_point_y[ i ] equal to 0 indicates that the sign of the principal point of the i-th camera in the vertical direction is positive. sign_ principal_point_y [ i ] equal to 1 indicates that the sign is negative.

exponent_principal_point_y[ i ] specifies the exponent part of the principal point of the i-th camera in the vertical direction. The value of exponent_ principal_point_y[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified principal point y[ i ].

mantissa_principal_point_y[ i ] specifies the mantissa part of the principal point of the i-th camera in the vertical direction The length of the mantissa_principal_point_y[ i ] syntax element in units of bits is variable and is determined as follows:

–	If exponent_principal_point_y[ i ] = = 0, the length is Max( 0, prec_principal_point − 30 ).

–	Otherwise (0 < exponent_principal_point_y[ i ] < 63), the length is Max( 0, exponent_principal_point_y[ i ] + prec_principal_point − 31 ).

sign_r[ i ][ j ][ k ] equal to 0 indicates that the sign of (j, k) component of the rotation matrix for the i-th camera is positive. sign_r [ i ] [ j ][ k ] equal to 1 indicates that the sign is negative.

exponent_r[ i ][ j ][ k ] specifies the exponent part of (j, k) component of the rotation matrix for the i-th camera. The value of exponent_r [ i ][ j ][ k ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified rotation matrix.

mantissa_r[ i ][ j ][ k ] specifies the mantissa part of (j, k) component of the rotation matrix for the i-th camera.

The length of the mantissa_r[ i ][ j ][ k ] syntax element in units of bits is variable and determined as follows:

–	If exponent_r[ i ] = = 0, the length is Max( 0, prec_rotation_param − 30 ).

–	Otherwise (0 < exponent_r[ i ] < 63), the length is Max( 0, exponent_r[ i ] + prec_rotation_param − 31 ).

The rotation matrix R[ i ] for i-th camera is represented as follows:

rE[i][0][0]rE[i][0][1]rE[i][0][2]rE[ i ][ 1 ][ 0 ]rE[i][1][1]rE[i][1][2]rE[ i ][ 2 ][ 0 ]rE[i][2][1]rE[i][2][2]	(J-82)

sign_t_x[ i ] equal to 0 indicates that the sign of horizontal component of the translation vector for the i-th camera is positive. sign_trans_x[ i ] equal to 1 indicates that the sign is negative.

exponent_t_x[ i ] specifies the exponent part of horizontal component of the translation vector for the i-th camera. The value of exponent_t_x[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified translation vector.

mantissa_t_x[ i ] specifies the mantissa part of horizontal component of the translation vector for the i-th camera.

The length v of the mantissa_t[ i ][ j ] syntax element in units of bits is variable and is determined as follows:

–	If exponent_t_x[ i ] = = 0, the length v = Max( 0, prec_translation_param − 30 ).

–	Otherwise (0 < exponent_t[ i ] < 63), the length v = Max( 0, exponent_t[ i ] + prec_translation_param − 31 ).

The variable x={focal_length, principal_point, rotation, translation} is computed as follows. Where e=exponent, s=sign, n=mantissa and v=mantissa_len_minus1 + 1:

–	If 0 < e < 63, x = (−1)s * 2e−31 * (1 + n  2v).

–	Otherwise (e is equal to 0), x = (−1)s * 2−(30+v) * n.

NOTE – The above specification is similar to that found in IEC 60559:1989, Binary floating-point arithmetic for microprocessor systems.

# Patent rights declarations

NICT (National Institute of Information and Communications Technology) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NISRI (Nagoya Industrial Science Research Institute) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).