| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0111 |
| --- | --- |

| Title: | CE5-related: Implicit split process for intra SDC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weiran Li Xiaozhen Zheng， Yun He | Tel: Email: | +86-755-28425082 zhengxiaozhen@huawei.com |
| Source: | Tsinghua University, Hisilicon Technologies |  |  |

_____________________________

# Abstract

This contribution provides an implicit split method for intra SDC coding, which targets at the alignment between the split process of HEVC and 3D-HEVC. Experimental results show that the proposed method has about 0.1% impact on coding performance of AI and CTC test cases.

# Introduction

In HEVC design, the maximum intra prediction block size can be controlled by the syntax log2_min_transform_block_size_minus2 and log2_diff_max_min_transform_block_size, and the maximum size is up to 32x32. Because the intra prediction block size is always bounded with the transform block, the intra prediction block size will be implicit split when current transform block size is greater than the maximum transform block size until the transform block size is equal to the maximum transform block size. This design provides the flexible capability to encoder. However, 3D-HEVC has broken this split process because SDC's coding size is always equal to the coding block size. It brings two drawbacks compared to HEVC's design:

1. Restrict the flexibility of encoder. No matter what the maximum transform block size is set by encoder, encoder still need to handle the size that is greater the maximum size due to intra SDC coding.

2. The 64x64 intra prediction block need to be handled, which is greater than the maximum size (32x32) in HEVC. It will increase the implementation cost because: a. the storage cost for the intra prediction block is increased from 32x32 to 64x64, b. the computing cost is also increased since the 64x64 planar mode is computation expensive, c. the hardware pipeline for intra prediction need to be modified, d. the derivation process for the neighbouring samples become complicated since the additional test case (64x64) need to be handled. Those problems are also mentioned in [1] at the previous meeting.

Considering the problems mentioned above, this contribution provides a implicit split method which can align the split process for 3D-HEVC with HEVC design.

# Proposed method

The proposed method is based on the concept of HEVC's split process. In HEVC, the parameter splitFlag is set to 1 if log2TrafoSize is greater than Log2MaxTrafoSize (the maximum intra prediction block size). Similarly, if the SDC coding block size is greater than Log2MaxTrafoSize, the parameter splitFlag is also set to 1 until the SDC coding block size is equal to Log2MaxTrafoSize in the proposed method. Considering SDC need to code one residual value (depth_dc_abs and depth_dc_sign_flag), every SDC coding block after the split process has it's residual value (depth_dc_abs and depth_dc_sign_flag). Therefore, multiple SDC residual values need to coded for the SDC coding block whose block size is greater than Log2MaxTrafoSize. Set the current coding block size to log2TrafoSize, and set the maximum transform block size to Log2MaxTrafoSize. Then, if log2TrafoSize is greater than Log2MaxTrafoSize, the number of SDC residual is calculated as (1<<( log2TrafoSize- Log2MaxTrafoSize))* (1<<( log2TrafoSize- Log2MaxTrafoSize))

Same to HEVC's split process design, all SDC coding block after the split process share the same intra prediction mode. The implicit split process is illustrated in Figure 1.

Figure 1: Implicit process for the SDC block whose block size (log2TrafoSize) is greater than the maximum intra prediction block size (Log2MaxTrafoSize)

# Experimental results

The proposed method is implemented on HTM9.0r1 and the platform of CE5 test (F0126 - generic SDC). The coding efficiency is evaluated in random access (CTC) and All-Intra configurations. The table 1 and table 2 show the test results generated from HTM9.0r1 whose anchor is HTM9.0r1. The table 3 and table 4 show the test results generated from the platform of CE5 test whose anchor is HTM9.0r1 + F0126 (generic SDC)

Table 1: Results for all - intra (anchor: htm9.0r1)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | 0.09% | 0.12% | 100.3% | 101.3% | 99.9% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | 0.19% | 0.16% | 100.3% | 104.0% | 102.9% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | 0.08% | 0.06% | 100.4% | 100.1% | 99.8% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | 0.30% | 0.28% | 100.0% | 106.2% | 97.9% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.00% | 0.35% | 0.49% | 101.6% | 103.5% | 96.6% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | 0.18% | 0.12% | 100.5% | 103.9% | 98.6% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | 0.05% | 0.09% | 100.5% | 99.4% | 95.9% |
| Shark | 0.0% | 0.0% | 0.0% | 0.00% | 0.11% | 0.11% | 101.4% | 106.1% | 100.2% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | 0.12% | 0.11% | 100.4% | 101.8% | 100.9% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | 0.20% | 0.22% | 100.8% | 103.8% | 97.8% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | 0.17% | 0.18% | 100.6% | 103.1% | 99.0% |

Table 2: Results for CTC (anchor: htm9.0r1)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.0% | -0.02% | 0.03% | 0.01% | 99.9% | 101.7% | 99.2% |
| Kendo | 0.0% | -0.2% | -0.1% | -0.06% | 0.07% | 0.05% | 100.1% | 99.1% | 100.3% |
| Newspaper_CC | 0.0% | -0.1% | 0.1% | 0.02% | 0.07% | 0.14% | 100.1% | 101.8% | 100.6% |
| GT_Fly | 0.0% | -0.1% | -0.4% | -0.05% | 0.02% | -0.01% | 100.0% | 99.2% | 99.1% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.1% | 0.04% | 0.17% | 0.24% | 100.2% | 100.1% | 99.4% |
| Poznan_Street | 0.0% | -0.4% | -0.1% | -0.07% | 0.02% | -0.02% | 100.1% | 100.5% | 100.7% |
| Undo_Dancer | 0.0% | -0.1% | 0.1% | 0.01% | 0.02% | -0.04% | 99.9% | 100.2% | 100.3% |
| Shark | 0.0% | 0.0% | -0.1% | -0.01% | 0.04% | 0.07% | 99.9% | 100.8% | 100.9% |
| 1024x768 | 0.0% | -0.1% | 0.0% | -0.02% | 0.06% | 0.07% | 100.0% | 100.9% | 100.0% |
| 1920x1088 | 0.0% | -0.1% | -0.1% | -0.02% | 0.05% | 0.05% | 100.0% | 100.2% | 100.1% |
| average | 0.0% | -0.1% | -0.1% | -0.02% | 0.06% | 0.06% | 100.0% | 100.4% | 100.1% |

Table 3: Results for all - intra (anchor: htm9.0r1+generic SDC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.00% | 0.00% | 0.00% | 0.10% | 0.12% | 85.0% | 102.1% | 99.3% |
| Kendo | 0.00% | 0.00% | 0.00% | 0.00% | 0.15% | 0.17% | 74.6% | 91.2% | 87.0% |
| Newspaper_CC | 0.00% | 0.00% | 0.00% | 0.00% | 0.12% | 0.08% | 84.7% | 105.7% | 100.0% |
| GT_Fly | 0.00% | 0.00% | 0.00% | 0.00% | -0.16% | -0.07% | 86.4% | 108.3% | 99.2% |
| Poznan_Hall2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.24% | 0.49% | 89.2% | 109.8% | 101.8% |
| Poznan_Street | 0.00% | 0.00% | 0.00% | 0.00% | 0.04% | 0.14% | 83.7% | 102.0% | 95.7% |
| Undo_Dancer | 0.00% | 0.00% | 0.00% | 0.00% | 0.05% | 0.10% | 89.5% | 110.0% | 102.4% |
| Shark | 0.00% | 0.00% | 0.00% | 0.00% | 0.10% | 0.21% | 85.9% | 106.2% | 99.4% |
| 1024x768 | 0.00% | 0.00% | 0.00% | 0.00% | 0.12% | 0.12% | 81.4% | 99.7% | 95.4% |
| 1920x1088 | 0.00% | 0.00% | 0.00% | 0.00% | 0.06% | 0.17% | 86.9% | 107.2% | 99.7% |
| average | 0.00% | 0.00% | 0.00% | 0.00% | 0.08% | 0.15% | 84.9% | 104.4% | 98.1% |

Table 4: Results for CTC (anchor: htm9.0r1+generic SDC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.01% | -0.03% | -0.01% | 0.03% | 0.03% | 106.4% | 109.6% | 106.2% |
| Kendo | 0.00% | 0.00% | -0.06% | -0.01% | 0.13% | 0.14% | 96.9% | 102.7% | 97.5% |
| Newspaper_CC | 0.00% | 0.02% | 0.00% | -0.02% | 0.08% | 0.13% | 91.8% | 102.3% | 95.0% |
| GT_Fly | 0.00% | -0.12% | -0.03% | -0.02% | 0.01% | 0.01% | 91.7% | 100.1% | 95.1% |
| Poznan_Hall2 | 0.00% | -0.05% | -0.06% | -0.02% | 0.13% | 0.19% | 99.5% | 105.3% | 100.0% |
| Poznan_Street | 0.00% | 0.01% | 0.06% | 0.00% | 0.04% | 0.08% | 92.5% | 104.1% | 94.6% |
| Undo_Dancer | 0.00% | -0.11% | -0.23% | -0.02% | 0.03% | 0.03% | 96.5% | 105.8% | 98.1% |
| Shark | 0.00% | 0.27% | -0.08% | 0.02% | 0.10% | 0.13% | 105.6% | 113.1% | 108.6% |
| 1024x768 | 0.00% | 0.00% | -0.03% | -0.01% | 0.08% | 0.10% | 98.4% | 104.9% | 99.6% |
| 1920x1088 | 0.00% | 0.00% | -0.07% | -0.01% | 0.06% | 0.09% | 97.2% | 105.7% | 99.3% |
| average | 0.00% | 0.00% | -0.05% | -0.01% | 0.07% | 0.09% | 97.6% | 105.4% | 99.4% |

Besides CTC test cases, we also set the maximum TU size to 16x16, and test the proposed implicit split method at this case. Considering the similar split concept is proposed at JCT3V-G0123 at this meeting, we also test the performance for G0123’s design at the same test condition. The results are shown in table 5 and 6.

Table 5: Results for G0111’s split method (Maximum TU size = 16x16)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | 0.56% | 0.77% | 102.0% | 104.3% | 98.6% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | 0.76% | 1.02% | 101.9% | 108.2% | 101.2% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | 0.45% | 0.71% | 101.3% | 105.4% | 99.4% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | 0.32% | 0.55% | 100.5% | 107.9% | 100.6% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.00% | 0.64% | 1.53% | 104.4% | 109.2% | 100.6% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | 0.27% | 0.57% | 100.8% | 103.9% | 98.2% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | 0.16% | 0.55% | 101.3% | 101.3% | 97.4% |
| Shark | 0.0% | 0.0% | 0.0% | 0.00% | 0.41% | 0.70% | 101.4% | 101.9% | 96.7% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | 0.59% | 0.83% | 101.7% | 106.0% | 99.7% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | 0.36% | 0.78% | 101.7% | 104.8% | 98.7% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | 0.45% | 0.80% | 101.7% | 105.3% | 99.1% |

Table 6: Results for G0123’s split method (Maximum TU size = 16x16)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | 0.23% | 0.36% | 101.0% | 104.7% | 99.2% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | 0.25% | 0.42% | 100.8% | 105.2% | 101.3% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | 0.23% | 0.40% | 100.6% | 101.9% | 100.6% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | 0.19% | 0.32% | 100.1% | 104.6% | 101.4% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.00% | 0.29% | 0.66% | 101.6% | 105.2% | 100.8% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | 0.12% | 0.29% | 99.8% | 102.1% | 99.0% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | 0.11% | 0.23% | 99.9% | 99.9% | 98.8% |
| Shark | 0.0% | 0.0% | 0.0% | 0.00% | 0.21% | 0.32% | 100.0% | 107.6% | 97.9% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | 0.24% | 0.39% | 100.8% | 103.9% | 100.4% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | 0.19% | 0.36% | 100.3% | 103.9% | 99.6% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | 0.21% | 0.37% | 100.5% | 103.9% | 99.9% |

Compare to the method proposed in JCT3V-G0123, the main difference between G0111 and G0123 is that G0111 need to code 4 SDC residual for 64x64 SDC at CTC case whereas G0123 only need to code 1 SDC residual for 64x64 SDC. We have implemented the G0123’s method on top of G0111’s codebase, and got the identical results as G0123. The results are shown in Table 7 and Table 8.

Table 7: Results for G0111’s split method+ 1 SDC residual coding (anchor: generic SDC in G0122, CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.3% | 0.0% | -0.04% | -0.05% | -0.02% | 104.6% | 100.0% | 97.9% |
| Kendo | 0.0% | -0.1% | -0.1% | -0.04% | -0.05% | 0.02% | 105.0% | 99.1% | 100.2% |
| Newspaper_CC | 0.0% | 0.1% | -0.1% | 0.00% | 0.01% | 0.06% | 106.0% | 98.3% | 99.8% |
| GT_Fly | 0.0% | 0.1% | 0.1% | 0.01% | 0.04% | 0.01% | 105.1% | 97.7% | 99.1% |
| Poznan_Hall2 | 0.0% | -0.2% | -0.1% | -0.05% | 0.03% | 0.07% | 106.0% | 99.0% | 98.9% |
| Poznan_Street | 0.0% | 0.0% | -0.1% | -0.01% | 0.00% | 0.02% | 106.1% | 99.8% | 98.7% |
| Undo_Dancer | 0.0% | 0.1% | -0.1% | 0.01% | 0.03% | 0.03% | 104.2% | 99.5% | 100.2% |
| Shark | 0.0% | 0.1% | -0.1% | 0.00% | 0.04% | 0.06% | 104.2% | 99.5% | 99.8% |
| 1024x768 | 0.0% | -0.1% | 0.0% | -0.03% | -0.03% | 0.02% | 105.2% | 99.1% | 99.3% |
| 1920x1088 | 0.0% | 0.0% | -0.1% | -0.01% | 0.03% | 0.04% | 105.1% | 99.1% | 99.4% |
| average | 0.0% | 0.0% | -0.1% | -0.02% | 0.01% | 0.03% | 105.2% | 99.1% | 99.3% |

Table 8: Results for G0111’s split method+ 1 SDC residual coding (anchor: generic SDC in G0122, AI)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | 0.02% | 0.02% | 159.7% | 93.8% | 100.0% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | 0.01% | 0.03% | 160.6% | 96.1% | 103.1% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | 0.03% | 0.01% | 149.9% | 92.6% | 101.5% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | 0.02% | 0.02% | 143.9% | 91.8% | 101.5% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.00% | 0.12% | 0.16% | 162.1% | 89.2% | 95.5% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | 0.03% | 0.04% | 149.8% | 94.7% | 99.6% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | 0.02% | 0.03% | 149.6% | 90.3% | 100.2% |
| Shark | 0.0% | 0.0% | 0.0% | 0.00% | 0.05% | 0.04% | 149.2% | 91.3% | 96.3% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | 0.02% | 0.02% | 156.7% | 94.2% | 101.5% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | 0.05% | 0.06% | 150.9% | 91.4% | 98.6% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | 0.04% | 0.04% | 153.1% | 92.5% | 99.7% |

# Conclusion

This contribution proposes a implicit split process for intra SDC coding block that align the split process for 3D-HEVC with HEVC design. The proposed split method can keep the same encoder flexibility as HEVC design, and simplify the decoder operation for SDC blocks. The experimental results show that the proposed method almost does not have the impact on the coding performance, encoding time and decoding time. Therefore, it's recommended to incorporate the proposed method into the next version of HTM and working draft.

# Changes to the description of Test Model

#1: Code 4 SDC residual at 64x64 SDC modes

H.7.3.8.5.1 Depth mode parameter syntax

| depth_mode_parameters( x0 , y0 , log2CbSize ) { | Descriptor |
| --- | --- |
| depth_intra_mode[ x0 ][ y0 ] | ae(v) |
| if ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_DMM_WFULL ¦ ¦ DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_SDC_DMM_WFULL ) |  |
| wedge_full_tab_idx[ x0 ][ y0 ] | ae(v) |
| if( DmmFlag[ x0 ][ y0 ] ¦ ¦ SdcFlag[ x0 ][ y0 ] ) { |  |
| if ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_SDC_PLANA && log2CbSize > Log2MaxTrafoSize ) |  |
| { |  |
| dcNumSeg = ( 1 << (log2CbSize - Log2MaxTrafoSize ) ) * ( 1 << (log2CbSize - Log2MaxTrafoSize ) ) |  |
| } |  |
| else if |  |
| { |  |
| dcNumSeg = ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_SDC_PLANAR ) ? 1 : 2 |  |
| } |  |
| depth_dc_flag[ x0 ][ y0 ] | ae(v) |
| if ( depth_dc_flag[ x0 ][ y0 ] ) |  |
| for( i = 0; i < dcNumSeg; i ++ ) { |  |
| depth_dc_abs[ x0 ][ y0 ][ i ] | ae(v) |
| if ( depth_dc_abs[ x0 ][ y0 ][ i ]) |  |
| depth_dc_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |

H.8.4.4.1 General decoding process for intra blocks

The variable splitFlag is derived as follows:

- If cIdx is equal to 0 and SdcFlag[  xTb0 ][ xTb0 ] is equal to 0, splitFlag is set equal to split_transform_flag[ xTbY ][ yTbY ][ trafoDepth ].

- Otherwise, if all of the following conditions are true, splitFlag is set equal to 1.

  - cIdx is greater than 0

  - split_transform_flag[ xTbY ][ yTbY ][ trafoDepth ] is equal to 1

  - log2TrafoSize is greater than 2

  - SdcFlag[  xTb0 ][ xTb0 ] is equal to 1 and log2TrafoSize is greater than Log2MaxTrafoSize

- Otherwise, splitFlag is set equal to 0.

- #2 Code 1 SDC residual at 64x64 SDC modes

- H.8.4.4.1 General decoding process for intra blocks

- Inputs to this process are:

- a sample location ( xTb0, yTb0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable log2TrafoSize specifying the size of the current transform block,

- a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding unit,

- a variable predModeIntra specifying the intra prediction mode,

- a variable cIdx specifying the colour component of the current block.

Output of this process is a modified reconstructed picture before deblocking filtering.

The luma sample location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture is derived as follows:

  - ( xTbY, yTbY ) = ( cIdx  = =  0 ) ? ( xTb0, yTb0 ) : ( xTb0  <<  1, yTb0  <<  1 )	(Error: Reference source not found-1)

The variable splitFlag is derived as follows:

- If cIdx is equal to 0, splitFlag is set equal to split_transform_flag[ xTbY ][ yTbY ][ trafoDepth ].

- Otherwise, if all of the following conditions are true, splitFlag is set equal to 1.

  - cIdx is greater than 0

  - split_transform_flag[ xTbY ][ yTbY ][ trafoDepth ] is equal to 1

  - log2TrafoSize is greater than 2

- Otherwise, splitFlag is set equal to 0.

Depending on the value of splitFlag, the following applies:

–	If splitFlag is equal to 1, the following ordered steps apply:

  - The variables xTb1 and yTb1 are derived as follows:

  - The variable xTb1 is set equal to xTb0 + ( 1  <<  ( log2TrafoSize − 1 ) ).

  - The variable yTb1 is set equal to yTb0 + ( 1  <<  ( log2TrafoSize − 1 ) ).

  - The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xTb0, yTb0 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before deblocking filtering.

  - The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xTb1, yTb0 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before deblocking filtering.

  - The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xTb0, yTb1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before deblocking filtering.

  - The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xTb1, yTb1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before deblocking filtering.

–	Otherwise (splitFlag is equal to 0), the following ordered steps apply:

  - The variable nTbS is set equal to 1  <<  log2TrafoSize.

  - The general intra sample prediction process as specified in subclause 8.4.4.2.1 is invoked with the transform block location ( xTb0, yTb0 ), the intra prediction mode predModeIntra, the transform block size nTbS, and the variable cIdx as inputs, and the output is an (nTbS)x(nTbS) array predSamples.

- If SdcFlag[ xTb0 ][ yTb0 ] is equal to 0, following applies:

      - The scaling and transformation process as specified in subclause 8.6.2 is invoked with the luma location ( xTbY, yTbY ), the variable trafoDepth, the variable cIdx, and the transform size trafoSize set equal to nTbS as inputs, and the output is an (nTbS)x(nTbS) array resSamples.

[Ed. (GT):Meeting notes say: "Implement an enabling flag at (position t.b.d.)". However there seems to  be no decision on the position yet.]

      - If dlt_flag[ nuh_layer_id ] is equal to 1 and predModeIntra is equal to INTRA_DC, INTRA_ANGULAR10, INTRA_ANGULAR26, INTRA_DMM_WFULL, INTRA_DMM_WPREDTEX, INTRA_DMM_CPREDTEX, or INTRA_CHAIN, the following applies, for i in the range of 0 to nTbS − 1, inclusive, and j in the range of 0 to nTbS − 1, inclusive:

          - idx = DepthValue2Idx[ predSamples[ i ][ j ] ] + resSamples[ i ][ j ]		(Error: Reference source not found-2)

          - SL[ xTb0 + i ][ yTb0 + j ] = 
	Idx2DepthValue[ clip3( 0, num_depth_values_in_dlt[ nuh_layer_id ] – 1, idx ) ]	(Error: Reference source not found-3)

      - Otherwise, the following applies:

        - The picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause 8.6.5 is invoked with the transform block location ( xTb0, yTb0 ), the transform block size nTbS, the variable cIdx, the (nTbS)x(nTbS) array predSamples, and the (nTbS)x(nTbS) array resSamples as inputs.

  - If SdcFlag[ xTb0 ][ yTb0 ] is equal to 1) the following ordered steps apply

    - The depth value reconstruction process as specified in subclause Error: Reference source not found is invoked with the location ( xTb0, yTb0 ), the transform size trafoSize set equal to nTbS, the (1 << log2TrafoSize )x(1 << log2TrafoSize) array predSamples, and the intra prediction mode predModeIntra, as the inputs and the output is a (nTbS)x(nTbS) array resSamples.

  - For x in the range of 0 to nTbS − 1 and y in the range of 0 to nTbS − 1, the following applies:

    - When cIdx is equal to 0, ResSamplesL[ xTb0 + x ][ yTb0 + y ] is set equal to 0.

    - When cIdx is equal to 1, ResSamplesCb[ xTb0 + x ][ yTb0 + y ] is set equal to 0.

    - When cIdx is equal to 2, ResSamplesCr[ xTb0 + x ][ yTb0 + y ] is set equal to 0.

# References

[1] X. Zheng, Y. Lin, X. Chen, J. Zheng, CE5.h related: Simplification on SDC planar coding, JCT3V-F0170, Geneva, CH, November, 2013

# Patent rights declaration(s)

Tsinghua University, Hisilicon Technologies and Huawei Technologie may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).