| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0113 |
| --- | --- |

| Title: | CE5: Test results on simplification on SDC planar coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng， Yongbing Lin, Xu Chen, Jianhua Zheng | Tel: Email: | +86-755-28425082 zhengxiaozhen@huawei.com |
| Source: | Hisilicon Technologies |  |  |

_____________________________

# Abstract

This contribution reports CE5 test results of simplification on SDC planar coding. Two simplification methods are provided that can reduce the storage cost of prediction sample block in SDC planar coding. Meanwhile, the removal of 64x64 SDC is also tested in the proposal. Both test cases at common test condition and all intra condition are verified.

# Introduction

Current SDC design is applied to planar and DMM1 modes. For the case of SDC planar mode, the SDC coding blocksize varies from 4x4 to 64x64. Regarding the sample-based SDC design [1], SDC residual is added to the prediction block. Therefore, the prediction sample blocksize for SDC planar mode can be up to 64x64 which is larger than HEVC main profile's maximum intra prediction sample blocksize (32x32). Consequently, current SDC planar coding is not backward compatible with HEVC's design, and the hardware storage of prediction sample block need to be expanded from 32x32 to 64x64. To avoid this additional hardware cost, the contribution JCT3V-F0170 proposes a sub-sample method to reduce the storage cost of prediction sample block in SDC planar coding. According to the decision at 6th JCT3V meeting, the proposed simplification method are required to tested in CE5 as well as the test of the removal of 64x64 SDC.

# Proposed method

The construction of prediction sample block in current planar coding process is illustrated in Figure 1. The neighbouring samples located at the upper row and the left column are used to derive the prediction sample block. Therefore, the prediction sample blocksize is equal to the coding block size, and the maximum prediction sample blocksize can be up to 64x64.

Figure 1: Illustration of the construction of prediction sample block in current SDC planar coding

To avoid the increase of the storage cost of prediction sample block in SDC coding, the 2:1 sub-sampling method is applied to the derivation of the neighboring samples located whose process is show at Figure 2. Then, the sub-sampling neighboring samples are used at the construction process of prediction sample block. Consequently, the prediction sample blocksize is also sub-sample to the half size as the original's size.

For the case of 64x64 coding block, the proposed 2:1 sub-sampling method can reduce the SDC planar prediction sample size to 32x32. Therefore, the storage cost of prediction sample block in SDC planar coding is same as HEVC's design. At the reconstruction process, the prediction sample block after the sub-sampling is used to derive the sub-sample reconstruction block. Then, the value of a sample in the sub-sample reconstruction block is filled in a 2x2 block at the corresponding position in the reconstruction block.

Figure 2: Illustration of the construction of prediction sample block in the proposed SDC planar coding

As alternative design, the first 32 samples from the left-column or the top-row neighbouring samples are used to derive the prediction sample block when the coding blocksize is 64x64. It can also reduce the storage cost of prediction sample block, and make the SDC planar coding be compatible with HEVC’s design. The coding flow is illustrated at Figure 3.

Figure 3: Illustration of the construction of prediction sample block by using the first 32 samples from the left-column and the top-row in SDC planar coding

# Experimental results

The proposed method is implemented on HTM9.0. The coding efficiency is evaluated in random access (CTC) and All-Intra configurations. Three test cases are verified. The first test case only applies the proposed sub-sampling method to SDC planar coding with 64x64 blocksize. The second test case derives the SDC planar prediction sample block by the first 32 samples at left-column or top-row neighbouring samples. The third test case is related to the removal of 64x64 SDC.

Table 1: sub-sampling for 64x64 SDC planar (Test case1, CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | 0.1% | 0.03% | 0.02% | 0.01% | 100.2% | 102.0% | 99.7% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.01% | 0.00% | -0.11% | 100.1% | 99.1% | 100.0% |
| Newspaper_CC | 0.0% | -0.1% | 0.1% | 0.01% | 0.00% | 0.02% | 100.2% | 99.8% | 100.1% |
| GT_Fly | 0.0% | -0.2% | -0.1% | -0.03% | -0.02% | -0.07% | 100.2% | 100.2% | 99.3% |
| Poznan_Hall2 | 0.0% | 0.2% | 0.0% | 0.06% | 0.04% | 0.00% | 100.7% | 100.0% | 100.4% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | -0.01% | -0.01% | -0.03% | 100.6% | 99.8% | 99.6% |
| Undo_Dancer | 0.0% | 0.0% | 0.1% | 0.02% | 0.01% | -0.01% | 100.1% | 100.7% | 100.8% |
| Shark | 0.0% | -0.1% | 0.1% | 0.00% | -0.03% | 0.01% | 100.0% | 99.7% | 100.7% |
| 1024x768 | 0.0% | 0.0% | 0.1% | 0.02% | 0.00% | -0.03% | 100.2% | 100.3% | 99.9% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.01% | 0.00% | -0.02% | 100.3% | 100.1% | 100.2% |
| average | 0.0% | 0.0% | 0.0% | 0.01% | 0.00% | -0.02% | 100.3% | 100.2% | 100.1% |

Table 2: sub-sampling for 64x64 SDC planar (Test case1, AI)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.01% | 100.0% | 100.8% | 100.3% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | 0.00% | 100.0% | 101.3% | 100.5% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.1% | 100.0% | 100.1% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.0% | 99.9% | 99.4% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.2% | 99.2% | 98.0% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.2% | 101.4% | 99.0% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.0% | 100.5% | 99.5% |
| Shark | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | -0.01% | 100.7% | 102.7% | 104.3% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.0% | 100.7% | 100.3% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.2% | 100.7% | 100.0% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.2% | 100.7% | 100.2% |

Table 3: first 32 samples derivation for 64x64 SDC planar (Test case2, CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.0% | 0.00% | 0.01% | 0.04% | 100.1% | 101.9% | 101.0% |
| Kendo | 0.0% | -0.1% | -0.2% | -0.05% | 0.01% | -0.03% | 99.8% | 101.5% | 99.7% |
| Newspaper_CC | 0.0% | -0.1% | 0.0% | 0.00% | -0.01% | 0.00% | 100.0% | 98.6% | 99.7% |
| GT_Fly | 0.0% | 0.0% | -0.2% | -0.01% | 0.06% | -0.02% | 100.0% | 99.8% | 99.2% |
| Poznan_Hall2 | 0.0% | 0.2% | -0.1% | 0.02% | 0.03% | 0.12% | 100.0% | 99.7% | 99.5% |
| Poznan_Street | 0.0% | -0.4% | -0.2% | -0.07% | 0.01% | -0.02% | 100.2% | 100.0% | 100.1% |
| Undo_Dancer | 0.0% | 0.1% | 0.1% | 0.03% | 0.04% | -0.01% | 99.8% | 100.3% | 100.6% |
| Shark | 0.0% | -0.1% | 0.0% | 0.00% | 0.05% | 0.11% | 99.6% | 99.1% | 99.5% |
| 1024x768 | 0.0% | -0.1% | -0.1% | -0.02% | 0.00% | 0.00% | 100.0% | 100.7% | 100.1% |
| 1920x1088 | 0.0% | -0.1% | -0.1% | -0.01% | 0.04% | 0.04% | 99.9% | 99.8% | 99.8% |
| average | 0.0% | -0.1% | -0.1% | -0.01% | 0.02% | 0.02% | 99.9% | 100.1% | 99.9% |

Table 4: first 32 samples derivation for 64x64 SDC planar (Test case2, AI)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | 0.01% | 0.01% | 99.9% | 101.3% | 100.6% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | 0.04% | 0.00% | 99.6% | 101.2% | 100.4% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | 0.02% | 0.01% | 100.2% | 100.2% | 101.1% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | 0.12% | 0.09% | 99.9% | 101.0% | 98.9% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.00% | 0.15% | 0.13% | 99.1% | 99.5% | 98.4% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | 0.11% | 0.09% | 100.6% | 101.7% | 101.0% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | 0.03% | 0.02% | 100.6% | 100.9% | 99.8% |
| Shark | 0.0% | 0.0% | 0.0% | 0.00% | 0.05% | 0.03% | 99.6% | 100.0% | 99.3% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | 0.03% | 0.01% | 99.9% | 100.9% | 100.7% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | 0.09% | 0.07% | 100.0% | 100.6% | 99.5% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | 0.07% | 0.05% | 100.0% | 100.7% | 99.9% |

Table 5: Removal of 64x64 SDC (Test case3, CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.0% | 0.00% | 0.13% | 0.07% | 100.0% | 102.6% | 98.9% |
| Kendo | 0.0% | -0.3% | -0.1% | -0.07% | 0.30% | 0.30% | 99.5% | 100.4% | 100.0% |
| Newspaper_CC | 0.0% | -0.1% | 0.1% | 0.00% | 0.11% | 0.10% | 99.5% | 100.1% | 99.7% |
| GT_Fly | 0.0% | -0.4% | -0.6% | -0.09% | 0.08% | 0.10% | 99.5% | 100.4% | 99.4% |
| Poznan_Hall2 | 0.0% | -0.2% | 0.1% | 0.01% | 0.14% | 0.25% | 99.9% | 101.5% | 99.8% |
| Poznan_Street | 0.0% | -0.4% | -0.3% | -0.09% | 0.04% | 0.01% | 99.9% | 101.2% | 100.4% |
| Undo_Dancer | 0.0% | 0.0% | 0.2% | 0.03% | 0.05% | -0.02% | 99.5% | 100.7% | 100.6% |
| Shark | 0.0% | -0.2% | -0.1% | -0.03% | 0.11% | 0.15% | 99.6% | 100.8% | 100.1% |
| 1024x768 | 0.0% | -0.2% | 0.0% | -0.02% | 0.18% | 0.15% | 99.7% | 101.0% | 99.5% |
| 1920x1088 | 0.0% | -0.2% | -0.1% | -0.04% | 0.08% | 0.10% | 99.7% | 100.9% | 100.0% |
| average | 0.0% | -0.2% | -0.1% | -0.03% | 0.12% | 0.12% | 99.7% | 100.9% | 99.9% |

Table 6: Removal of 64x64 SDC (Test case3, AI)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | 0.16% | 0.23% | 101.8% | 108.8% | 100.5% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | 0.23% | 0.25% | 100.9% | 109.4% | 100.0% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | 0.08% | 0.06% | 99.8% | 106.0% | 100.8% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | 0.30% | 0.27% | 99.4% | 109.1% | 100.5% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.00% | 0.43% | 0.65% | 103.9% | 115.0% | 100.3% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | 0.21% | 0.13% | 100.1% | 106.5% | 98.9% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | 0.05% | 0.09% | 100.1% | 107.7% | 100.2% |
| Shark | 0.0% | 0.0% | 0.0% | 0.00% | 0.26% | 0.26% | 100.0% | 107.0% | 101.0% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | 0.16% | 0.18% | 100.9% | 108.0% | 100.5% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | 0.25% | 0.28% | 100.7% | 109.0% | 100.2% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | 0.22% | 0.24% | 100.8% | 108.7% | 100.3% |

# Conclusion

This contribution suggests applying the sub-sampling to the construction process of prediction sample block in SDC planar mode. Through the test results for CTC and AI, the proposed solution doesn't have impact on all test cases. The storage cost of prediction sample block is reduced. Therefore, it is recommended to incorporate the proposed sub-sampling solution into next version of 3D-HEVC Test Model and HTM9.

# Changes to the description of Test Model

#Test case: f0170-subsample

##### 8.4.4.2.4 Specification of intra prediction mode INTRA_PLANAR

Inputs to this process are:

–	the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

–	a variable nTbS specifying the transform block size.

If nTbS is equal to 64, outputs of this process are the predicted samples predSamples[ x ][ y ], with x, y = 0..31; otherwise outputs of this process are the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

If nTbS is equal to 64, k1 is set to 1 and k2 is set to 2; otherwise k1 is set to 0 and k2 is set to 1. The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..(nTbS >> k1) − 1, are derived as follows:

predSamples[ x ][ y ] = ( ( ( nTbS >> k1 )  − 1 − x ) * p[ −1 ][ k2 * y ] + ( x + 1 ) * p[ nTbS ][ −1 ] + 	( ( nTbS >> k1 )− 1 − y ) * p[k2 * x ][ −1 ] + 		( y + 1 ) * p[ −1 ][ nTbS ] + nTbS )  >>  ( Log2( nTbS >> k1 ) + 1 )	(8-1)

- H.8.4.4.3 Depth value reconstruction process

  - If nTbS is equal to 64, for x, y = 0..31, the reconstructed depth value samples resSamples[ x ][ y ] are derived as specified in the following:

      - dltIdxPred = DepthValue2Idx[ dcPred[ wedgePattern[ x ][ y ] ] ] 	(Error: Reference source not found-2)

      - dltIdxResi = SdcResidual[ xTb ][ yTb ][ wedgePattern[ x ][ y ] ]	(Error: Reference source not found-3)

      - resSamples[ 2 * x ][ 2 * y ] = 	resSamples[ 2 * x + 1 ][ 2 * y ] = resSamples[ 2 * x ][ 2 * y + 1 ] = resSamples[ 2 * x + 1 ][ 2 * y + 1 ] = predSamples[ x ][ y ]  + 	Idx2DepthValue[ dltIdxPred + dltIdxResi ]  − 
	dcPred[ wedgePattern[ x ][ y ] ]	(Error: Reference source not found-4)

  - Otherwise, for x, y = 0..nTbS − 1, the reconstructed depth value samples resSamples[ x ][ y ] are derived as specified in the following:

      - dltIdxPred = DepthValue2Idx[ dcPred[ wedgePattern[ x ][ y ] ] ] 	(Error: Reference source not found-5)

      - dltIdxResi = SdcResidual[ xTb ][ yTb ][ wedgePattern[ x ][ y ] ]	(Error: Reference source not found-6)

      - resSamples[ x ][ y ] =	predSamples[ x ][ y ]  + 	Idx2DepthValue[ dltIdxPred + dltIdxResi ]  − 
	dcPred[ wedgePattern[ x ][ y ] ]	(Error: Reference source not found-7)

#Test case: f0170-firstN

- H.8.4.4.1 General decoding process for intra blocks

–	Otherwise (splitFlag is equal to 0), the following ordered steps apply:

  - If SdcFlag[ xTb0 ][ yTb0 ] is equal to 1 and log2TrafoSize is equal to 6, the variable nTbS is set equal to 1  <<  ( log2TrafoSize – 1), otherwise the variable nTbS is set equal to 1  <<  log2TrafoSize.

  - The general intra sample prediction process as specified in subclause 8.4.4.2.1 is invoked with the transform block location ( xTb0, yTb0 ), the intra prediction mode predModeIntra, the transform block size nTbS, and the variable cIdx as inputs, and the output is an (nTbS)x(nTbS) array predSamples.

  - Depending on SdcFlag[  xTb0 ][ xTb0 ], the following applies:

    - If SdcFlag[ xTb0 ][ yTb0 ] is equal to 0, following applies:

      - The scaling and transformation process as specified in subclause 8.6.2 is invoked with the luma location ( xTbY, yTbY ), the variable trafoDepth, the variable cIdx, and the transform size trafoSize set equal to nTbS as inputs, and the output is an (nTbS)x(nTbS) array resSamples.

      - The picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause 8.6.5 is invoked with the transform block location ( xTb0, yTb0 ), the transform block size nTbS, the variable cIdx, the (nTbS)x(nTbS) array predSamples, and the (nTbS)x(nTbS) array resSamples as inputs.

    - Otherwise ( SdcFlag[ xTb0 ][ yTb0 ] is equal to 1) the following ordered steps apply:

      - The depth value reconstruction process as specified in subclause  is invoked with the location ( xTb0, yTb0 ), the transform size trafoSize set equal to nTbS, the (nTbS)x(nTbS) array predSamples, and the intra prediction mode predModeIntra, as the inputs and the output is a (nTbS)x(nTbS) array resSamples.

- H.8.4.4.3 Depth value reconstruction process

Inputs to this process are:

- a luma location ( xTb, yTb ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

- a variable nTbS specifying the transform block size,

- predicted samples predSamples[ x ][ y ], with x, y =0..clip(0, 31, nTbS – 1)

- the intra prediction mode predModeIntra,

Output of this process is:

- reconstructed depth value samples resSamples[ x ][ y ], with x, y = 0.. nTbS − 1.

  - If nTbS is equal to 64, for x, y = 0..31, the reconstructed depth value samples resSamples[ x ][ y ] are derived as specified in the following:

      - dltIdxPred = DepthValue2Idx[ dcPred[ wedgePattern[ x ][ y ] ] ] 	(Error: Reference source not found-8)

      - dltIdxResi = SdcResidual[ xTb ][ yTb ][ wedgePattern[ x ][ y ] ]	(Error: Reference source not found-9)

      - resSamples[ 2 * x ][ 2 * y ] = 	resSamples[ 2 * x + 1 ][ 2 * y ] = resSamples[ 2 * x ][ 2 * y + 1 ] = resSamples[ 2 * x + 1 ][ 2 * y + 1 ] = predSamples[ x ][ y ]  + 	Idx2DepthValue[ dltIdxPred + dltIdxResi ]  − 
	dcPred[ wedgePattern[ x ][ y ] ]	(Error: Reference source not found-10)

  - Otherwise, for x, y = 0..nTbS − 1, the reconstructed depth value samples resSamples[ x ][ y ] are derived as specified in the following:

      - dltIdxPred = DepthValue2Idx[ dcPred[ wedgePattern[ x ][ y ] ] ] 	(Error: Reference source not found-11)

      - dltIdxResi = SdcResidual[ xTb ][ yTb ][ wedgePattern[ x ][ y ] ]	(Error: Reference source not found-12)

resSamples[ x ][ y ] =	predSamples[ x ][ y ]  + 	Idx2DepthValue[ dltIdxPred + dltIdxResi ]  − 
	dcPred[ wedgePattern[ x ][ y ] ]	(Error: Reference source not found-13)

# Reference

[1] J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei , "JCT3V-D0110 3D-CE6.h related: Sample-based simplified depth coding ", JCT3V-D0110, Incheon, April, 2013

# Patent rights declaration(s)

Hisilicon Technologies and Huawei Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).