| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0114 |
| --- | --- |

| Title: | AHG 10: Complexity Assessment on Illumination Compensation (IC) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Gwo Giun Lee, Bo-Syun Li, Chun-Fu Chen, Zheng-Han Yu. Chun-Hsi Huang No. 1, University Road, Tainan City, Taiwan | Tel: Email: | +886-6-2360663 n28991146@mail.ncku.edu.tw |
| Source: | National Cheng Kung University (NCKU) |  |  |

_____________________________

# Abstract

This proposal provides a case study of complexity assessment on illumination compensation (IC) in 3D-HEVC and complexity metrics, including number of operations, data storage requirement, and data transfer rate are assessed. The analyzed results in this proposal might be considered as anchor complexity to assist JCT3V in evaluating the proposals coming from several proponents in a fair manner. This proposal also presents the experimental results without IC coding tools. The results reveal that the coding performance will be degraded without IC in HTM-9.0 configuration and the coding loss is 0.6%, 0.6%, and 0.4% for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate, respectively, at HTM-9.0 configuration without IC.

# Introduction

The complexity issue has been studied for several decades; however, these studies may analyze the complexity based on the specific platforms. Thus, the assessed complexity will be the platform-dependent and will be varied on different platform since the analyzed results cannot be transparent to another platform. The proposals in previous JCT3V meetings [1]-[6], provide a guideline to evaluate the intrinsic complexity of an algorithm. Thus, this proposal follows the analysis method to extract the intrinsic algorithmic complexity that is platform-independent and transparent to platforms. The coding tool, illumination compensation (IC), would be considered as an exercise to assess its complexity in this proposal.

# Method of Complexity Assessment

To assess the complexity, the flowchart for completing one data granularity should be established first to determine data causality. One important issue should be concerned during the flowchart construction; the major processing or key modules should be identified in this stage in order to achieve the further analysis work. However, the major processing identification would be affected by the analyzer; thus, this proposal also provides a general concept to construct the flowchart. Subsequently, the complexity metrics, such as data storage requirement, and data transfer rate, can be assessed in accordance with the constructed flowchart and the identified major processes or key modules at various data granularities. The data granularity means that amount of data should be performed or produced within one process.

- Number of operations

To estimate the number of operations of an algorithm, the starting point would be mathematical expressions due to the fact that mathematical expressions are more intrinsic to algorithms and mathematical expressions would not be affected by platform. When encountering some if-else conditions, the worst case of conditions would be considered. Some basic operation types are considered only, such as addition, multiplication, and division, due to the fact that some equivalent operations would be treated as identical complexity. E.g., the subtraction can be easily implementation by the addition with 2’s complement representation plus a NOT logic and an incermentor and the shift operations can be implemented with the wire reconnection.

- Data storage requirement

The data storage requirement was assessed according to the flowchart and the data granularity since the data causality has been identified in the flowchart. That is, all the required input data and generated output data should be stored to complete one data granularity and some intermediate data would be stored according to the algorithm. Since the assessed results would like to be transparent to different platforms, the data storage configuration shall not be considered at the initial stage.

- Data transfer rate

The data transfer rate was assessed according to the flowchart and the data granularity since the data causality has been identified in the flowchart. That is, all the required input data and produced output data should be transferred during the process of one data granularity. The intermediate data would not be taken into consideration since these data would be disappeared after finishing the process. In addition, transfer rate of input data and output data shall be discussed individually since the characteristic of sending data and fetching data are different.

In this proposal, the flowchart is constructed according to the specification and the test model [7] and the major process is defined according to that is there any data to be processed or not. The data granularities studied in this proposal is one 6464 block with all supported PU sizes.

# Complexity Assessment

## Algorithm description

According to the specification text [7] and the test model [8], the flowchart of IC in uni-prediction case is shown in Fig 1. The major processing of IC is the calculation of parameter α and β and prediction block generation. According to flowchart, the complexity of IC is assessed at various data granularities and the details are described in the following sections.

Fig 1 Flowchart of current design (HTM-9.0)

## Complexity assessment

This proposal uses three metrics, including number of operations, data storage requirement, data transfer rate, to evaluate the complexity of IC functionality. The to-be-analyzed data granularity is one 6464 block (Largest coding unit), which is composed of various size of prediction units (PUs). In the following sections, the PU size is defined as N×N to clearly provide the algebraic expressions. After obtaining the complexity at PU level, the results can be directly scaled to 6464 block level since the operation of PUs are identical and independent. For current PU, the neighboring pixels on left and above locations are named as Curri for i = 0…N-1, and the corresponding neighboring pixels of reference block are named as Refi for i = 0 …N-1, as shown in Fig 2. In the following assessment, the variable N will be used to express the required complexity in number of operations, data storage requirement, and data transfer rate.

Fig 2 Neighbouring pixels for the derivation of illumination compensation parameters

### Number of operations

- Uni-Prediction:

  - Parameter α and β derivation:

    - Luma: 7N+3 additions, 2N+2 multiplications, 1 division LUT.

    - Chroma: 7(N/2)+3 additions, 2(N/2)+2 multiplications, 1 division LUT.

α=Curr(i)Ref(i)−Curr(i)Ref(i)Ref(i)Ref(i)−Ref(i)Ref(i)

β=i=0N−1Curr(i)−α×i=0N−1Ref(i)

  - Prediction block generation:

    - Luma: N2 additions, N2 multiplications.

    - Chroma: (N/2)2 additions, (N/2)2 multiplications.

PredSamples1[x][y]=α×Ref[x][y]+β

- Bi-Prediction:

In the Bi-prediction case, need to fetch additional corresponding neighboring pixels of reference block and reference block to calculate the additional prediction pixels, which named as PredSamples2. Finally, the prediction block (PredAverage) is derived by averaging PredSamples1 and PredSamples2.

  - Bi-Prediction block generation:

    - Luma: N2 additions, N2 shift.

    - Chroma: (N/2)2 additions, (N/2)2 shift.

PredAverage[x][y]=(PredSamples1[x][y]+PredSamples2[x][y])>>1

Table 1 Algebraic expression of number of operations for one PU

| Type of operations |  | Number of operations |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Uni-Prediction |  |  |  | Bi-Prediction |  |  |
|  |  | Luma |  | Chroma |  | Luma |  | Chroma |
| Add/Sub | N2+7N+3 |  | ((N/2)2+7(N/2)+3)2 |  | (N2+7N+3)2+N2 |  | (((N/2)2+7(N/2)+3)2+(N/2)2)2 |  |
| Shift | - |  | - |  | N2 |  | ((N/2)2) 2 |  |
| Mul. | N2+2N+2 |  | ((N/2)2+2(N/2)+2)2 |  | (N2+2N+2)2 |  | (((N/2)2+2(N/2)+2)2)2 |  |
| Div. LUT | 1 |  | 12 |  | 12 |  | 122 |  |

### Data storage requirement

Input buffer of IC is current and reference neighboring pixels at above and left locations, reference block, and output buffer. There are some intermediate buffers for storing the results of uni-prediction for bi-prediction. The algebraic expressions established according to PU size are listed in .

Table 2 Algebraic expression of data storage requirement

| Data | Storage requirement |  |  |  |
| --- | --- | --- | --- | --- |
|  | Uni-Prediction |  | Bi-Prediction |  |
|  | Luma | Chroma | Luma | Chroma |
| Left and above neighboring current pixels | N | (N/2)2 | N | (N/2)2 |
| Left and above neighboring reference pixels | N | (N/2)2 | 2N | (2N/2)2 |
| Prediction block (Uni-prediction) | N2 | ((N/2)2)2 | N2 | ((N/2)2)2 |
| Prediction block (Bi-prediction) | - | - | N2 | ((N/2)2)2 |
| Total | N2 + 2N | ((N/2)2 + 2(N/2) )2 | 2N2 + 3N | (2(N/2)2 + 3(N/2))2 |

### Data transfer rate

Input data and output data should be transferred in IC are the reference neighboring pixels at above and left location and the reconstructed block. The algebraic expression established according to PU size is listed in Table 3.

Table 3 Algebraic expression of data transfer rate

| Data | Amount of transferred data per PU |  |  |  |
| --- | --- | --- | --- | --- |
|  | Uni-Prediction |  | Bi-Prediction |  |
|  | Luma | Chroma | Luma | Chroma |
| Left and above neighboring current pixels | N | (N/2)2 | N | (N/2)2 |
| Left and above neighboring reference pixels | N | (N/2)2 | 2N | (2N/2)2 |
| Prediction block | N2 | ((N/2)2)2 | N2 | ((N/2)2)2 |
| Total | N2 + 2N | ((N/2)2 + 2(N/2) )2 | N2 + 3N | ((N/2)2 + 3(N/2))2 |

## Overall report

According to the algebraic expressions on number of operations, data storage requirement, and data transfer rate. Table 4, Table 5, and Table 6 show the numeric representations of required resource for one 6464 block with various PU size. Therefore, Table 7 provides an overall report on IC’s complexity at 6464 block level.

Table 4 Number of operations for one 64x64 block with various PU sizes

|  | Number of operations |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Uni-Prediction |  |  |  | Bi-Prediction |  |  |
| PU_size | Add/ Sub | Mul | Div LUT | Add/ Sub |  | Mul | Div LUT |
| 6464 | 7,046 | 6,406 | 3 | 20,236 |  | 12,812 | 6 |
| 3232 | 7,960 | 6,680 | 12 | 22,064 |  | 13,360 | 24 |
| 1616 | 9,824 | 7,264 | 48 | 25,792 |  | 14,528 | 96 |
| 88 | 13,696 | 8,576 | 192 | 33,536 |  | 17,152 | 384 |

Table 5 Data storage requirement for various PU sizes

|  | Data storage requirement (bits) |  |
| --- | --- | --- |
| PU_size | Uni-Prediction | Bi-Prediction |
| 6464 | 6,400 | 12,544 |
| 3232 | 1,664 | 3,200 |
| 1616 | 448 | 832 |
| 88 | 128 | 224 |

Table 6 Data transfer rate for one 6464 block with various PU sizes

|  | Data transfer rate (bits per 6464 block) |  |
| --- | --- | --- |
| PU_size | Uni-Prediction | Bi-Prediction |
| 6464 | 6,400 | 6,528 |
| 3232 | 6,656 | 6,912 |
| 1616 | 7,168 | 7,680 |
| 88 | 8,192 | 9,216 |

Table 7 Overall complexity of IC at 6464 block level

| Complexity Metric | Worst case |  |
| --- | --- | --- |
|  | Uni-Prediction | Bi-Prediction |
| Number of operations (Add/Sub) | 13,696 | 33536 |
| Number of operations (Mul./Div. LUT) | 8,768 | 17,536 |
| Data storage requirement (bits) | 6,400 | 12,544 |
| Data transfer rate (bits per 6464 block) | 8,192 | 9,216 |

## Experimental results

The anchor software is HTM-9.0. This proposal also evaluated coding performance of without IC in HTM-9.0 configuration and results showed in Table 8. The coding loss is 0.6%, 0.6%, and 0.4% for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate at without IC in HTM-9.0 configuration.

Table 8 Experimental results with IC OFF

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 2.6% | 2.2% | 0.9% | 0.9% | 0.6% | 90.9% | 100.0% | 99.9% |
| Kendo | 0.0% | 4.6% | 6.0% | 2.0% | 1.8% | 1.5% | 91.7% | 101.0% | 100.8% |
| Newspaper_CC | 0.0% | 1.2% | 2.5% | 0.8% | 0.7% | 0.5% | 92.4% | 100.8% | 100.0% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.9% | 100.2% | 100.2% |
| Poznan_Hall2 | 0.0% | 1.4% | 4.0% | 1.2% | 1.1% | 0.6% | 93.5% | 95.8% | 99.9% |
| Poznan_Street | 0.0% | -0.1% | 1.2% | 0.2% | 0.2% | 0.1% | 93.4% | 101.0% | 100.0% |
| Undo_Dancer | 0.0% | -0.2% | -0.1% | 0.0% | 0.0% | 0.0% | 94.0% | 99.8% | 100.2% |
| Shark | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.0% | 100.2% | 99.9% |
| 1024x768 | 0.0% | 2.8% | 3.6% | 1.2% | 1.1% | 0.9% | 91.7% | 100.6% | 100.2% |
| 1920x1088 | 0.0% | 0.2% | 1.0% | 0.3% | 0.3% | 0.1% | 96.2% | 99.4% | 100.0% |
| average | 0.0% | 1.2% | 2.0% | 0.6% | 0.6% | 0.4% | 94.5% | 99.8% | 100.1% |

# Conclusion

From Table 1 to Table 7, the complexity of IC is assessed to assist the JCT3V team in evaluating the proposals related to IC. The results can be considered as anchor complexity. Thus, with the help of the intrinsic complexity analysis, this proposal would assist the designer to evaluate the developed coding tools. This proposal provides not only the evaluated complexity of coding tools but also an instruction on that how to assess the intrinsic algorithmic complexity. Due to the fact that the assessed complexity is independent from any platform or architecture, these numbers provide a fair complexity comparison. Hence, this proposal recommends that this analysis method would be taken into consideration for the further coding tool development.

# Reference

- JCT3V-F0010, “JCT-3V AHG Report: Complexity Assessment (AHG10),” Geneva, CH, 25 Oct. - 1 Nov. 2013.

- JCT3V-F0093, “3D-CE3.h: Results on simple merge candidate list construction for 3DV,” Geneva, CH, 25 Oct. - 1 Nov. 2013.

- JCT3V-E0010, “JCT-3V AHG Report: Complexity Assessment (AHG10),” Vienna, AT, 27 July - 2 Aug. 2013.

- JCT3V-E0132, “AHG 10: Complexity Assessment on Simplified Depth Coding (SDC),” Vienna, AT, 27 July - 2 Aug. 2013.

- JCT3V-D0010, “JCT-3V AHG Report: Complexity assessment (AHG10),“ Incheon, KR, 20-26 April 2013.

- JCT3V-D0301, “AHG 10 Overview of Algorithmic Intrinsic Complexity Measures,“ Incheon, KR, 20-26 April 2013.

- JCT3V-F1001, “3D-HEVC Draft Text 2,” Geneva, CH, 25 Oct. - 1 Nov. 2013

- JCT3V-F1005, “Test Model Description of 3D-HEVC and MV-HEVC,” Geneva, CH, 25 Oct. - 1 Nov. 2013.