| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0114 |
| --- | --- |

| Title: | AHG 10: Complexity Assessment on Illumination Compensation (IC) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Gwo Giun Lee, Bo-Syun Li, Chun-Fu Chen, Zheng-Han Yu, Chun-Hsi Huang No. 1, University Road, Tainan City, Taiwan | Tel: Email: | +886-6-2360663 n28991146@mail.ncku.edu.tw |
| Source: | National Cheng Kung University (NCKU) |  |  |

_____________________________

# Abstract

This proposal provides a case study of complexity assessment on illumination compensation (IC) in 3D-HEVC and complexity metrics, including number of operations, data storage requirement, and data transfer rate are assessed. The analyzed results in this proposal might be considered as anchor complexity to assist JCT3V in evaluating the proposals coming from several proponents in a fair manner. This proposal also presents the experimental results without IC coding tools. The results reveal that the coding performance will be degraded without IC in HTM-9.0r1 configuration and the coding loss is 0.6%, 0.6%, and 0.4% for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate, respectively, at HTM-9.0r1 configuration without IC.

# Introduction

The complexity issue has been studied for several decades; however, these studies may analyze the complexity based on the specific platforms. Thus, the assessed complexity will be the platform-dependent and will be varied on different platform since the analyzed results cannot be transparent to another platform. The proposals in previous JCT3V meetings [1]-[6], provide a guideline to evaluate the intrinsic complexity of an algorithm. Thus, this proposal follows the analysis method to extract the intrinsic algorithmic complexity that is platform-independent and transparent to platforms. The coding tool, illumination compensation (IC), would be considered as an exercise to assess its complexity in this proposal.

# Method of Complexity Assessment

To assess the complexity, the flowchart for completing one data granularity should be established first to determine data causality. One important issue should be concerned during the flowchart construction; the major processing or key modules should be identified in this stage in order to achieve the further analysis work. However, the major processing identification would be affected by the analyzer; thus, this proposal also provides a general concept to construct the flowchart. Subsequently, the complexity metrics, such as data storage requirement, and data transfer rate, can be assessed in accordance with the constructed flowchart and the identified major processes or key modules at various data granularities. The data granularity means that amount of data should be performed or produced within one process.

- Number of operations

To estimate the number of operations of an algorithm, the starting point would be mathematical expressions due to the fact that mathematical expressions are more intrinsic to algorithms and mathematical expressions would not be affected by platform. When encountering some if-else conditions, the worst case of conditions would be considered. Some basic operation types are considered only, such as addition, multiplication, and division, due to the fact that some equivalent operations would be treated as identical complexity. E.g., the subtraction can be easily implementation by the addition with 2’s complement representation plus a NOT logic and an incermentor and the shift operations can be implemented with the wire reconnection.

- Data storage requirement

The data storage requirement was assessed according to the flowchart and the data granularity since the data causality has been identified in the flowchart. That is, all the required input data and generated output data should be stored to complete one data granularity and some intermediate data would be stored according to the algorithm. Since the assessed results would like to be transparent to different platforms, the data storage configuration shall not be considered at the initial stage.

- Data transfer rate

The data transfer rate was assessed according to the flowchart and the data granularity since the data causality has been identified in the flowchart. That is, all the required input data and produced output data should be transferred during the process of one data granularity. The intermediate data would not be taken into consideration since these data would be disappeared after finishing the process. In addition, transfer rate of input data and output data shall be discussed individually since the characteristic of sending data and fetching data are different.

In this proposal, the flowchart is constructed according to the specification and the test model [7] and the major process is defined according to that is there any data to be processed or not. The data granularities studied in this proposal is one 6464 block with all supported PU sizes.

# Complexity Assessment

## Algorithm description

According to the specification text [7] and the test model [8], the flowchart of IC in uni-prediction case is shown in Fig. 1. The major processing of IC is the calculation of parameter α and β and prediction block generation. According to flowchart, the complexity of IC is assessed at various data granularities and the details are described in the following sections.

Fig. 1 Flowchart of current design (HTM-9.0r1)

## Complexity assessment

This proposal uses three metrics, including number of operations, data storage requirement, data transfer rate, to evaluate the complexity of IC functionality. The to-be-analyzed data granularity is one 6464 block (Largest coding unit), which is composed of various size of prediction units (PUs). In the following sections, the PU size is defined as N×N to clearly provide the algebraic expressions. After obtaining the complexity at PU level, the results can be directly scaled to 6464 block level since the operation of PUs are identical and independent. For current PU, the neighboring pixels on left and above locations are named as Curri for i = 0…N-1, and the corresponding neighboring pixels of reference block are named as Refi for i = 0 …N-1, as shown in Fig. 2. In the following assessment, the variable N will be used to express the required complexity in number of operations, data storage requirement, and data transfer rate.

Fig. 2 Neighbouring pixels used for the derivation of illumination compensation parameters

### Number of operations

- Uni-Prediction:

  - Compensation model derivation, α and β:

α=Curr(i)Ref(i)−Curr(i)Ref(i)Ref(i)Ref(i)−Ref(i)Ref(i)        β=i=0N−1Curr(i)−α×i=0N−1Ref(i)

    - Luma: 4N-1 additions/subtractions, 2N+3 multiplications, 1 division LUT.

    - Chroma: 4(N/2)-1 additions/subtractions, 2(N/2)+3 multiplications, 1 division LUT.

  - Prediction block generation:

PredSamples1[x][y]=α×Ref[x][y]+β

    - Luma: N2 additions, N2 multiplications.

    - Chroma: (N/2)2 additions, (N/2)2 multiplications.

- Bi-Prediction:

In the Bi-prediction case, need to fetch additional corresponding neighboring pixels of reference block and reference block to calculate the additional prediction pixels, which named as PredSamples2. Finally, the prediction block (PredAverage) is derived by averaging PredSamples1 and PredSamples2.

  - Bi-Prediction block generation:

PredAverage[x][y]=(PredSamples1[x][y]+PredSamples2[x][y])>>1

    - Luma: N2 additions, N2 shift.

    - Chroma: (N/2)2 additions, (N/2)2 shift.

The algebraic expressions established according to PU size are listed in Table 1 and Table 2.

Table 1 Algebraic expressions of number of operations for one PU (Uni-prediction case)

| Type of operations |  | Number of operations |  |  |
| --- | --- | --- | --- | --- |
|  |  | Luma |  | Chroma |
| Add/Sub | Compensation model derivation: 4(N-1)+3 Pred. block generation: N2 Total: N2+4N-1 |  | Compensation model derivation: (4(N/2-1)+3)2 Pred. block generation: ((N/2)2)2 Total: ((N/2)2+4(N/2)-1)2 |  |
| Mul. | Compensation model derivation: 2N+3 Pred. block generation: N2 Total: N2+2N+3 |  | Compensation model derivation: (2(N/2)+3)2 Pred. block generation: ((N/2)2)2 Total: ((N/2)2+2(N/2)+3)2 |  |
| Div. LUT | 1 |  | 12 |  |

Table 2 Algebraic expressions of number of operations for one PU (Bi-prediction case)

| Type of operations |  | Number of operations |  |  |
| --- | --- | --- | --- | --- |
|  |  | Luma |  | Chroma |
| Add/Sub | Two uni-predictions: (N2+4N-1)2 Pred. block averaging: N2 Total: (N2+4N-1)2+N2 |  | Two uni-prediction: (((N/2)2+4(N/2)-1)2)2 Pred. block averaging: ((N/2)2)2 Total: (((N/2)2+4(N/2)-1)2)2+((N/2)2)2 |  |
| Shift | Pred. block averaging: N2 |  | Pred. block averaging: ((N/2)2)2 |  |
| Mul. | Two uni-predictions: (N2+2N+3)2 Total: (N2+2N+3)2 |  | Two uni-predictions: (((N/2)2+2(N/2)+3)2)2 Total: (((N/2)2+2(N/2)+3)2)2 |  |
| Div. LUT | 12 |  | 122 |  |

### Data storage requirement

Input buffer of IC is current and reference neighboring pixels at above and left locations, reference block, and output buffer. There are some intermediate buffers for storing the results of uni-prediction for bi-prediction; in addition, only Luma component is counted since the storage for Luma could be reused for Chroma component directly. The algebraic expressions established according to PU size is listed in Table 3.

Table 3 Algebraic expressions of data storage requirement

| Data | Storage requirement |  |
| --- | --- | --- |
|  | Uni-Prediction | Bi-Prediction |
| Left and above pixels of current block | Left neighboring pixels: (N/2) Above neighboring pixels: (N/2) Total: (N/2)+(N/2) | Left neighboring pixels: (N/2) Above neighboring pixels: (N/2) Total: (N/2)+(N/2) |
| Left and above pixels of reference block | Left neighboring pixels: (N/2) Above neighboring pixels: (N/2) Total: (N/2)+(N/2) | Left neighboring pixels: ((N/2))2 Above neighboring pixels: ((N/2))2 Total: ((N/2)+(N/2))2 |
| Reference block | Total: N2 | Total: (N2)2 |
| Total | N2 + 2N | 2N2+3N |

### Data transfer rate

Input data and output data should be transferred in IC are the reference neighboring pixels at above and left location and the reconstructed block. The algebraic expressions established according to PU size are listed in Table 4 and Table 5.

Table 4 Algebraic expressions of data transfer rate (Uni-prediction case)

| Data | Storage requirement |  |
| --- | --- | --- |
|  | Luma | Chroma |
| Left and above pixels of current block | Left neighboring pixels: (N/2) Above neighboring pixels: (N/2) Total: (N/2)+(N/2) | Left neighboring pixels: (((N/2)/2))2 Above neighboring pixels: (((N/2)/2))2 Total: (((N/2)/2))2+(((N/2)/2))2 |
| Left and above pixels of reference block | Left neighboring pixels: (N/2) Above neighboring pixels: (N/2) Total: (N/2)+(N/2) | Left neighboring pixels: (((N/2)/2))2 Above neighboring pixels: (((N/2)/2))2 Total: (((N/2)/2))2+(((N/2)/2))2 |
| Reference block | Total: N2 | Total: ((N/2)2)2 |
| Prediction block | Total: N2 | Total: ((N/2)2)2 |
| Total | 2N2 + 2N | (2(N/2)2+2(N/2))2 |

Table 5 Algebraic expressions of data transfer rate (Bi-prediction case)

| Data | Storage requirement |  |
| --- | --- | --- |
|  | Luma | Chroma |
| Left and above pixels of current block | Left neighboring pixels: (N/2) Above neighboring pixels: (N/2) Total: (N/2)+(N/2) | Left neighboring pixels: (((N/2)/2))2 Above neighboring pixels: (((N/2)/2))2 Total: (((N/2)/2))2+(((N/2)/2))2 |
| Left and above pixels of reference block | Left neighboring pixels: ((N/2))2 Above neighboring pixels: ((N/2))2 Total: ((N/2)+(N/2))2 | Left neighboring pixels: ((((N/2)/2))2)2 Above neighboring pixels: ((((N/2)/2))2)2 Total: ((((N/2)/2))2+(((N/2)/2))2)2 |
| Reference block | Total: (N2)2 | Total: (((N/2)2)2)2 |
| Prediction block | Total: N2 | Total: ((N/2)2)2 |
| Total | 3N2+3N | (3(N/2)2+3(N/2))2 |

## Overall report

According to the algebraic expressions on number of operations, data storage requirement, and data transfer rate. Table 6, Table 7, and Table 8 show the numeric representations of required resource for one 6464 block with various PU size. Therefore, Table 9 provides an overall report on IC’s complexity at 6464 block level.

Table 6 Number of operations for one 64x64 block with various PU sizes

|  | Number of operations |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Uni-Prediction |  |  |  | Bi-Prediction |  |  |
| PU_size | Add/ Sub | Mul | Div LUT | Add/ Sub |  | Mul | Div LUT |
| 6464 | 6,650 | 6,409 | 3 | 19,444 |  | 12,818 | 6 |
| 3232 | 7,144 | 6,692 | 12 | 20,432 |  | 13,384 | 24 |
| 1616 | 8,096 | 7,312 | 48 | 22,336 |  | 14,624 | 96 |
| 88 | 6,080 | 5,312 | 64 | 16,256 |  | 10,624 | 128 |

Table 7 Data storage requirement for various PU sizes

|  | Data storage requirement (bits) |  |
| --- | --- | --- |
| PU_size | Uni-Prediction | Bi-Prediction |
| 6464 | 33,792 | 67,072 |
| 3232 | 8,704 | 17,152 |
| 1616 | 2,304 | 4,480 |
| 88 | 640 | 1,216 |

Table 8 Data transfer rate for one 6464 block with various PU sizes

|  | Data transfer rate (bits per 6464 block) |  |
| --- | --- | --- |
| PU_size | Uni-Prediction | Bi-Prediction |
| 6464 | 100,352 | 150,528 |
| 3232 | 102,400 | 153,600 |
| 1616 | 106,496 | 159,744 |
| 88 | 73,728 | 110,592 |

Table 9 Overall complexity of IC at 6464 block level

| Complexity Metric | Worst case |  |
| --- | --- | --- |
|  | Uni-Prediction | Bi-Prediction |
| Number of operations (Add/Sub) | 8,096 | 22,336 |
| Number of operations (Mul.) | 7,312 | 14,624 |
| Number of operations (Div. LUT) | 64 | 128 |
| Data storage requirement (bits) | 33,792 | 67,072 |
| Data transfer rate (bits per 6464 block) | 106,496 | 159,744 |

## Experimental results

The anchor software is HTM-9.0r1. This proposal also evaluated coding performance of without IC in HTM-9.0r1 configuration and results showed in Table 10. The coding loss is 0.6%, 0.6%, and 0.4% for video PSNR vs. video bitrate, video PSNR vs. total bitrate, and synth PSNR vs. total bitrate at without IC in HTM-9.0r1 configuration.

Table 10 Experimental results with IC OFF

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 2.6% | 2.2% | 0.9% | 0.9% | 0.6% | 90.8% | 100.2% | 99.9% |
| Kendo | 0.0% | 4.6% | 6.0% | 2.0% | 1.8% | 1.5% | 91.4% | 100.4% | 101.0% |
| Newspaper_CC | 0.0% | 1.2% | 2.5% | 0.8% | 0.7% | 0.6% | 92.0% | 100.9% | 99.5% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 98.4% | 95.7% | 96.9% |
| Poznan_Hall2 | 0.0% | 1.4% | 4.0% | 1.2% | 1.1% | 0.7% | 93.3% | 95.7% | 100.1% |
| Poznan_Street | 0.0% | -0.1% | 1.2% | 0.2% | 0.2% | 0.1% | 92.2% | 97.0% | 96.9% |
| Undo_Dancer | 0.0% | -0.2% | -0.1% | 0.0% | 0.0% | 0.0% | 94.3% | 101.9% | 103.0% |
| Shark | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.3% | 101.7% | 102.9% |
| 1024x768 | 0.0% | 2.8% | 3.6% | 1.2% | 1.1% | 0.9% | 91.4% | 100.5% | 100.1% |
| 1920x1088 | 0.0% | 0.2% | 1.0% | 0.3% | 0.3% | 0.2% | 95.7% | 98.4% | 100.0% |
| average | 0.0% | 1.2% | 2.0% | 0.6% | 0.6% | 0.4% | 94.1% | 99.2% | 100.0% |

# Conclusion

From Table 1 to Table 9, the complexity of IC is assessed to assist the JCT3V team in evaluating the proposals related to IC. The results can be considered as anchor complexity. Thus, with the help of the intrinsic complexity analysis, this proposal would assist the designer to evaluate the developed coding tools. This proposal provides not only the evaluated complexity of coding tools but also an instruction on that how to assess the intrinsic algorithmic complexity. Due to the fact that the assessed complexity is independent from any platform or architecture, these numbers provide a fair complexity comparison. Hence, this proposal recommends that this analysis method would be taken into consideration for the further coding tool development.

# Reference

- JCT3V-F0010, “JCT-3V AHG Report: Complexity Assessment (AHG10),” Geneva, CH, 25 Oct. - 1 Nov. 2013.

- JCT3V-F0093, “3D-CE3.h: Results on simple merge candidate list construction for 3DV,” Geneva, CH, 25 Oct. - 1 Nov. 2013.

- JCT3V-E0010, “JCT-3V AHG Report: Complexity Assessment (AHG10),” Vienna, AT, 27 July - 2 Aug. 2013.

- JCT3V-E0132, “AHG 10: Complexity Assessment on Simplified Depth Coding (SDC),” Vienna, AT, 27 July - 2 Aug. 2013.

- JCT3V-D0010, “JCT-3V AHG Report: Complexity assessment (AHG10),“ Incheon, KR, 20-26 April 2013.

- JCT3V-D0301, “AHG 10 Overview of Algorithmic Intrinsic Complexity Measures,“ Incheon, KR, 20-26 April 2013.

- JCT3V-F1001, “3D-HEVC Draft Text 2,” Geneva, CH, 25 Oct. - 1 Nov. 2013

- JCT3V-F1005, “Test Model Description of 3D-HEVC and MV-HEVC,” Geneva, CH, 25 Oct. - 1 Nov. 2013.

# Appendix – Complexity assessment on motion compensation plus illumination compensation

In the main body of this proposal, the complexity of IC has been displayed and this proposal provides the complexity assessment from the view point of IC alone, i.e. the red block in Fig. 3. On the other hand, IC is additional process to the normal motion compensation (MC) process; therefore, the appendix provides the complexity assessment from the view point of MC plus IC, i.e. the blue block in Fig. 3. The results show that there are lots of data could be reused when MC and IC are considered together. For example, the reference block produced by MC could be directly transferred to IC and hence it will not be counted as data transfer in complexity assessment; consequently, the results of data storage requirement would not be increased a lot because only the extended taps for interpolation are required.

Fig. 3 Flowchart of motion compensation plus illumination compensation

The following section provides the additional complexity when MC and IC are considered together in the complexity assessment with respect to IC alone. The complexity accompanied with MC will be highlighted in yellow color.

- Number of operations: MC will generate the reference block via 8-tap filter for Luma and 4-tap filter for Chroma. When the two-dimensional filter is computed separately, the computation load could be reduced significantly by storing the intermediate data for reusing.

  - Reference block generation:

Luma filter: [-1, 4, -11, 40, 40, -11, 4, -1]                Chroma filter: [-2 58 10 -2]

    - Luma: 7N(N+7)+7N2 additions, 6N(N+7)+6N2 multiplications, N(N+7)+N2 shifts

    - Chroma: 3(N/2)((N/2)+3)+3(N/2)2 additions, 4(N/2)((N/2)+3)+4(N/2)2 multiplications, (N/2)((N/2)+3)+(N/2)2 shifts

On the other hand, if the intermediate is not stored, all the data should be recomputed; hence the algebraic expressions for Luma and Chroma are:

    - Luma: 97N2 additions, 96N2 multiplications, 92N2 shifts

    - Chroma: 53N2 additions, 54N2 multiplications, 52N2 shifts

The algebraic expressions of number of operations with and without intermediate data reuse are established according to PU size are listed in Table 11, Table 12, Table 16, and Table 17.

- Data storage requirement: MC will fetch the data with the size of PU size plus 7 extended pixels; therefore, the data storage requirement of reference block will be changed. In addition, if the intermediate data are stored for reducing the computation, one additional buffer is required. The algebraic expressions of data storage requirement with and without intermediate data reuse are established according to PU size are listed in Table 13 and Table 18.

  - Reference block with extended taps:

    - (N+7)(N+7) pixels

  - Buffer for the intermediate data to store the interpolated results by horizontal filter

    - N(N+7) pixels

- Data transfer rate: MC will fetch the with the size of PU size plus 7 extended pixels in both horizontal and vertical directions for Luma component and the half size of PU width plus 3 extended pixels in both horizontal and vertical directions for Chroma component. Therefore, the data transfer rate will be changed. However, the data transfer rate does not be influenced by reusing the intermediate data; thus, the complexities of reusing intermediate data or not are identical. The algebraic expressions of data transfer rate are established according to PU size are listed in Table 14 and Table 15.

  - Reference block with extended taps,

    - (N+7)(N+7) pixels are required for Luma component

    - (N+3)(N+3) pixels are required for Chroma component

The summary report with and without intermediate data reuse are illustrated in Table 19, Table 20, Table 21, Table 22, Table 23, Table 24, and Table 25.

Table 11 Algebraic expressions of number of operations for one PU (Uni-prediction case, intermediate data is reused)

| Type of operations |  | Number of operations |  |  |
| --- | --- | --- | --- | --- |
|  |  | Luma |  | Chroma |
| Add/Sub | Reference block generation: 7N(N+7)+7N2 Compensation model derivation: 4(N-1)+3 Pred. block generation: N2 Total: 15N2+53N-1 |  | Reference block generation: (3(N/2)((N/2)+3)+3(N/2)2)2 Compensation model derivation: (4(N/2-1)+3)2 Pred. block generation: ((N/2)2)2 Total: (7(N/2)2+13(N/2)-1)2 |  |
| Shift | Reference block generation: N(N+7)+N2 |  | Reference block generation: ((N/2)((N/2)+3)+(N/2)2)2 |  |
| Mul. | Reference block generation: 6N(N+7)+6N2 Compensation model derivation: 2N+3 Pred. block generation: N2 Total: 13N2+44N+3 |  | Reference block generation: (4(N/2)((N/2)+3)+4(N/2)2)2 Compensation model derivation: (2(N/2)+3)2 Pred. block generation: ((N/2)2)2 Total: (9(N/2)2+14(N/2)+3)2 |  |
| Div. LUT | 1 |  | 12 |  |

Table 12 Algebraic expressions of number of operations for one PU (Bi-prediction case, intermediate data is reused)

| Type of operations |  | Number of operations |  |  |
| --- | --- | --- | --- | --- |
|  |  | Luma |  | Chroma |
| Add/Sub | Two uni-predictions: (15N2+53N-1)2 Pred. block averaging: N2 Total: (15N2+53N-1)2+N2 |  | Two uni-prediction: ((7(N/2)2+13(N/2)-1)2)2 Pred. block averaging: ((N/2)2)2 Total: ((16(N/2)2+4(N/2)-1)2)2+((N/2)2)2 |  |
| Shift | Two uni-predictions: (2N2+7N)2 Pred. block averaging: N2 |  | Two uni-prediction: ((2(N/2)2+3(N/2))2)2 Pred. block averaging: ((N/2)2)2 |  |
| Mul. | Two uni-predictions: (13N2+44N+3)2 Total: (13N2+44N+3)2 |  | Two uni-predictions: ((9(N/2)2+14(N/2)+3)2)2 Total: ((9(N/2)2+14(N/2)+3)2)2 |  |
| Div. LUT | 12 |  | 122 |  |

Table 13 Algebraic expressions of data storage requirement (Intermediate data is reused)

| Data | Storage requirement |  |
| --- | --- | --- |
|  | Uni-Prediction | Bi-Prediction |
| Left and above pixels of current block | Left neighboring pixels: (N/2) Above neighboring pixels: (N/2) Total: (N/2)+(N/2) | Left neighboring pixels: (N/2) Above neighboring pixels: (N/2) Total: (N/2)+(N/2) |
| Left and above pixels of reference block | Left neighboring pixels: (N/2) Above neighboring pixels: (N/2) Total: (N/2)+(N/2) | Left neighboring pixels: ((N/2))2 Above neighboring pixels: ((N/2))2 Total: ((N/2)+(N/2))2 |
| Buffer for intermediate data | N(N+7) | N(N+7) |
| Reference block | Total: (N+7)2 | Total: ((N+7)2)2 |
| Total | (N+7)2+N2+2N | 2(N+7)2+N2+3N |

Table 14 Algebraic expressions of data transfer rate (Uni-prediction case)

| Data | Storage requirement |  |
| --- | --- | --- |
|  | Luma | Chroma |
| Left and above pixels of current block | Left neighboring pixels: (N/2) Above neighboring pixels: (N/2) Total: (N/2)+(N/2) | Left neighboring pixels: (((N/2)/2))2 Above neighboring pixels: (((N/2)/2))2 Total: (((N/2)/2))2+(((N/2)/2))2 |
| Left and above pixels of reference block | Left neighboring pixels: (N/2) Above neighboring pixels: (N/2) Total: (N/2)+(N/2) | Left neighboring pixels: (((N/2)/2))2 Above neighboring pixels: (((N/2)/2))2 Total: (((N/2)/2))2+(((N/2)/2))2 |
| Reference block | Total: (N+7)2 | Total: ((N/2+3)2)2 |
| Prediction block | Total: (N)2 | Total: ((N/2)2)2 |
| Total | (N+7)2+N2+2N | ((N/2+3)2+(N/2)2+2(N/2))2 |

Table 15 Algebraic expressions of data transfer rate (Bi-prediction case)

| Data | Storage requirement |  |
| --- | --- | --- |
|  | Luma | Chroma |
| Left and above pixels of current block | Left neighboring pixels: (N/2) Above neighboring pixels: (N/2) Total: (N/2)+(N/2) | Left neighboring pixels: (((N/2)/2))2 Above neighboring pixels: (((N/2)/2))2 Total: (((N/2)/2))2+(((N/2)/2))2 |
| Left and above pixels of reference block | Left neighboring pixels: ((N/2))2 Above neighboring pixels: ((N/2))2 Total: ((N/2)+(N/2))2 | Left neighboring pixels: ((((N/2)/2))2)2 Above neighboring pixels: ((((N/2)/2))2)2 Total: ((((N/2)/2))2+(((N/2)/2))2)2 |
| Reference block | Total: ((N+7)2)2 | Total: (((N/2+3)2)2)2 |
| Prediction block | Total: N2 | Total: ((N/2)2)2 |
| Total | 2(N+7)2+N2+3N | (2(N/2+3)2+(N/2)2+3(N/2))2 |

Table 16 Algebraic expressions of number of operations for one PU (Uni-prediction case, intermediate data isn’t reuse)

| Type of operations |  | Number of operations |  |  |
| --- | --- | --- | --- | --- |
|  |  | Luma |  | Chroma |
| Add/Sub | Reference block generation: 97N2 Compensation model derivation: 4(N-1)+3 Pred. block generation: N2 Total: 64N2+4N-1 |  | Reference block generation: (53(N/2)2)2 Compensation model derivation: (4(N/2-1)+3)2 Pred. block generation: ((N/2)2)2 Total: (16(N/2)2+4(N/2)-1)2 |  |
| Shift | Reference block generation: 92N2 |  | Reference block generation: (52(N/2)2)2 |  |
| Mul. | Reference block generation: 96N2 Compensation model derivation: 2N+3 Pred. block generation: N2 Total: 55N2+2N+3 |  | Reference block generation: (54(N/2)2)2 Compensation model derivation: (2(N/2)+3)2 Pred. block generation: ((N/2)2)2 Total: (21(N/2)2+2(N/2)+3)2 |  |
| Div. LUT | 1 |  | 12 |  |

Table 17 Algebraic expressions of number of operations for one PU (Bi-prediction case, intermediate data isn’t reuse)

| Type of operations |  | Number of operations |  |  |
| --- | --- | --- | --- | --- |
|  |  | Luma |  | Chroma |
| Add/Sub | Two uni-predictions: (64N2+4N-1)2 Pred. block averaging: N2 Total: (64N2+4N-1)2+N2 |  | Two uni-prediction: ((16(N/2)2+4(N/2)-1)2)2 Pred. block averaging: ((N/2)2)2 Total: ((16(N/2)2+4(N/2)-1)2)2+((N/2)2)2 |  |
| Shift | Two uni-predictions: (92N2)2 Pred. block averaging: N2 |  | Two uni-prediction: ((52(N/2)2)2)2 Pred. block averaging: ((N/2)2)2 |  |
| Mul. | Two uni-predictions: (55N2+2N+3)2 Total: (55N2+2N+3)2 |  | Two uni-predictions: ((21(N/2)2+2(N/2)+3)2)2 Total: ((21(N/2)2+2(N/2)+3)2)2 |  |
| Div. LUT | 12 |  | 122 |  |

Table 18 Algebraic expressions of data storage requirement (Intermediate data isn’t reused)

| Data | Storage requirement |  |
| --- | --- | --- |
|  | Uni-Prediction | Bi-Prediction |
| Left and above pixels of current block | Left neighboring pixels: (N/2) Above neighboring pixels: (N/2) Total: (N/2)+(N/2) | Left neighboring pixels: (N/2) Above neighboring pixels: (N/2) Total: (N/2)+(N/2) |
| Left and above pixels of reference block | Left neighboring pixels: (N/2) Above neighboring pixels: (N/2) Total: (N/2)+(N/2) | Left neighboring pixels: ((N/2))2 Above neighboring pixels: ((N/2))2 Total: ((N/2)+(N/2))2 |
| Reference block | Total: (N+7)2 | Total: ((N+7)2)2 |
| Total | (N+7)2+2N | 2(N+7)2+3N |

Table 19 Number of operations for one 64x64 block with various PU sizes (Intermediate data is reused)

|  | Number of operations |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Uni-Prediction |  |  |  | Bi-Prediction |  |  |
| PU_size | Add/ Sub | Mul | Div LUT | Add/ Sub |  | Mul | Div LUT |
| 6464 | 73,565 | 66,825 | 3 | 153,274 |  | 133,650 | 6 |
| 3232 | 77,492 | 70,180 | 12 | 161,128 |  | 140,360 | 24 |
| 1616 | 85,328 | 76,944 | 48 | 176,800 |  | 153,888 | 96 |
| 88 | 96,960 | 87,232 | 64 | 198,016 |  | 174,464 | 128 |

Table 20 Number of operations for one 64x64 block with various PU sizes (Intermediate data isn’t reused)

|  | Number of operations |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Uni-Prediction |  |  |  | Bi-Prediction |  |  |
| PU_size | Add/ Sub | Mul | Div LUT | Add/ Sub |  | Mul | Div LUT |
| 6464 | 295,421 | 268,553 | 3 | 596,986 |  | 537,106 | 6 |
| 3232 | 295,924 | 268,836 | 12 | 597,992 |  | 537,672 | 24 |
| 1616 | 296,912 | 269,456 | 48 | 599,968 |  | 538,912 | 96 |
| 88 | 294,848 | 267,456 | 64 | 593,792 |  | 534,912 | 128 |

Table 21 Data storage requirement for various PU sizes (Intermediate data is reused)

|  | Data storage requirement (bits) |  |
| --- | --- | --- |
| PU_size | Uni-Prediction | Bi-Prediction |
| 6464 | 77,704 | 77,704 |
| 3232 | 22,664 | 22,664 |
| 1616 | 7,432 | 7,432 |
| 88 | 2,888 | 2,888 |

Table 22 Data storage requirement for various PU sizes (Intermediate data isn’t reused)

|  | Data storage requirement (bits) |  |
| --- | --- | --- |
| PU_size | Uni-Prediction | Bi-Prediction |
| 6464 | 41,352 | 82,192 |
| 3232 | 12,680 | 25,104 |
| 1616 | 4,488 | 8,848 |
| 88 | 1,928 | 3,792 |

Table 23 Data transfer rate for one 6464 block with various PU sizes

|  | Data transfer rate (bits per 6464 block) |  |
| --- | --- | --- |
| PU_size | Uni-Prediction | Bi-Prediction |
| 6464 | 111,128 | 172,080 |
| 3232 | 125,024 | 198,848 |
| 1616 | 156,032 | 258,816 |
| 88 | 172,544 | 291,840 |

Table 24 Overall complexity of MC+IC at 6464 block level (Intermediate data is reused)

| Complexity Metric | Worst case |  |
| --- | --- | --- |
|  | Uni-Prediction | Bi-Prediction |
| Number of operations (Add/Sub) | 96,960 | 198,016 |
| Number of operations (Mul.) | 87,232 | 174,464 |
| Number of operations (Div. LUT) | 64 | 128 |
| Data storage requirement (bits) | 77,704 | 118,544 |
| Data transfer rate (bits per 6464 block) | 172,544 | 291,840 |

Table 25 Overall complexity of MC+IC at 6464 block level (Intermediate data isn’t reused)

| Complexity Metric | Worst case |  |
| --- | --- | --- |
|  | Uni-Prediction | Bi-Prediction |
| Number of operations (Add/Sub) | 296,912 | 599,968 |
| Number of operations (Mul.) | 269,456 | 538,912 |
| Number of operations (Div. LUT) | 64 | 128 |
| Data storage requirement (bits) | 41,352 | 82,192 |
| Data transfer rate (bits per 6464 block) | 172,544 | 291,840 |