| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0115r1 |
| --- | --- |

| Title: | Preliminary Results of MFC plus Depth |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lucia Grimaldi, Klaas Schueuer Rheinstrasse 45, Aufgang 1 Berlin, Germany Peng Yin, Taoran Lu, Hariharan Ganapathy, Tao Chen, Walt Husak 432 Lakeside Dr. Sunnyvale CA 94085 USA | Tel: Email: | +1 408 330 3251 {pyin,kschu,wjh}@dolby.com |
| Source: | Dolby Laboratories Inc. |  |  |

_____________________________

# Abstract

This contribution shows a comparison between MFC plus depth (MFC+D) and FC plus depth (FC+D) in multi-view rendering. All tests have been performed without compression. The study aims at providing some upper bound on the expected gain. The texture views are given by the upsampled FC left and right views for FC+D case and by the reconstructed MFC left and right views for the MFC+D case, respectively. The original full resolution depth data is used in all the tests. The simulation results show that the average PSNR gain of synthesized views is in a similar range to that of original views. In addition, subjective evaluation justifies the objective gain.

# Introduction

Two most commonly used data and compression formats for stereoscopic video coding are Frame-Compatible (FC) coding and Multiview Video Coding (MVC). For glasses free auto-stereoscopic display application, it is desirable to adopt Multiview Video plus Depth (MVD) format to enable depth-image-based rendering of additional viewpoints at the decoder side. Multiview video coding with depth information (MVC+D) is standardized in Annex I of AVC-based coding technology which specifies an MVC-compatible MVD coding format [1]. This technology can support both FC plus depth format (FC+D) as well as stereo video coding plus depth format. It defines only one profile, Multiview Depth High Profile, which is backward compatible to stereo MVC.

FC coding is widely used in satellite broadcast and cable services, where MVC finds limited deployment due to lack of playback devices (e.g., STBs) equipped with MVC decoders. It is well known that FC coding sacrifices the spatial resolution by low-pass filtering and sub-sampling, usually by half for each view. The benefit is to maintain compatibility with existing single layer AVC codecs and currently deployed infrastructures. In order to deliver the original picture resolution while maintaining backward compatibility, a spatial scalable extension of FC has been standardized. The Multi-resolution Frame Compatible Stereo coding (MFC), is part of Annex H of AVC-based coding technology [2].

Multi-view autostereoscopic 3D (AS3D) display technology has been improving over the years and has started showing promises in applications for home entertainment. When an HD panel was used in multi-view AS3D display, it has limited accommodation of the spatial resolution for each view. Lately, 4k and 8k panels have become common in AS3D displays. This significantly improves the spatial resolution of each view that can be accommodated and perceived. Apparently this calls for an input of higher quality from video compression prior to multi-view rendering. However, FC by its nature sacrifices the quality of each view and that may limit the quality of the synthesized views, as shown in Figure 2. It is advantageous to deliver higher resolution pictures in stereoscopic video coding and fully exploit the benefit of high quality multi-view rendering and display on higher resolution panels. Therefore, it is desirable to define a new profile MFC Depth High in Annex I to support the MFC+D format which is backward compatible to MFC.

# MFC compatible extension including depth (MFC+D)

The MFC+D format is proposed to follow the same philosophy as MVC+D [1]. The texture view coding is compatible with MFC high profile and a new profile MFC Depth High (profile_id 135) is to be introduced for coding of multiview depth map data in Annex I. Multiview texture and depth data are coded independently, and no restrictions on strict association between texture and depth map are imposed. The depth coding follows the same approach as already defined for MVC+D in Annex I. This requires minimal changes in Annex I specification to support MFC Depth High Profile. The draft specification text with change marks enabled is attached with this contribution. An illustration of MFC+D data is shown in Figure 1.

Figure 1 Illustration of MFC+D

# Preliminary results of MFC+D

## Experiment Setup

It is shown in [3] that MFC provides noticeable coding efficiency improvement compared to FC coding. Based on both objective and subjective evaluation, in the case of 25% bitrate overhead for the enhancement layer (EL), MFC delivers better picture quality than FC. The gain becomes more significant when the overhead allocated to EL is higher. It is desirable to investigate whether such coding efficiency can be maintained for synthesized views. Independent of compression, it is interesting and informative to have some upper bound on the expected gain.

The setup of different test cases is shown in . The texture views are replaced by upsampled FC left and right views for FC+D case and the reconstructed MFC left and right views for MFC+D case, respectively. Depth is always at its original full resolution. The test conditions follow the 3DV common test conditions for 2-view case [4], except that no compression is involved.  specifies the multiview test sequences with associated depth data, and the corresponding input and output views. 3 intermediate views (0.25, 0.5 and 0.75) are synthesized. The FC format for each sequence is indicated in the last column of . PSNR is computed for both videos (left and right views) and intermediate views (synthesized views).

Table 1 Test Cases

| Test cases | Video (texture views) | Depth |
| --- | --- | --- |
| Reference | Original full resolution left/right views | Original full resolution left/right depths |
| FC+D | Upsampled FC left/right views | Original full resolution left/right depths |
| MFC+D | MFC reconstructed left/right views | Original full resolution left/right depths |

Table 2 Test Sequences

| Seq. ID | Test sequence | Frames to be encoded | 2-view input | FC format |
| --- | --- | --- | --- | --- |
| S01 | Poznan_Hall2 | 200 | 7-6 | SbS |
| S02 | Poznan_Street | 250 | 5-4 | SbS |
| S03 | Undo_Dancer | 250 | 1-5 | TaB |
| S04 | GT_Fly | 250 | 9-5 | TaB |
| S05 | Kendo | 300 | 1-3 | SbS |
| S06 | Balloons | 300 | 1-3 | SbS |
| S08 | Newspaper1 | 300 | 2-4 | TaB |
| S10 | Shark | 300 | 1-5 | TaB |

## Simulation results

Table 3 shows the average PSNR gain of MFC+D compared with FC+D for the texture views as well as the synthesized views. The main goal of these tests is to provide an upper bound on the expected gain. The detailed data can be found in the attached excel sheet. It is interesting to note that the average Y-PSNR gain of synthesized views is in a similar range to that of left and right views, which is about 8dB. This gain is expected to reduce if view synthesis is performed on compressed data.

Spectrum analysis is also conducted to verify if the synthesized views can maintain the additional orthogonal high frequencies achieved by MFC in the videos. Figure 2 shows the results of the 51st frame of S01in the FC case. The upper image is given by the upsampled FC left view and the lower image shows the synthesized view at the position 0.5. Figure 3 shows the results in the MFC case. The upper image is the reconstructed MFC left view, whereas the lower image is the synthesized view at the position 0.5. The spectrum shows that the synthesized view retains most of the spectrum from the original video.

In addition, subjective testing has been performed to compare the visual quality of synthesized view between MFC+D and FC+D. The synthesized views of MFC+D appear much sharper, contain more high frequencies and feel more pleasant compared to those of FC+D.

An informal viewing session may be provided to show the difference between MFC+D and FC+D using a 4K auto-stereoscopic display.

Table 3 Average PSNR gain of MFC+D vs. FC+D

| average | PSNR Difference: MFC-FC |  |  |
| --- | --- | --- | --- |
|  | Y (dB) | U (dB) | V (dB) |
| video 0 | 8.1966 | 4.0120 | 4.7480 |
| video 1 | 8.2967 | 4.1403 | 4.6481 |
| syn 0.25 | 7.8414 | 3.1639 | 3.8529 |
| syn 0.50 | 8.0726 | 2.9808 | 3.4927 |
| syn 0.75 | 7.7604 | 3.0484 | 3.5042 |

Figure 2 Spectrum visualization of FC+D (upper: upsampled FC left view; lower: synthesized view at position 0.5)

Figure 3 Spectrum visualization of MFC+D ((upper: reconstructed MFC left view; lower: synthesized view at position 0.5)

# Conclusion

In this contribution, test results are provided to show the potential benefit of using MFC+D compared to FC+D. We propose to establish an AHG on further study of MFC+D in multi-view rendering and will provide results with compression at the next meeting.

# Acknowledgement

The authors would like to thank Dong Tian from MERL for crosschecking the simulation results [5].

# References

- M. M. Hannuksela, Y. Chen, T. Suzuki, S. Hattori, G. J. Sullivan, “Text for Rec. ITU-T H.264:2012 and ISO/IEC 14496-10:2012/ FDAM 2 MVC extension for inclusion of depth maps”, JCT3V-C1003, Geneva, CH, 17–23 Jan. 2013

- P. Yin, Y-K. Wang, G. J. Sullivan, “Draft 4 of Multi-resolution Frame Compatible Stereo Coding”, JCT3V-F1007, Geneva, CH, 25 Oct. – 1 Nov. 2013

- P. Yin, T. Lu, T. Chen, H. Ganapathy, G. Lakshminarayanan and W. Husak, “Description of MFC Video Coding Technology Proposal by Dolby”, Doc. m26661, 102th MPEG Meeting, Shanghai, China, Oct. 2012.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments”, JCT3V-F1100, Geneva, CH, 25 Oct. – 1 Nov. 2013

- D. Tian, “MFC+D: Crosscheck on Dolby Proposal JCT3V-G0115”, JCT3V-G0117, San José, US, 11–17 Jan. 2014

# Patent rights declaration(s)

Dolby Laboratories Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).