| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0110v2 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0116v2 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On default output layer sets |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kemal Ugur Miska M. Hannuksela | Tel: Email: | +90 536 567 2997 kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

It is asserted that the default output layer set mechanism in the current SHVC/MV-HEVC design is not suitable for various use-cases, such as ROI and view scalability. In addition, for common configurations of SHVC and MV-HEVC, it is asserted that the default output layer set mechanism does not bring any coding efficiency benefit. For these reasons, it is proposed to remove the default output layer set functionality from the SHVC/MV-HEVC design.

The zip package of version 2 contains the specification text including the proposed changes in the semantics as well as some editorial changes e.g. in syntax element naming.

# Introduction

The current SHVC/MV-HEVC design includes a default output layer sets mechanism, in which a default set of output layers is assigned for each layer set specified in the VPS. Additionally, zero or more non-default output layer sets can be specified in the VPS extension. Two ways of constructing default output layer sets are included: either all layers are output (used in multi-view extension) or only the highest layer is output (used in scalability extension).

Two problems are identified with the default output layer sets mechanism:

Problem 1: ROI Scalability

It is asserted that neither of the methods to construct a default output layer set is well suitable for ROI scalability. Consider the following use-case where there are 3 layers, the first one is a low resolution base layer, the second one is used for spatially enhancing the base layer and the third layer is the ROI enhancement layer.

In this example, the decoder shall output second and third layers, which enables the display process to compose the final picture for display. However, the current MV-HEVC/SHVC design requires the default output layer set to either contain all the three layers or only the highest layer. Outputting all three layers is not desirable, as layer 1 enhances layer 0 in its entirety and hence layer 0 should not be output. Outputting only the highest layer is not desirable as it only contains a region of layer 1. The current MV-HEVC/SHVC does not allow setting the default output layer set for this use case optimally, because it is mandated to have a default output layer set and none of the current methods for constructing default output layer sets is suitable to support this behaviour.

Problem 2: View scalability

Suppose that there is a three-view bitstream with the following prediction structure:

In this example suppose both V0-V1 and V1-V2 are valid stereo pairs corresponding to different display disparities and the player could choose which stereo pair should be displayed. Similar to ROI scalability, the decoder shall output second and third layers to compose the final stereo picture for the V1-V2 pair. The layer set containing the V1-V2 pair however also contains V0 due to the prediction structure. The current default output layer set mechanism requires using either V0-V1-V2 output or V2 output for this layer set. However, the content creator may consider both options as disallowed or undesirable. Thus, the current MV-HEVC/SHVC design does not allow setting the default output layer set for this use case optimally, because it is mandated to have a default output layer set and none of the current methods for constructing default output layer sets is suitable.

It is remarked that the assignment of output layers affects the DPB process and the bitstream conformance has to be ensured for each output layer set. Consequently, the current MV-HEVC/SHVC design that enables (default) output layer sets that may be undesirable by the encoder imposes requirements for the encoder to derive DPB-related parameter values and to verify bitstream conformance for output layer sets for these undesirable output layer sets.

# Proposal

As far as we have understood, the main reason of including default output layer set construction mechanism was to save bits in the common cases (i.e. coding efficiency). However, when the amount of bit savings are analyzed for common multi-view and scalable prediction structures, the amount of savings this mechanism brings are found to be very small. Therefore, we propose to remove the derivation of the default output layer sets and only allow explicitly specified output layer sets.

2-layer spatial scalability (only highest layer is output): same number of bits are used

This is derived as follows: default_one_target_output_layer_idc is not signaled but output_layer_flag for each layer needs to be additionally signaled (also no need to signal output_layer_set_idx_minus1 because there is a single layer-set)

3-view case (all layers are output): the proposal uses one bit more than the current MV-HEVC/SHVC design.

This is derived as follows: default_one_target_output_layer_idc is not signaled but output_layer_flag for each layer is signaled (also no need to signal output_layer_set_idx_minus1 because there is a single layer-set.

2-view case (all layers are output): same number of bits are used

The only case where deriving the default output layer sets is beneficial could be coding an N-view case (e.g 10 views), where the proposal would use N more bits per layer set (while reduce 2 bits per VPS due to the removal of default_one_target_output_layer_idc).

# Proposed text

| vps_number_layer_sets_minus1 | u(10) |
| --- | --- |
| vps_num_profile_tier_level_minus1 | u(6) |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| NumOutputLayerSets = vps_number_layer_sets_minus1 + 1 |  |
| more_output_layer_sets_than_default_flag | u(1) |
| if( more_output_layer_sets_than_default_flag ) { |  |
| num_add_output_layer_sets_minus1 | u(10) |
| numOutputLayerSets += num_add_output_layer_sets_minus1 + 1 |  |
| } |  |
| if( numOutputLayerSets > 1 ) |  |
| default_one_target_output_layer_idc | u(2) |
| for( i = 1; i < numOutputLayerSets; i++ ) { |  |
| if( i > vps_number_layer_sets_minus1 ) { |  |
| output_layer_set_idx_minus1[ i ] | u(v) |
| lsIdx = output_layer_set_idx_minus1[ i ] + 1 |  |
| } |  |
| for( j = 0 ; j < NumLayersInIdList[ lsIdx ] − 1; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| profile_level_tier_idx[ i ] | u(v) |
| } |  |
| if( vps_max_layers_minus1 > 0 ) |  |
| alt_output_layer_flag | u(1) |
| rep_format_idx_present_flag | u(1) |
| if( rep_format_idx_present_flag ) |  |
| vps_num_rep_formats_minus1 | u(8) |

The proposed changes in the semantics as well as some editorial changes are presented with change marks in the accompanying specification text document.

# Patent rights declaration(s)

The authors are not aware of any current or pending patent rights relating to the technology described in this contribution.