| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0118 |
| --- | --- |

| Title: | CE1: De-blocking friendly sub-PU processing in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu Ying Chen Geert Van Der Auwera 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 86-10-57760696 hongbinl@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

Sub-PU level motion prediction may lead to blocking artifact because sub-PUs (prediction unit) within a PU are allowed to be predicted with different motion vectors or even from different pictures while the boundary of such sub-PUs are not considered as edges in the de-blocking process. Since, de-blocking as a whole module is typically not modified in codec extensions. In order to use the HEVC de-blocking module but to reduce the blocking artifact, this contribution proposes to modify transform tree structure for each coding unit (CU) that contains sub-PU, right before the de-blocking process. The basic idea is to make sub-PU boundary be transform unit boundary. From the side-by-side comparison, it reportedly shows that blocking artifact caused by sub-PU level motion prediction is suppressed. It is also reported that -0.29% and -0.18% bit rate reduction are achieved on texture and synthesized view respectively. It is reported that this proposal doesn’t require any change of the de-blocking process in HEVC.

# Introduction

Sub-PU level motion prediction, such as sub-PU level inter-view motion prediction and backward view synthesis prediction in 3D-HEVC [1], allows sub-PUs within a PU to be predicted using different motion vectors or disparity vectors. However, sub-PU boundaries are not considered as edges in the de-blocking process. Therefore, block artifacts may occur at sub-PU boundaries. Meanwhile, in backward view synthesis prediction (BVSP), though different sub-PUs may use different disparity vectors in motion compensation process, one same disparity vector is stored for all sub-PUs within the PU. This prevents true disparity vector of view synthesis predicted sub-PU to be used in motion vector prediction process, merge list construction process or neighboring block disparity vector derivation process, thus may lead to bit rate increase.

An example is illustrated in Figure 1 to show blocking artifact caused by sub-PU level motion prediction. Sub-PU level inter-view motion prediction is included In HTM-9.0r1 [2] but not included in HTM-8.0 [3]. As can be seen in Figure 1, much worse blocking artifact can be observed on reconstructed picture generated by HTM-9.0r1.

Figure 1. Blocking artifact caused by sub-PU level motion prediction (58th picture in first non-base view of Balloons sequence, qp = 35). Left: HTM-8.0, Right: HTM-9.0r1.

## De-blocking filters in codec extensions

In MVC and MV-HEVC, no block level changes are introduced therefore de-blocking modules of these two codecs are the same as those of the base specifications, i.e., AVC and HEVC.

In 3D-AVC, there were techniques modifying the de-blocking filters for specially predicted regions. However, such techniques are considered as changing the fundamental design, therefore were eventually excluded from 3D-AVC.

SHVC won’t have the chance to change the de-blocking filter, although SVC slightly changed the de-blocking filter for some reasons.

So, in 3D-HEVC, it is expected that de-blocking filter shall not be changed.

## Motivation of de-blocking filters

De-blocking filter serves the purpose of reducing blocking artifacts introduced due to the reason that different blocks are predicted (motion compensated) from different blocks of the same picture or different blocks from different pictures.

## Key idea of sub-PU processing

In both VSP or sub-PU based inter-view motion prediction, the key idea is to derive for a single PU more accurate motion information for blocks smaller than such a PU.

Although both techniques benefit from the prediction of motion vectors, another advantage is one PU can be predicted and signalled once without being further split into multiple blocks, which requires significant signaling overhead.

So practically, sub-PU processing always benefits from independently motion compensated blocks, which lead to blocking artifacts.

With the current 3D-HEVC design, the de-blocking process considers the whole PU as one block and is not able to filter the block boundaries.

# Proposed method

## Motivation

To take advantage of the de-blocking scheme in HEVC, a PU coded with sub-PU processing should be considered as containing multiple blocks.

So a straightforward way of applying that is to create virtual block boundaries within a PU when it is coded with sub-PU processing.

In HEVC de-blocking, block boundaries are identified either by PU/CU boundary or TU (transform unit) boundary. Creating virtual boundaries for such a PU can be achieved by either making the current PU a CU and introducing coding tree structures within a PU or making the current PU containing TU boundaries. The later seems to be simpler.

## Proposal

To address the above issue while not changing the de-blocking process as in HEVC version 1, it is proposed to convert each sub-PU to be a TU before de-blocking process. Therefore, sub-PU boundary is converted to TU boundary and is considered as edge in the following de-blocking process.

Note in HEVC version 1 de-blocking module, the edges are identified when either CU/PU boundaries or TU boundaries exist.

Sub-PUs in SPIVMP or BVSP (In BVSP, 8x8 sub-PUs are converted to TUs though disparity vector is derived at smaller block size, e.g., 8x4 or 4x8 block) are converted to TUs as follows:

- For each sub-PU, when the TU covering the sub-PU is larger than the size of the sub-PU, the sub-PU is associated with a TU of the same size.

Figure 2. Conversion of sub-PU to TU.

Note that the above process happens after the whole picture is decoded but before it is de-blocked.

illustrates an example of associating TUs to sub-PUs. In , CU size is 32x32 and its partition mode is Nx2N, and PU0 does not contain sub-PU while PU1 contains sub-PU. The sub-PU size is 8x8. As can be seen, if a TU is larger than 8x8, sub-PUs within it are converted to 8x8 TUs.

Proposed method is only applied to CU with partition size 2Nx2N, 2NxN, Nx2N and NxN, and asymmetric partition is not processed.

Furthermore, to allow true disparity vector of view synthesis predicted sub-PU to be used in motion vector prediction process, merge list construction process or neighboring block disparity vector derivation process etc., motion parameter of BVSP predicted PU are stored at sub-PU level instead of PU level.

# Simulation results

Proposed method is integrated into HTM-9.0r1 and compared with it under common test condition [4]. Sub-PU size is set to be equal to 8x8 in the simulation.

Side-by-side subjective comparison is shown for several clips of Balloons and Kendo in Figure 3 and Figure 4.

Figure 3. Subjective comparison of Balloons on first non-base view with qp = 35 (58th picture). Left: proposed method, Right: HTM-9.0r1.

Figure 4. Subjective comparison of Kendo on first non-base view with qp = 35 (11th picture). Left: proposed method, Right: HTM-9.0r1.

Objective comparison is shown in Table 1. As can be seen, proposed method can achieve -0.29% and -0.18% average coding gain on texture and synthesized view respectively.

Table 1 Comparison of proposed method with HTM-9.0r1 (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.77% | -0.52% | -0.35% | -0.36% | -0.24% | 105.5% | 112.7% |
| Kendo | 0.00% | -0.77% | -0.94% | -0.44% | -0.43% | -0.31% | 95.0% | 100.7% |
| Newspaper_CC | 0.00% | -0.44% | -0.23% | -0.18% | -0.20% | -0.10% | 96.3% | 100.5% |
| GT_Fly | 0.00% | -0.43% | -0.47% | -0.19% | -0.21% | -0.11% | 102.4% | 101.5% |
| Poznan_Hall2 | 0.00% | -0.72% | -0.70% | -0.37% | -0.39% | -0.24% | 97.5% | 89.7% |
| Poznan_Street | 0.00% | -0.58% | -0.39% | -0.22% | -0.21% | -0.14% | 96.8% | 98.3% |
| Undo_Dancer | 0.00% | -0.76% | -0.53% | -0.28% | -0.29% | -0.19% | 103.0% | 98.0% |
| Shark | 0.00% | -0.56% | -0.47% | -0.25% | -0.26% | -0.11% | 118.0% | 102.1% |
| 1024x768 | 0.00% | -0.66% | -0.56% | -0.33% | -0.33% | -0.22% | 98.9% | 104.6% |
| 1920x1088 | 0.00% | -0.61% | -0.51% | -0.26% | -0.27% | -0.16% | 103.6% | 97.9% |
| average | 0.00% | -0.63% | -0.53% | -0.29% | -0.29% | -0.18% | 101.8% | 100.4% |

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 2,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-F1001, 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013.

- 3DV-HTM version 9.0r1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-9.0r1/

- 3DV-HTM version 8.0: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-8.0/

- D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-F1100, 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).