| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0119 |
| --- | --- |

| Title: | CE2: Sub-PU based MPI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Chen Hongbin Liu Li Zhang 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-845-6589 cheny@qti.qualcomm.com +86-10-57760696 hongbinl@qti.qualcomm.com +1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to apply sub-PU level Motion Parameter Inheritance (MPI) merge candidate in 3D-HEVC for depth coding. For each depth PU, smaller size (8x8) sub-PUs can grab different motion vectors from the co-located 8x8 blocks in texture to form a MPI candidate. Experimental results show that the proposed method achieves additional 0.2% BD-rate saving. The proposed technique is the same as in JCT3V-F0128.

# Introduction

In last meeting, a sub-PU level inter-view motion prediction method was adopted in 3D-HEVC [1], which achieves coding gain by doing motion prediction with smaller block size instead of the full PU size.

With the sub-PU level inter-view motion prediction, the inter-view merge candidate in the current 3D-HEVC [2] is derived as follows.

- The current PU is divided into multiple sub-PUs of a smaller size, e.g., 8x8, as specified in VPS.

- For each sub-PU within current PU, the followings are applied

  - A disparity vector is added to the middle position of current sub-PU to reach a 4x4 reference block in the reference view.

  - If the 4x4 reference block has available motion parameters, the associated motion parameters are used for current sub-PU.

The sub-PU level inter-view motion prediction is proposed only for texture coding, however, it is noted that the concept of sub-PU motion prediction is also applicable for depth coding, i.e, MPI in depth coding.

In current 3D-HEVC depth coding, MPI candidate is derived from the co-located texture block. An example of the motion parameter derivation in the MPI candidate is shown in Figure 1. Derivation of the MPI candidate for depth coding., where the corresponding texture block is selected as the 4x4 block located to the right bottom of the center of the current PU.

Figure 1. Derivation of the MPI candidate for depth coding.

# Proposal

An MPI merge candidate based on sub-PU level motion prediction, namely sub-PU MPI is proposed. With the proposed sub-PU level MPI merge candidate, for each 8x8 block within current depth PU, the motion parameters of the co-located texture block which covers the middle position of current 8x8 block is inherited (with a right shift by 2 of the motion vectors) for current sub-PU.

With the proposed sub-PU MPI, different motion vectors can be inherited for 8x8 sub-PUs within one depth PU.

To support flexible sub-PU size to be used for sub-PU MPI, one syntax element representing sub-PU size is added into video parameter set extension 2 of 3D-HEVC. This syntax element is only coded once (instead of coded for each depth layer) and used for all depth layers.

It is also noted that always accessing the co-located texture block is less complex compared to dynamically accessing a block in a reference view depending on a disparity vector, as required by sub-PU level inter-view motion prediction.

# Experimental results

The proposed method is integrated into current 3D-HTM 9.0r1 software [3] and simulations are performed based on the common test conditions (CTC) [4]. Anchor: 3D-HTM 9.0r1.

In the simulations, the sub-PU MPI size is set equal to 8x8, while it could be made flexible as in sub-PU inter-view motion prediction.

The coding performance of the above test is provided in Table 1. As reported in Table 1, the overall average bitrate reduction is 0.2% (0.23%) for the synthesized views.

The encoding, decoding and rendering estimates of the proposed scheme are calculated on the Linux cluster; these estimates may not be accurate due to parallel simulations.

It is also worth mentioning that when sub-PU motion prediction is enabled for texture, sub-PU MPI can be more efficiently coded, since the texture PUs tend to have more chances to be more variations of motion vectors with PUs.

Table 1: Coding results of sub-PU MPI for 3-view case

|  | Video 1 | Video 2 | Video PSNR / video bitrate |  | Video PSNR / total bitrate |  | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.06% | -0.05% | -0.02% | -0.23% |  |  | -0.29% | 94.0% | 103.3% | 94.2% |
| Kendo | -0.11% | -0.04% | -0.05% | -0.44% |  |  | -0.42% | 98.5% | 96.7% | 97.5% |
| Newspapercc | -0.03% | 0.08% | 0.01% | -0.30% |  |  | -0.29% | 93.2% | 117.0% | 93.6% |
| GhostTownFly | 0.31% | 0.30% | 0.07% | -0.09% |  |  | -0.16% | 111.0% | 98.7% | 105.9% |
| PoznanHall2 | 0.27% | 0.09% | 0.08% | -0.09% |  |  | -0.01% | 97.8% | 96.0% | 95.9% |
| PoznanStreet | -0.03% | -0.07% | -0.01% | -0.13% |  |  | -0.14% | 93.9% | 92.2% | 94.6% |
| UndoDancer | -0.04% | 0.06% | 0.01% | -0.27% |  |  | -0.32% | 93.8% | 107.0% | 95.3% |
| Shark | 0.16% | 0.14% | 0.03% | -0.36% |  |  | -0.24% | 118.7% | 115.7% | 114.4% |
| 1024x768 | -0.07% | 0.00% | -0.02% | -0.32% |  |  | -0.33% | 95.2% | 105.7% | 95.1% |
| 1920x1088 | 0.13% | 0.10% | 0.04% | -0.19% |  |  | -0.17% | 103.0% | 101.9% | 101.2% |
| average | 0.06% | 0.06% | 0.01% |  |  | -0.24% | -0.23% | 100.1% | 103.3% | 98.9% |

# References

- J. An, K. Zhang, J.-L. Lin, S. Lei, “3D-CE3.h related: Sub-PU level inter-view motion prediction,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-F0110, 6th Meeting: 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013.

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 2,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-F1001, 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013.

- 3DV-HTM version 9.0r1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-9.0r1/

- D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-F1100, 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).