| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0119_text |
| --- | --- |

| Title: | Proposed text for JCT3V-G0119 based on 3D-HEVC Draft Text 2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Chen Hongbin Liu Li Zhang 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-845-6589 cheny@qti.qualcomm.com +86-10-57760696 hongbinl@qti.qualcomm.com +1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

All the changes are highlighted in yellow or red strikethrough.

# Syntax table

- H.7.3.2.1.2 Video parameter set extension 2 syntax

| … |  |
| --- | --- |
| log2_sub_pb_mpi_size_minus3 | ue(v) |
| cp_precision | ue(v) |
| for( i = 0; i < NumViews; i++ ) { |  |
| cp_present_flag[ i ] | u(1) |
| if( cp_present_flag[ i ] ) { |  |
| cp_in_slice_segment_header_flag[ i ] | u(1) |
| if( !cp_in_slice_segment_header_flag[ i ] ) |  |
| for( j = 0; j < i; j++ ) { |  |
| vps_cp_scale[ i ][ j ] | se(v) |
| vps_cp_off[ i ][ j ] | se(v) |
| vps_cp_inv_scale_plus_scale[ i ][ j ] | se(v) |
| vps_cp_inv_off_plus_off[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| iv_mv_scaling_flag | u(1) |
| } |  |

# Decoding process

- H.7.4.3.1.2 Video parameter set extension 2 semantics

...

log2_sub_pb_mpi_size_minus3 specifies the value of the variable SubPbSizeMPI that is used in the decoding of prediction units using the sub prediction block based texture merging candidate. The value of log2_sub_pb_mpi_size_minus3 shall be in the range of 0 to 3, inclusive.

The variable SubPbSizeMPI is derived as specified in the following:

  - SubPbSizeMPI = 1 <<  ( log2_sub_pb_mpi_size_minus3 + 3 )	(Error: Reference source not found-1)

cp_precision specifies the precision of vps_cp_scale[ i ][ j ], vps_cp_off[ i ][ j ], vps_cp_inv_scale_plus_scale[ i ][ j ], and vps_cp_inv_off_plus_off[ i ][ j ] in the VPS and cp_scale[ j ], cp_off[ j ], cp_inv_scale_plus_scale[ j ], and cp_inv_off_plus_off[ j ] in the slice segment header. The value of cp_precision shall be in the range of 0 to 5, inclusive.

…

- H.8.5.3.2.1 Derivation process for luma motion vectors for merge mode

…

- 12. The variable  subPbMotionFlag is set equal to ( ( N  = =  IvMC ) || ( N = = T ) ).

…

- H.8.5.3.2.14 Derivation process for the sub prediction block based texture merging candidate

This process is not invoked when mpi_flag[ nuh_layer_id ] is equal to 0.

Inputs to this process are:

- a luma location ( xPb, yPb ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables nPbW and nPbH specifying the width and the height, respectively, of the current prediction unit,

Outputs of this process are:

- a flag availableFlagT specifying whether the texture merging candidate is available,

- the prediction utilization flags predFlagL0T and predFlagL1T,

- the reference indices refIdxL0T and refIdxL1T (when availableFlagT is equal to 1),

- the motion vectors mvL0T and mvL1T (when  availableFlagT is equal to 1).

The variable availableFlagT is set equal to 0. The variables predFlagL0T and predFlagL1T are set equal to 0. The variables refIdxL0T and refIdxL1T are set equal to −1. Both components of the motion vectors mvL0T and mvL1T are set equal to 0.

The variables nSbW and nSbH are derived as:

  - nSbW = Min( nPbW, SubPbSizeMPI )		(Error: Reference source not found-2)

  - nSbH  = Min( nPbH,  SubPbSizeMPI )		(Error: Reference source not found-3)

The texture luma location ( xRef, yRef ) is derived by:

  - xRefFull = xPb + ( ( nPbW – 1 )  >>  1 )		(Error: Reference source not found-4)

  - yRefFull = yPb + ( ( nPbH – 1 )  >>  1 ) 		(Error: Reference source not found-5)

  - xRef = Clip3( 0, PicWidthInSamplesL – 1, ( xRefFull  >>  3 )  <<  3 )	(Error: Reference source not found-6)

  - yRef = Clip3( 0, PicHeightInSamplesL – 1,( yRefFull  >>  3 )  <<  3 )	(Error: Reference source not found-7)

  - [ Ed. (GT): Is clipping necessary? ]

Let textPic be the picture with PicOrderCntVal and ViewIdx equal to PicOrderCnt and ViewIdx of the current picture and DepthFlag being equal to 0 and let textPU the PU at covering the position ( xRef, yRef ) in textPic. The variable curSubBlockIdx is set equal to 0 and the variable lastAvailableFlag is set equal to 0.

For X in the range of 0 to 1, inclusive, the following applies:

  - The variable textPredFlagLX is set equal to PredFlagLX of textPic. The variable textRefIdxLX is set equal to RefIdxLX of textPic. The variable textMvLX is set equal to the MvLX of textPic. The variable availableFlag is set equal to 0.

  - When X is equal to 0 or the current slice is a B slice, for i in the range of 0 to NumRefPicsLX − 1, inclusive, the following applies:

    - When all of the following conditions are true, availableFlag is set equal to 1,

      - textPredFlagLX[ xRef ][ yRef ] is equal to 1

      - PicOrderCnt( RefPicListX[ i ] ) is equal to PicOrderCnt( textPic )

      - ViewIdx( RefPicListX[ i ] ) is equal to ViewIdx( textPic )

    - When predFlagLXT is equal to 0 and availableFlag is equal to 1, the following applies:

        - mvLXT[ 0 ] = ( textMvLX[ xRef ][ yRef ][ 0 ] + 2 )  >>  2	(Error: Reference source not found-8)

        - mvLXT[ 1 ] = ( textMvLX[ xRef ][ yRef ][ 1 ] + 2 )  >>  2	(Error: Reference source not found-9)

        - refIdxLX = i		(Error: Reference source not found-10)

        - predFlagLXT = 1		(Error: Reference source not found-11)

        - availableFlagT = 1		(Error: Reference source not found-12)

For yBlk in the range of 0 to ( nPbH / nSbH – 1 ), inclusive, and for xBlk in the range of 0 to ( nPbW / nSbW – 1 ), inclusive, the following applies:

  - The variable curAvailableFlag is set equal to 0.

  - For X in the range of 0 to 1, inclusive, the following applies:

      - The flag spPredFlagL1[ xBlk ][ yBlk ] is set equal to 0.

      - The motion vector spMvLX is set equal to ( 0, 0 ).

      - The reference index spRefIdxLX[ xBlk ][ yBlk ]is set equal to –1.

  - The luma location ( xText, yText ) is derived by

      - xText = xPb + xBlk * nSbW + nSbW / 2		(Error: Reference source not found-13)

      - yText = yPb + yBlk * nSbH + nSbH / 2		(Error: Reference source not found-14)

  - The variable textPb specifies the luma sub prediction block covering the location given by ( xText, yText ) inside textPic.

  - The luma location ( xTextPb, yTextPb ) is set equal to the top-left sample of textPb relative to the top-left luma sample of textPic.

  - When textPb is not coded with intra prediction mode, the following applies for X in the range of 0 to 1, inclusive:

    - When X is equal 0 or the current slice is a B slice, the following applies for Y in the range of X to (1 – X), inclusive:

      - The variables refPicListLYText, predFlagLYText[ x ][ y ], mvLYText, and refIdxLYText are set equal to RefPicListLY, PredFlagLY, MvLY, and RefIdxLY of the picture textPic, respectively.

      - When predFlagLYText[ xTextPb ][ yTextPb ] is equal to 1, the following applies for each i from 0 to num_ref_idx_lX_active_minus1, inclusive:

        - When PicOrderCnt( refPicListLYText[ refIdxLYText[ xTextPb ][ yTextPb ] ] ) is equal to PicOrderCnt( RefPicListLX[ i ] ) and spPredFlagLX[ xBlk ][ yBlk ] is equal to 0, the following applies.

            - spMvLX[ xBlk ][ yBlk ] = ( mvLYText[ xTextPb ][ yTextPb ] + 2 ) >> 2	(Error: Reference source not found-15)

            - spRefIdxLX[ xBlk ][ yBlk ] = i		(Error: Reference source not found-16)

            - spPredFlagLX[ xBlk][ yBlk ] = 1		(Error: Reference source not found-17)

            - curAvailableFlag = 1		(Error: Reference source not found-18)

  - Depending on curAvailableFlag, the following applies:

    - If curAvailableFlag is equal to 1, the following ordered steps apply:

      - When lastAvailableFlag is equal to 0, the following applies:

        - For X in the range of 0 to 1, inclusive, the following applies:

            - mvLXT            = spMvLX[ xBlk ][ yBlk ]	(Error: Reference source not found-19)

            - refIdxLXT       = spRefIdxLX[ xBlk ][ yBlk ]	(Error: Reference source not found-20)

            - availableFlagT = spPredFlag[ xBlk ][ yBlk ]	(Error: Reference source not found-21)

            - predFlagLXT   = spPredFlag[ xBlk ][ yBlk ]	(Error: Reference source not found-224)

        - When curSubBlockIdx is greater than 0, the following applies for k in the range of 0 to ( curSubBlockIdx − 1 ), inclusive:

          - The variables i and k are derived as specified in the following:

              - i = k % ( nPSW / nSbW )		(Error: Reference source not found-235)

              - j = k / ( nPSW / nSbW )		(Error: Reference source not found-246)

          - For X in the range of 0 to 1, inclusive, the following applies:

              - spMvLX[ i ][ j ] = spMvLX[ xBlk ][ yBlk ]	(Error: Reference source not found-257)

              - spRefIdxLX[ i ][ j ] = spRefIdxLX[ xBlk ][ yBlk ]	(Error: Reference source not found-268)

              - spPredFlagLX[ i ][ j ] = spPredFlagLX[ xBlk ][ yBlk ]	(Error: Reference source not found-279)

      - The variable lastAvailableFlag is set equal to 1.

      - The variables xLastAvail and yLastAvail are set equal to xBlk and yBlk, respectively.

    - Otherwise (curAvailableFlag is equal to 0), when lastAvailable Flag is equal to 1, the following applies for X in the range of 0 to 1, inclusive:

        - spMvLX[ xBlk ][ yBlk ] = spMvLX[ xLastAvail ][ yLastAvail ]	(Error: Reference source not found-28)

        - spRefIdxLX[ xBlk ][ yBlk ] = spRefIdxLX[ xLastAvail ][ yLastAvail ]	(Error: Reference source not found-291)

        - spPredFlagLX[ xBlk ][ yBlk ] = spPredFlagLX[ xLastAvail ][ yLastAvail ]	(Error: Reference source not found-302)

  - The variable curSubBlockIdx is set equal to curSubBlockIdx + 1.

For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = 0.. nPbW − 1 and y = 0.. nPbH − 1:

  - For X in the range of 0 to 1, inclusive, the following applies:

    - The variables SubPbPredFlagLX, SubPbMvLX and SubPbRefIdxLX are derived as specified in following:

        - SubPbPredFlagLX[ xPb + x ][ yPb + y ] = spPredFlagLX[ x / nSbW ][ y / nSbW ]	(Error: Reference source not found-313)

        - SubPbMvLX[ xPb + x ][ yPb + y ] = spMvLX[ x / nSbW ][ y / nSbW ]	(Error: Reference source not found-324)

        - SubPbRefIdxLX[ xPb + x ][ yPb + y ] = spRefIdxLX[ x / nSbW ][ y / nSbW ]	(Error: Reference source not found-335)

    - The derivation process for chroma motion vectors in subclause 8.5.3.2.9 is invoked with SubPbMvLX[ xPb + x ][ yPb + y ] as input and the output is SubPbMvCLX[ xPb + x ][ yPb + y ].

- H.8.5.3.3.9 Decoding process for sub prediction block wise inter sample prediction

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xBl, yBl ) specifying the top-left sample of the current luma prediction block relative to the top-left sample of the current luma coding block,

- a variable nCbS specifying the size of the current luma coding block,

- two variables nPbW and nPbH specifying the width and the height of the luma prediction block,

Outputs of this process are:

- an (nCbSL)x(nCbSL) array predSamplesL of luma prediction samples, where nCbSL is derived as specified below,

- an (nCbSC)x(nCbSC) array predSamplesCb of chroma prediction samples for the component Cb, where nCbSC is derived as specified below,

- an (nCbSC)x(nCbSC) array predSamplesCr of chroma residual samples for the component Cr, where nCbSC is derived as specified below.

- if DepthFlag is equal to 0, the variable nSbW is set equal to Min( nPSW, SubPbSize[ nuh_layer_id ] ) and the variable nSbH is set equal to Min( nPSH, SubPbSize[ nuh_layer_id ] ).

- Otherwise, (DepthFlag is equal to 1), the variable nSbW is set equal to Min( nPSW, SubPbSizeMPI ) and the variable nSbH is set equal to Min( nPSH, SubPbSizeMPI ).

For x in the range of 0 to ( nPsW / nSbW − 1 ), inclusive, the following applies:

- For y in the range of 0 to ( nPsH / nSbH − 1 ), inclusive, the following applies:

  - The luma location ( xSb, ySb ) specifying the top-left sample of the current luma sub prediction block relative to the top-left sample of the current luma coding block is derived as specified in the following:

      - xSb = xBl + x * nSbW		(Error: Reference source not found-34)

      - ySb = yBl + y * nSbH		(Error: Reference source not found-35)

  - For X in the range of 0 to 1, inclusive, the variables mvLX, mvCLX, refIdxLX, and predFlagLX are derived as specified in the following:

      - mvLX = SubPbMvLX[ xSb ][ ySb ]		(Error: Reference source not found-36)

      - mvCLX = SubPbMvCLX[ xSb ][ ySb ]		(Error: Reference source not found-37)

      - refIdxLX = SubPbRefIdxLX[ xSb ][ ySb ]		(Error: Reference source not found-38)

      - predFlagLX = SubPbPredFlagLX[ xSb ][ ySb ]		(Error: Reference source not found-39)

  - if DepthFlag is equal to 0, the decoding process for inter sample prediction as specified in subclause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the luma prediction block location ( xBl, yBl ) equal to ( xSb, ySb ), the luma coding block size block nCbS, the luma prediction block width nPbW equal to nSbW, the luma prediction block height nPbH equal to nSbH, the luma motion vectors mvL0 and mvL1, the chroma motion vectors mvCL0 and mvCL1, the reference indices refIdxL0 and refIdxL1, and the prediction list utilization flags predFlagL0 and predFlagL1 as inputs, and the inter prediction samples (predSamples) that are an (nCbSL)x(nCbSL) array predSamplesL of prediction luma samples and two (nCbSC)x(nCbSC) arrays predSamplesCr and predSamplesCr of prediction chroma samples, one for each of the chroma components Cb and Cr, as outputs.

  - Otherwise, (DepthFlag is equal to 1), following applies:

    - The decoding process for inter sample prediction as specified in subclause Error: Reference source not found5 is invoked with the luma coding block location ( xCb, yCb ), the luma prediction block location ( xSb, ySb ), the luma prediction block width nSbW, the luma prediction block height nSbH, the luma motion vector mvLX, the crhoma motion vector mvCLX, and the reference picture sample arrays refPicLXL, refPicLXCb and refPicLXCr as input, and the outputs are the prediction sample arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr.

    - The luma prediction samples array predSampleL is derived by invoking the weighted sample prediction process specified in subclause 8.5.3.3.4 with the luma prediction block width nSbW, the luma prediction block height nSbH, and the sample arrays predSamplesL0L and predSamplesL1L, and the variables predFlagL0, predFlagL1, refIdxL0, refIdxL1, and cIdx equal to 0 as inputs.

    - The chroma prediction samples array predSampleCb is derived by invoking the weighted sample prediction process specified in subclause 8.5.3.3.4 with the chroma prediction block width nSbW / 2, the chroma prediction block height nSbH / 2, and the sample arrays predSamplesL0Cb and predSamplesL1Cb, and the variables predFlagL0, predFlagL1, refIdxL0, refIdxL1, and cIdx equal to 1 as inputs.

    - The chroma prediction samples array predSampleCr is derived by invoking the weighted sample prediction process specified in subclause 8.5.3.3.4 with the chroma prediction block width nSbW / 2, the chroma prediction block height nSbH / 2, and the sample arrays predSamplesL0Cr and predSamplesL1Cr, and the variables predFlagL0, predFlagL1, refIdxL0, refIdxL1, and cIdx equal to 2 as inputs.