| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0120 |
| --- | --- |

| Title: | CE1: Simplifications to sub-PU level inter-view motion prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu Ying Chen Li Zhang 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 86-10-57760696 hongbinl@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com 1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In 3D-HEVC, sub-PU level inter-view motion prediction (SPIVMP) is not friendly to parallel processing, and motion vectors of a sub-PU may not be derived till all sub-PUs have been processed. This contribution proposes a simplification for SPIVMP to make it friendly to parallel processing. One default set of motion parameters (including up to two motion vectors) from a given block in the reference view is directly used to generate motion information for sub-PUs without available motion, as well as pruning the SPIVMP candidate with other candidates (spatial merge candidate A1 and B1). With the proposed method, motion prediction for each sub-PU can be done in parallel and pruning of other candidates never needs to wait till all sub-PUs are motion predicted. It is reported that there is only minor coding loss (0.08%) on synthesized view.

# Introduction

In the 6th JCT-3V meeting, sub-PU level inter-view motion prediction (SPIVMP) [1] is adopted, in which each sub-PU may have its own motion parameters derived from a reference block in a reference view.

However, SPIVMP is not friendly to parallel processing for motion prediction of the sub-PUs. All sub-PUs within a PU have to be processed serially to generate sub-PU level motion parameters for each sub-PU or to perform pruning with other merge candidates.

## Motion hole-filling process

In SPIVMP, one Dynamic set of Motion Parameters (DMP, including up to two motion vectors and the associated reference indices) is maintained and updated for the current PU.

The DMP is changing constantly while the sub-PUs are processed. It is assumed that the sub-PUs are processed in raster scan order. The DMP is always updated if the sub-PU’s corresponding block contains available motion information: DMP is set to be the motion information of the corresponding block.

An example of updating the DMP is shown in Figure 1, in which the first sub-PU, the seventh sub-PU and the twelfth sub-PU have available motion parameters (denoted as MP1, MP2 and MP3 respectively) in their reference blocks, in raster scan order; while the remaining sub-PUs do not have available motion parameters in their reference blocks. The DMP is set to MP1 at the first sub-PU, and is changed to MP2 when the seventh sub-PU is reached, and is further changed to MP3 when the twelfth sub-PU is reached.

Figure 1: Update of the DMP in sub-PU level inter-view motion prediction.

The DMP is used to fill the motion information for sub-PUs which do not have available motion information in their corresponding blocks. When such a sub-PU is identified, its motion information is set to be DMP. This process is called motion hole-filling in this document.

### Problems of the motion hole-filling

There is dependency on the motion prediction process among sub-PUs within one PU, since a sub-PU which is a hole can only be predicted after all previous sub-PUs are predicted. In the worst case scenario, all sub-PUs cannot be motion predicted until the last sub-PU of the current PU accesses its corresponding block’s motion information.

Note that in HEVC version 1, motion prediction of the PUs coded in merge mode within a CU is made independent to other PUs of the same CU. This way, motion estimation process can be accelerated without waiting for one PU to be motion compensated to start the motion compensation of the other PU.

The current design however, makes the encoder side complexity reduction as enabled in the HEVC merge more difficult to achieve when the SPIVMP candidate is involved.

## Merge candidate pruning

In SPIVMP, one Representative set of Motion Parameters (RMP) needs to be generated for the purpose of merge candidate pruning with spatial merge candidate A1 and B1. The RMP is set to be the motion parameters of the first sub-PU that contains available motion information, in raster scan order. Note that RMP is the first occurrence of the DMP.

### Problems of the merge candidate pruning

In the worst case, all sub-PUs have to be motion predicted before the SPIVMP can be pruned with other candidates. The merge candidate list generation process depends on scanning of all reference blocks to include or exclude other merge candidates. This introduces unnecessary delay.

# Proposal

To make sub-PU level inter-view motion prediction friendly to parallel processing, one default set of motion parameters is generated before processing any sub-PU. For each sub-PU within the current PU, if its corresponding reference block does not contain any available motion information, the default set of motion parameters is used for the sub-PU.

The default set of motion parameters is used for motion hole-filing and pruning by setting both the DMP and the RMP to be the default set of motion parameters. In this way, motion prediction for each sub-PU can be done in parallel and pruning of other candidates never needs to wait till all sub-PUs are motion predicted.

The default set of motion parameters is generated as follows:

- If the block covering the pixel ((nPSW/nSubPsW/2)*nSubPsW + nSubPsW/2, (nPSH/nSubPsH/2)*nSubPsH + nSubPsH/2) within the reference block (of the PU) has available motion parameter, the motion parameters of the block is used as the default set of motion parameters. Here, nPSW and nPSH are the width and height of the PU, and nSubPsW and nSubPsH are the width and height of the sub-PU.

    - Note that, in case that the sub-PU size is same with the PU size, motion parameter (if available) of the block that covers the pixel (nPSW/2, nPSH/2) within the reference block (of the PU) is used as the default set of motion parameters.

- Otherwise, zero motion vector and zero reference index for both reference picture list 0 and reference picture list 1 (if current slice is B slice) is used as the default set of motion parameters.

# Experimental results

The proposed method is integrated into HTM-9.0r1 [2] and simulations are performed based on the common test conditions (CTC) [3].

Two tests are simulated in this section.

- Test#1: Both DMP and RMP are set to the default set of motion parameters.

- Test#2: Only DMP is set to the default set of motion parameters and the pruning is not changed.

The coding performance for Test#1 is shown in Table 1. As reported in Table 1, there is only 0.08% coding loss for the synthesized views.

Table 1: Comparison of Test#1 with HTM-9.0r1 (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.26% | 0.30% | 0.12% | 0.12% | 0.08% | 104.0% | 102.3% |
| Kendo | 0.00% | 0.38% | 0.28% | 0.12% | 0.13% | 0.10% | 94.1% | 94.8% |
| Newspaper_CC | 0.00% | 0.15% | 0.18% | 0.06% | 0.04% | 0.07% | 94.3% | 92.9% |
| GT_Fly | 0.00% | -0.07% | -0.20% | -0.03% | 0.00% | 0.01% | 90.1% | 92.5% |
| Poznan_Hall2 | 0.00% | 0.32% | 0.49% | 0.14% | 0.12% | 0.21% | 93.3% | 95.9% |
| Poznan_Street | 0.00% | 0.22% | 0.22% | 0.05% | 0.06% | 0.06% | 88.5% | 90.7% |
| Undo_Dancer | 0.00% | 0.48% | 0.47% | 0.12% | 0.10% | 0.11% | 99.0% | 99.5% |
| Shark | 0.00% | -0.07% | -0.03% | 0.00% | 0.01% | 0.03% | 108.8% | 109.5% |
| 1024x768 | 0.00% | 0.26% | 0.26% | 0.10% | 0.10% | 0.08% | 97.5% | 96.7% |
| 1920x1088 | 0.00% | 0.18% | 0.19% | 0.06% | 0.06% | 0.08% | 95.9% | 97.6% |
| average | 0.00% | 0.21% | 0.21% | 0.07% | 0.07% | 0.08% | 96.5% | 97.3% |

The coding performance for Test#2 is shown in Table 2. As reported in Error: Reference source not found, there is only 0.02% coding loss for the synthesized views when the DMP is set to be the default set of motion parameters and the pruning is not change

Table 2: Comparison of Test#2 with HTM-9.0r1 (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.05% | 0.05% | 0.02% | 0.03% | 0.00% | 111.1% | 112.8% |
| Kendo | 0.00% | 0.02% | -0.08% | -0.02% | 0.00% | -0.01% | 95.3% | 96.0% |
| Newspaper_CC | 0.00% | -0.07% | 0.05% | 0.00% | 0.00% | 0.01% | 105.2% | 105.9% |
| GT_Fly | 0.00% | -0.04% | -0.01% | 0.00% | 0.00% | 0.02% | 100.8% | 97.2% |
| Poznan_Hall2 | 0.00% | 0.14% | 0.15% | 0.04% | 0.03% | 0.11% | 87.6% | 93.7% |
| Poznan_Street | 0.00% | 0.04% | -0.13% | -0.01% | -0.01% | -0.02% | 91.4% | 92.3% |
| Undo_Dancer | 0.00% | 0.08% | 0.01% | 0.01% | 0.01% | 0.02% | 100.3% | 100.0% |
| Shark | 0.00% | 0.10% | 0.01% | 0.01% | 0.00% | 0.02% | 118.6% | 112.7% |
| 1024x768 | 0.00% | 0.00% | 0.01% | 0.00% | 0.01% | 0.00% | 103.9% | 104.9% |
| 1920x1088 | 0.00% | 0.06% | 0.00% | 0.01% | 0.01% | 0.03% | 99.7% | 99.2% |
| average | 0.00% | 0.04% | 0.00% | 0.01% | 0.01% | 0.02% | 101.3% | 101.3% |

# References

- J. An, K. Zhang, J. Lin, S. Lei, “3D-CE3: Sub-PU level inter-view motion prediction”, JCT3V-E0184, Geneva, Switzerland, 25 Oct. – 01 Nov. 2013.

- https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-9.0r1/

- D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments”, JCT3V-F1100, Geneva, Switzerland, 25 Oct. – 01 Nov. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).