| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0121 |
| --- | --- |

| Title: | CE4: Further improvements on advanced residual prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Ying Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA Qin Yu Siwei Ma School of EECS, Peking University, Institute of Digital Media, Beijing, China | Tel: Email: | 1-858-651-6660 lizhang@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated, Peking University |  |  |

_____________________________

# Abstract

In current 3D-HEVC, the advanced residual prediction (ARP) is employed to code the residue of the current prediction unit in non-base texture views more efficiently. The techniques in this proposal include: 1) block-level ARP by splitting one prediction unit into several blocks to perform ARP separately; 2) disabling of the Illumination Compensation (ic_flag not signalled) when the ARP is enabled. With these improvements, it is observed that there is a coding gain of 0.2% in terms of BD rate for both texture views and synthesized views compared to HTM under the common test conditions.

# Introduction

In 3D-HEVC, advanced residual prediction (ARP), as proposed in Error: Reference source not found, is enabled for non-base texture views. Error: Reference source not found illustrates the prediction structure of ARP in multiview video coding and 3D video coding for one reference picture list.

(a) Temporal ARP			(b) Inter-view ARP

Figure 1. Prediction structure of ARP in 3D-HEVC.

In ARP, three additional blocks in addition to current prediction unit (PU) are identified. They are defined as: temporal reference block in the current view (denoted by CurrRef), inter-view reference block (denoted by Base) within the picture of the same access unit as the current picture in base view and the reference block of Base (denoted by BaseRef) in the base view and an access unit different from the access unit containing the current picture. More specifically, the vectors utilized in identifying these blocks are listed in Table 1:

Table 1: Motion vectors utilized in ARP

|  | Temporal ARP | Inter-view ARP |
| --- | --- | --- |
| Base | DV from NBDV process (denoted by derivedMv) | Coded disparity motion vector of current PU (denoted by cMv) |
| CurrRef | Coded temporal motion vector of current PU (denoted by cMv) | Temporal motion vector of the block covering center position of Base (denoted by derivedMv) |
| BaseRef | cMv + derivedMv | cMv + derivedMv |

Note in Table 1, the temporal ARP indicates the case that current reference picture is a temporal reference picture while the inter-view ARP indicates the case that current reference picture is an inter-view reference picture. Error: Reference source not found depicts the prediction structure for both cases.

After the additional three blocks are identified, the residual is generated by current PU and its reference block (either CurrRef in temporal prediction or Base in inter-view prediction) and the rest two blocks are utilized to generate the residual predictor. In addition, a weighting factor w (being 0, 1 or 0.5) is further multiplied to the residual predictor and the weighted residual predictor is added to current temporal reference block to generate the final prediction of current block for one reference picture list.

The current design of ARP may be further improved due to the following reasons:

- In ARP process, only one vector, derivedMv (green solid arrow line in Error: Reference source not found) is obtained either from the coded disparity motion vectors of spatial/temporal neighbouring blocks, as depicted in Error: Reference source not found (a), (i.e., NBDV result of current PU) or from the temporal motion vectors of the block covering the center position in the inter-view reference block, as depicted in Error: Reference source not found (b). These derived motion vectors may be not accurate enough, especially for larger PUs.

- The current 3D-HEVC design doesn’t enable Illumination Compensation (IC) when ARP is applied (the weighting factor for ARP is not zero). However, the signalling of ARP weighting factor indices and the signaling of Illumination Compensation (IC) [1] are still both enabled.

# Proposed Scheme

To solve the above problems, in this proposal, two aspects are proposed to further improve the performance of ARP. Each aspect will be described in details in the following sub-sections. Both aspects have been proposed in JCT3V-F0123 [2].

## Block-level ARP

When block-level ARP is enabled, each PU is firstly split into several blocks (8 x 8 block size is used in the simulations), and each block share the same motion information as current PU. However, the derivedMv (i.e., disparity vector in temporal ARP or temporal motion vector in inter-view ARP) could be updated for each 8x8 block.

For Temporal ARP, the default derivedMv (denoted by DV[ i ] for the i-th 8x8 block in Figure 2 (a)) is firstly set to be the DV from NBDV process, as in current ARP. When the block covering the center position of the i-th 8x8 block within CurrRef contains a disparity motion vector, DV[ i ] is updated to be that disparity motion vector.

For Inter-view ARP, the default derivedMv (denoted by mvLX[ i ] for the i-th 8x8 block in Figure 2 (b)) is set to the temporal motion vector associated with a block covering the center position of Base, as in current ARP. When the block covering the center position of the i-th 8x8 block within Base contains a temporal motion vector, mvLX[ i ] is updated to be that temporal motion vector.

As illustrated above, for both cases, only the block level (8x8) motion information of the reference block located by current PU’s motion vector, either dotted line block CurrRef in Figure 2 (a) or Base in Figure 2 (b), is accessed to generate the final residual predictor.

With the proposed block-level ARP, the energy of the current residual block is further eliminated. Therefore, the residual of chroma components is not coded if the current PU is coded with ARP.

(a)	Temporal ARP				(b) Inter-view ARP

Figure 2. Prediction structure of block-level ARP.

## Disabling of IC when ARP is enabled

In current 3D-HEVC, the usage of IC is indicated by a flag, i.e., ic_flag which may be signalled at the coding unit (CU) level while the ARP weighting factor, when signalled, is also at CU-level. In addition, ARP and IC couldn’t be applied together for one inter-coded CU. Therefore, it is proposed that the signalling of ic_flag is always skipped when the ARP is enabled (i.e., ARP weighting factor is unequal to 0), to avoid the unnecessary signaling overhead of ic_flag.

We consider this aspect of the proposal as a clean-up for the current 3D-HEVC design.

# Experimental Results

This section provides simulation results of the proposed ARP improvement in comparison with the 3DV-HTM anchor. The platform 3D-HTM 9.0r1 [3] is utilized and the proposed method is implemented on top of it. All the simulation tests are performed based on the common test conditions [4].

To have a better understanding of the performance of the proposed two aspects related to ARP, separate results are also provided. Therefore, the following cases are tested and the corresponding performance compared to 3D-HTM anchor are summarized and listed in the following parts:

- Test #1: the full proposal as described in section 2;

- Test #2: block-based ARP;

- Test #3: disable of IC when ARP is enabled.

The coding performance of the whole proposal is given in Table 2. As shown in Table 2, the overall average bitrate reduction is 0.2% for the decoded texture views, coded and synthesized views. For dependent views, the coding gain could be up to 1.3%.

Table 2: Coding gain of the proposal with respect to anchor for 3-view case (Test #1)

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -1.0% | -1.2% | -0.4% | -0.4% | -0.5% | 104% | 103% | 103% |
| Kendo | -0.9% | -1.3% | -0.5% | -0.4% | -0.4% | 103% | 103% | 103% |
| Newspapercc | -0.6% | -0.5% | -0.2% | -0.2% | -0.2% | 108% | 102% | 106% |
| GhostTownFly | -0.2% | -0.2% | 0.0% | 0.0% | 0.0% | 103% | 100% | 101% |
| PoznanHall2 | -1.3% | -1.2% | -0.5% | -0.4% | -0.3% | 99% | 101% | 98% |
| PoznanStreet | -0.4% | -0.2% | -0.1% | -0.1% | -0.1% | 99% | 102% | 98% |
| UndoDancer | -0.5% | -0.8% | -0.1% | -0.1% | -0.1% | 110% | 101% | 107% |
| Shark | -0.5% | -0.6% | -0.1% | -0.1% | -0.1% | 119% | 102% | 114% |
| 1024x768 | -0.9% | -1.0% | -0.4% | -0.3% | -0.4% | 105% | 103% | 104% |
| 1920x1088 | -0.6% | -0.6% | -0.2% | -0.2% | -0.1% | 106% | 101% | 104% |
| average | -0.7% | -0.7% | -0.2% | -0.2% | -0.2% | 106% | 102% | 104% |

The coding performance of the proposed first improvement (i.e., block-based ARP) is given in Table 3. As shown in Table 3, the overall average bitrate reduction is 0.2% for the decoded texture views, coded and synthesized views.

Table 3: Coding gain with respect to anchor for 3-view case (Test #2): block-level ARP

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.7% | -1.1% | -0.3% | -0.3% | -0.3% | 104% | 102% | 103% |
| Kendo | -0.5% | -0.8% | -0.3% | -0.2% | -0.3% | 103% | 103% | 103% |
| Newspapercc | -0.5% | -0.5% | -0.2% | -0.2% | -0.2% | 108% | 102% | 106% |
| GhostTownFly | -0.2% | -0.3% | 0.0% | 0.0% | 0.0% | 103% | 101% | 101% |
| PoznanHall2 | -1.0% | -0.9% | -0.4% | -0.4% | -0.2% | 99% | 102% | 98% |
| PoznanStreet | -0.4% | -0.3% | -0.1% | -0.1% | -0.1% | 99% | 102% | 98% |
| UndoDancer | -0.4% | -0.8% | -0.1% | -0.1% | -0.2% | 110% | 101% | 107% |
| Shark | -0.5% | -0.6% | -0.1% | -0.1% | -0.1% | 119% | 103% | 114% |
| 1024x768 | -0.6% | -0.8% | -0.2% | -0.2% | -0.3% | 105% | 103% | 104% |
| 1920x1088 | -0.5% | -0.6% | -0.1% | -0.1% | -0.1% | 106% | 102% | 104% |
| average | -0.5% | -0.7% | -0.2% | -0.2% | -0.2% | 106% | 102% | 104% |

The coding performance of the proposed second improvement (i.e., disable of IC when ARP is enabled) is given in Table 4. As shown in Table 4, the overall average bitrate reduction is 0.1% for the decoded texture views.

Table 4: Coding gain with respect to anchor for 3-view case (Test #3): Disable of IC when ARP is enabled

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.4% | -0.3% | -0.1% | -0.1% | -0.2% | 103% | 99% | 103% |
| Kendo | -0.5% | -0.5% | -0.2% | -0.2% | -0.2% | 105% | 99% | 105% |
| Newspapercc | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 105% | 100% | 104% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 90% | 99% | 92% |
| PoznanHall2 | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 85% | 100% | 86% |
| PoznanStreet | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 95% | 100% | 94% |
| UndoDancer | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 92% | 100% | 90% |
| Shark | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 106% | 100% | 102% |
| 1024x768 | -0.3% | -0.3% | -0.1% | -0.1% | -0.1% | 104% | 100% | 104% |
| 1920x1088 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 93% | 100% | 93% |
| average | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 98% | 100% | 97% |

The encoding and rendering estimates of the proposed scheme are calculated on the Linux cluster; these estimates may not be accurate due to parallel simulations. The decoding time is calculated using one single machine.

# References

- G. Tech, K. Wegner, Y. Chen and S. Yea, “3D-HEVC Draft Text 2,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-F1001, 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013.

- L. Zhang, Y. Chen, M. Karczewicz, “CE4: Further improvements on advanced residual prediction,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-F0123, 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013.

- 3DV-HTM version 9.0r1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-9.0r1/

- D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-F1100, 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).