| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0121 _proposed text |
| --- | --- |

| Title: | Proposed text for JCT3V-G0121 based on 3D-HEVC Draft Text 2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Ying Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA Qin Yu Siwei Ma School of EECS, Peking University, Institute of Digital Media, Beijing, China | Tel: Email: | 1-858-651-6660 lizhang@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated, Peking University |  |  |

_____________________________

# Abstract

All the changes are highlighted in green or red strikethrough.

# Syntax tables

H.7.3.8.5	General Coding unit syntax

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |  |
| --- | --- | --- |
| if( transquant_bypass_enable_flag ) { |  |  |
| cu_transquant_bypass_flag | ae(v) |  |
| } |  |  |
| if( slice_type != I ) |  |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |  |
| nCbS = ( 1 << log2CbSize ) |  |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) { |  |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |  |
| if ( iv_res_pred_flag[ nuh_layer_id ] && RpRefPicAvailFlag ) |  |  |
| iv_res_pred_weight_idx | ae(v) |  |
| if ( !iv_res_pred_weight_idx && icEnableFlag ) |  |  |
| ic_flag | ae(v) |  |
| } |  |  |
| else { |  |  |
| … |  |  |
| } else { |  |  |
| … |  |  |
| } |  |  |
| } |  |  |
| if ( !iv_res_pred_weight_idx && icEnableFlag ) |  |  |
| ic_flag | ae(v) |  |
| if( DepthFlag && PredMode[ x0 ][ y0 ] ! = MODE_INTRA && !skip_flag[ x0 ][ y0 ] ) |  |  |
| … |  |  |

#### 7.3.8.8	Transform tree syntax

| transform_tree( x0, y0, xBase, yBase, log2TrafoSize, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( log2TrafoSize <= Log2MaxTrafoSize && log2TrafoSize > Log2MinTrafoSize && trafoDepth < MaxTrafoDepth && !( IntraSplitFlag && ( trafoDepth = = 0 ) ) ) |  |
| split_transform_flag[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if(!iv_res_pred_weight_idx && log2TrafoSize > 2 ) { |  |
| if( trafoDepth = = 0 ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if( trafoDepth = = 0 ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| x1 = x0 + ( 1 << ( log2TrafoSize − 1 ) ) |  |
| y1 = y0 + ( 1 << ( log2TrafoSize − 1 ) ) |  |
| transform_tree( x0, y0, x0, y0, log2TrafoSize − 1, trafoDepth + 1, 0 ) |  |
| transform_tree( x1, y0, x0, y0, log2TrafoSize − 1, trafoDepth + 1, 1 ) |  |
| transform_tree( x0, y1, x0, y0, log2TrafoSize − 1, trafoDepth + 1, 2 ) |  |
| transform_tree( x1, y1, x0, y0, log2TrafoSize − 1, trafoDepth + 1, 3 ) |  |
| } else { |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| transform_unit( x0, y0, xBase, yBase, log2TrafoSize, trafoDepth, blkIdx ) |  |
| } |  |
| } |  |

Semantics

cbf_cb[ x0 ][ y0 ][ trafoDepth ] equal to 1 specifies that the Cb transform block contains one or more transform coefficient levels not equal to 0. The array indices x0, y0 specify the top-left location ( x0, y0 ) of the considered transform unit. The array index trafoDepth specifies the current subdivision level of a coding block into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding blocks.

When cbf_cb[ x0 ][ y0 ][ trafoDepth ] is not present, the value of cbf_cb[ x0 ][ y0 ][ trafoDepth ] is inferred as follows:

- If iv_res_pred_weight_idx is equal to 0 and trafoDepth is greater than 0 and log2TrafoSize is equal to 2, cbf_cb[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ]

- Otherwise, cbf_cb[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 0.

cbf_cr[ x0 ][ y0 ][ trafoDepth ] equal to 1 specifies that the Cr transform block contains one or more transform coefficient levels not equal to 0. The array indices x0, y0 specify the top-left location ( x0, y0 ) of the considered transform unit. The array index trafoDepth specifies the current subdivision level of a coding block into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding blocks.

When cbf_cr[ x0 ][ y0 ][ trafoDepth ] is not present, the value of cbf_cr[ x0 ][ y0 ][ trafoDepth ] is inferred as follows:

- If iv_res_pred_weight_idx is equal to 0 and trafoDepth is greater than 0 and log2TrafoSize is equal to 2, cbf_cr[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ]

- Otherwise, cbf_cr[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 0.

# Decoding process

- H.8.5.3.3.7	Bilinear sample interpolation and residual prediction process

The process is only invoked if res_pred_flag is equal to 1.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture,

- a luma location ( xBl, yBl ) specifying the top-left sample of the current luma prediction block relative to the top-left sample of the current luma coding block,

- a variable nCbS specifying the size of the current luma coding block,

- variables nPbW and nPbH specifying the width and the height, respectively, of the current prediction unit, prediction list utilization flags, predFlagL0 and predFlagL1,

- the prediction list indication X,

- the prediction list utilization flag predFlagLX,

- the reference index refIdxLX,

- the motion vectors mvLX, mvCLX

Outputs of this process are:

- the (nPbW)x(nPbH) array predSamplesLXL,

- the (nPbW / 2)x(nPbH / 2) arrays predSamplesLXCb and predSamplesLXCr.

The location ( xP, yP )  is derived by:

  - xP = xCb + xBl		(Error: Reference source not found-1)

  - yP = yCb + yBl		(Error: Reference source not found-2)

The variable ivRefFlag is set equal to ( DiffPicOrderCnt( currPic, RefPicListX[ refIdxLX ] ) = =  0 ), and the  variable availFlag is set equal to 0.

- when ivRefFlag is equal to 1, the derivation process for a motion vector from a reference block for residual prediction as specified in subclause  is invoked with ( xP , yP ), nPbW and nPbH, RefPicListX[ refIdxLX ], and mvLX, as inputs, and availFlagDefault, motion vector mvTDefault and prediction list utilization variable YDefault as outputs.

- Set arpSize to be equal to 8.

- For each block (blkX=0..nPbW / arpSize -1, blkY=0..nPbH / arpSize -1), the following applies in order:

Depending on ivRefFlag and RpRefIdxLX, the following applies:

- If ivRefFlag is equal to 0 and RpRefIdxLX is not equal to −1, the variable availFlag is set equal to 1, the variable refIdxLX is set equal to RpRefIdxLX and the residual prediction motion vector scaling process as specified in subclause Error: Reference source not found is invoked with the prediction list utilization variable equal to X, the motion vector mvLX, and the RefPicListX[ refIdxLX ] and as inputs and modified mvLX as output.

- Otherwise, when ivRefFlag is equal to 1, the following applies:

- The derivation process for a motion vector from a reference block for residual prediction as specified in subclause  is invoked with ( xP + blkX* arpSize, yP + blkY* arpSize ), arpSizenPbW and arpSizenPbH, RefPicListX[ refIdxLX ], and mvLX, as inputs, and availFlag, view order index refVIdx, motion vector mvT and prediction list utilization variable Y as outputs.

- When ivRefFlag is equal to 1 and availFlagDefault is equal to 1, and availFlag is equal to 0 and RpRefIdxLX is not equal to −1, availFlag is set equal to 1, mvT is set equal to mvTDefault(0, 0), Y is set equal to YDefaultX.

The motion vector mvCLX is set equal to mvLX.

The arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr  are derived as specified in the following:

- The reference picture consisting of an ordered two-dimensional array refPicLXL of luma samples and two ordered two-dimensional arrays refPicLXCb and refPicLXCr of chroma samples is derived by invoking the process specified in subclause 8.5.2.2.1 with currRefIdx as input.

- The arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr are derived by invoking the bilinear sample interpolation process specified in subclause Error: Reference source not found with the luma locations ( xCb, yCb ), ( xBl, yBl ), , the luma prediction block width nPbW, the luma prediction block height nPbH,, the motion vectors mvLX, mvCLX, and the reference arrays with refPicLXL, refPicLXCb and refPicLXCr. as the inputs.

When availFlag is equal to 1 and iv_res_pred_weight_idx is not equal to 0, the following applies:

- Depending on ivRefFlag, the variables rpPic, rpRefPic, mvRp and curRefIdx are derived as specified in the following:

  - If ivRefFlag is equal to 0, the following applies:

    - Let rpPic be the picture with PicOrderCnt( rpPic ) equal to PicOrderCntVal and ViewIdx equal to availFlag ? refVIdx: RefViewIdx[ xP ][ yP ].

    - Let rpRefPic be the picture with PicOrderCnt( rpRefPic ) equal to RefPicListX[ RpRefIdxLX ] ) and ViewIdx equal to availFlag ? refVIdx: RefViewIdx[ xP ][ yP ],

    - The variable mvRp is set equal to availFlag ? mvT: MvDisp[ xP ][ yP ].

    - The variable curRefIdx is set equal to RpRefIdxLX.

  - Otherwise (ivRefFlag is equal to 1), the following applies:

    - Let rpPic be the picture RefPicListY[ RpRefIdxLY ].

    - Let rpRefPic be the picture with PicOrderCnt( rpRefPic ) equal to PicOrderCnt( rpPic ) and ViewIdx equal to RefViewIdx[ xP ][ yP ]

    - The variable mvRp is set equal to mvT.

    - The variable currRefIdx is set equal to RpRefIdxLY.

- The arrays rpSamplesLXL, rpSamplesLXCb, and rpSamplesLXCr are derived as specified in the following:

  - Let the reference picture sample arrays rpPicLXL, rpPicLXCb, and rpPicLXCr corresponding to decoded sample arrays SL, SCb, SCr derived in subclause 8.7 for the previously-decoded picture rpPic.

  - The arrays rpSamplesLXL, rpSamplesLXCb, and rpSamplesLXCr are derived by invoking the bilinear sample interpolation process specified in subclause Error: Reference source not found with the luma locations ( xCb, yCb ) and ( xBl, yBl ), the luma prediction block width nPbW, the luma prediction block height nPbH, the motion vectors mvLX equal to mvRp and mvCLX equal to mvRp, and the reference arrays with rpPicLXL, rpPicLXCb and rpPicLXCr as the inputs.

- The arrays rpRefSamplesLXL, rpRefSamplesLXCb, and rpRefSamplesLXCr  are derived as specified in the following:

  - Let the reference picture sample arrays rpRefPicLXL, rpRefPicLXCb, and rpRefPicLXCr corresponding to decoded sample arrays SL, SCb, SCr derived in subclause 8.7 for the previously-decoded picture rpRefPic.

  - The arrays rpRefSamplesLXL, rpRefSamplesLXCb, and rpRefSamplesLXCr are derived by invoking the bilinear sample interpolation process specified in subclause Error: Reference source not found with the luma locations ( xCb, yCb ), ( xBl, yBl ), the luma prediction block width nPbW, the luma prediction block height nPbH,, the motion vector mvLX equal to ( mvLX + mvRp ) and the motion vector mvCLX equal to ( mvCLX + mvRp ), and the reference arrays with rpRefPicLXL, rpRefPicLXCb and rpRefPicLXCr as the inputs.

- The variable shiftVal is set equal to ( iv_res_pred_weight_idx − 1 ).

- The modified prediction samples predSamplesLXL[ x + blkX*arpSize ][ y + blkY*arpSize ] with x = 0..(arpSize-1)( nPbW ) − 1 and y = 0..(arpSize-1)( nPbH ) − 1 are derived as specified in the following:

  - predSamplesLXL[ x ][ y ] =	 predSamplesLXL[ x ][ y ] + 
	( ( rpSamplesLXL[ x ][ y ] − rpRefSamplesLXL[ x ][ y ] )  >>  shiftVal )	(Error: Reference source not found-3)

- The modified prediction samples predSamplesLXCb[ x + blkX*arpSize/2 ][ y+ blkY*arpSize/2 ] with x = 0..(arpSize/2-1)( nPbW /2 ) − 1 and y = 0..(arpSize/2-1) ( nPbH /2 )−1 are derived as specified in the following:

  - predSamplesLXCb[ x ][ y ] =	 predSamplesLXCb[ x ][ y ] + 
	( ( rpSamplesLXCb[ x ][ y ] − rpRefSamplesLXCb[ x ][ y ] )  >>  shiftVal )	(Error: Reference source not found-4)

- The modified prediction samples predSamplesLXCr[ x + blkX*arpSize/2 ][ y+ blkY*arpSize/2 ] with x = 0..(arpSize/2-1)( nPbW /2 ) − 1 and y = 0..(arpSize/2-1) ( nPbH /2 )−1 are derived as specified in the following:

  - predSamplesLXCr[ x ][ y ] =	 predSamplesLXCr[ x ][ y ] + 
	( ( rpSamplesLXCr[ x ][ y ] − rpRefamplesLXCr[ x ][ y ] )  >>  shiftVal )	(Error: Reference source not found-5)

- H.8.5.3.3.7.4	Derivation process for a motion vector from a reference block for residual prediction

Inputs to this process are:

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables nPbW and nPbH specifying the width and the height, respectively, of the current prediction unit,

- a reference picture refPic,

- a motion vector mvDisp

- Outputs of this process are:

- a flag availFlag

- an view order index refVIdx,

- a motion vector mvT

- prediction list utilization variable Y.

The variable availFlag is set to 0 and the reference luma location ( xRef, yRef ) in refPicLX is derived by

  - xRef = Clip3( 0, PicWidthInSamplesL – 1, xP + ( nPSW  >>  1 ) + ( ( mvDisp[ 0 ] + 2 )  >>  2 ) )	(Error: Reference source not found-6)
yRef = Clip3( 0, PicHeightInSamplesL – 1, yP + ( nPSH  >>  1 ) + ( ( mvDisp[ 1 ] + 2 )  >>  2 ) )	(Error: Reference source not found-7)

Let variable refCU and refPU be the coding unit and prediction unit that cover the luma location ( xRef, yRef ) in refPic, respectively.

When the variable PredMode for the coding unit refCU is equal to MODE_SKIP or MODE_INTER, the following ordered steps apply for X in the range of 0 to 1, inclusive:

  - The variable refPredFlagLX is set equal to the prediction utilization flag predFlagLX of the prediction unit refPU.

  - When availFlag is equal to 0 and refPredFlagLX is equal to 1, the following applies:

    - Let refPicListRefX be the reference picture list X of refPic.

    - Let mvLX and refIdxLX be the motion vector and reference index of the prediction unit refPU corresponding to refPicListRefX, respectively.

    - When refPicListRefX[ refIdxLX ] is a temporal reference picture of refPic DiffPicOrderCnt( currPic, refPic) is equal to 0, DiffPicOrderCnt( refPic, refPicListRefX[ refIdxLX ]) is unequal to 0,and RpRefIdxLX is not equal to –1, availFlag is set to 1, refVIdx is set equal to the view order index of refPic, Y is set equal to X and the residual prediction motion vector scaling process as specified in subclause Error: Reference source not found is invoked with the prediction list utilization variable equal to X, the motion vector mvLX, and the reference picture refPicListRefX[ refIdxLX ] as the inputs, and the output being mvT.

    - When DiffPicOrderCnt( refPic, refPicListRefX[ refIdxLX ]) is equal to 0, DiffPicOrderCnt( currPic, refPic) is unequal to 0, availFlag is set to 1, refVIdx is set equal to the view order index of refPicListRefX[ refIdxLX ], mvT is set equal to mvLX and Y is set equal to X.