| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0122 |
| --- | --- |

| Title: | CE5: Generic SDC for all Intra modes in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu Ying Chen 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | hongbinl@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports results of JCT3V-F0126. Segment wise delta DC (SDC) coding is generalized to all HEVC intra modes and additional depth intra modes. It is reported that -0.50% average coding gain is achieved on synthesized view in CTC case.

# Introduction

In SDC coding of 3D-HEVC, only one DC residual is coded for each partition within a prediction unit (PU) coded with intra prediction mode. Optionally, depth lookup table (DLT) can be used to code the DC residual. When DLT is used, DC residual is generated as the difference between DLT index of original DC signal and DLT index of predicted DC signal. Because the range of indices to the DLT table is usually much smaller than the range of depth sample, DLT can further reduce bit rate of residual.

SDC can efficiently reduce bit rate of depth residual. However, it is applied only to Planar mode and DMM mode 1, and it does not work in HEVC intra modes (excluding Planar mode) or DMM4 mode. DMM modes are more like PU partitioning/prediction modes, however SDC mode (including DLT) is mainly for the purpose of coding the residual after the prediction based on the different characteristics of the depth. Therefore, it makes sense to extend the good functionality of the SDC mode also to the normal HEVC Intra modes.

# Proposed method

To further reduce the bit rate of depth residual, SDC is generalized to all depth intra prediction modes including HEVC intra prediction modes and DMM prediction modes. This not only makes the efficient prediction in HEVC to be possibly combined with the efficient residual coding introduced by SDC, but also unifies the existing syntax design related to various Intra modes.

Therefore, each of the intra prediction modes can work with the traditional HEVC (3D-HEVC) residual as well as the SDC residual.

## Detailed description

In SDC, one delta DC value needs to be generated for a whole PU or a partition.

To generate the delta DC, a predicted DC signal is calculated for each partition of the prediction unit (PU) as follows.

- HEVC intra modes

All pixels in the PU belong to one partition. Predicted DC signal is generated as the average of the four corner pixels of the pixel specific predictor of the PU.

- DMM4 mode

DMM4 mode contains two partitions. For each partition, the predicted DC signal is set to the predictor of the partition directly since all pixels within a partition share one same predictor.

When DLT is not used, delta DC is generated as the difference between original DC signal and predicted DC signal; when DLT is used, delta DC is generated as the difference between DLT index of original DC signal and DLT index of predicted DC signal.

The proposed method is only applied to CU with 2Nx2N partition pattern, and is not applied when DMM modes already encode one delta DC.

## Improvement of Delta DC (non-normative)

Encoder can use the difference between original DC and predicted DC, namely DeltaDCOrig, as the delta DC directly. However, such a value is not necessary the optimal from the VSO point of view. So the encoder can also check more delta DC values, e.g. a small set of values which are of small difference from DeltaDCOrig, and select the best one to achieve better coding performance. 3D-HTM 9.0r1 [1] and JCT3V-F0149 [2] show that checking more delta DC values can improve coding efficiency of DMM prediction modes and inter SDC respectively. This can further benefit the proposed method.

This contribution tests similar idea for intra SDC. Specifically, encoder selects best delta DC from {DeltaDCOrig – 2, DeltaDCOrig – 1, DeltaDCOrig, DeltaDCOrig + 1, DeltaDCOrig + 2} using view synthesis optimization method.

## Signalling

Because SDC coding is applied to all depth intra modes, one flag is required for all depth intra modes to indicate whether HEVC residual or SDC residual is coded. Signaling of delta DC is same with that of SDC in 3D-HEVC.

The major syntax change is in the depth_mode_parameters( ) syntax table, other small syntax changes, as well as detailed semantics and decoding processes modifications are provided in the attached document with working draft text for this proposal.

New added parts are heighted in green part and deleted parts are marked with strikethrough.

### Syntax and semantics

- H.7.3.8.5.1 	Depth mode parameter syntax

| depth_mode_parameters( x0 , y0 , log2CbSize ) { | Descriptor |
| --- | --- |
| if ( log2CbSize < 6 ) |  |
| hevc_intra_flag[ x0 ][ y0 ] | ae(v) |
| if ( PartMode[ xC ][ yC ] = = PART_2Nx2N ) |  |
| sdc_flag[ x0 ][ y0 ] | ae(v) |
| if ( !hevc_intra_flag[ x0 ][ y0 ] && ( ( log2CbSize > 3 ) ¦¦ ( log2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N ) ) ) |  |
| depth_intra_mode[ x0 ][ y0 ] | ae(v) |
| if ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_DMM_WFULL ¦ ¦ DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_SDC_DMM_WFULL ) |  |
| wedge_full_tab_idx[ x0 ][ y0 ] | ae(v) |
| if( DmmFlag[ x0 ][ y0 ] ¦ ¦ SdcFlag[ x0 ][ y0 ] ) { |  |
| dcNumSeg = ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_SDC_PLANAR ) ? 1 : 2 |  |
| depth_dc_flag[ x0 ][ y0 ] | ae(v) |
| if ( depth_dc_flag[ x0 ][ y0 ] ) |  |
| for( i = 0; i < dcNumSeg; i ++ ) { |  |
| depth_dc_abs[ x0 ][ y0 ][ i ] | ae(v) |
| if ( depth_dc_abs[ x0 ][ y0 ][ i ]) |  |
| depth_dc_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |

- H.7.3.8.5 Coding unit syntax

| coding_unit( x0, y0, log2CbSize , ctDepth) { |  |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| … |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : 0 |  |
| log2PbSize = log2CbSize − ( PartMode = = PART_NxN ? 1 : 0 ) |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( vps_depth_modes_flag[ nuh_layer_id ] ) |  |
| depth_mode_parameters( x0 + i , y0+ j , log2CbSize ) |  |
| if( DepthIntraMode[ x0 + i ][ y0 + j ] = = INTRA_DEP_NONE ) |  |
| if( hevc_intra_flag[ x0 ][ y0 ]) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( DepthIntraMode[ x0 + i ][ y0 + j ] = = INTRA_DEP_NONE) { |  |
| if( hevc_intra_flag[ x0 ][ y0 ]) { |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] ) |  |
| mpm_idx[ x0 + i ][ y0+ j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| } |  |
| if ( !SdcFlag[ x0 ][ y0 ] ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| … |  |
| if( !pcm_flag ) { |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0])) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf && !inter_sdc_flag ) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| … |  |

- H.7.4.9.5.1 Depth mode parameter semantics

hevc_intra_flag[ x0 ][ y0 ] equal to 1 specifies that intra modes with intraPredMode in the range of 0 to 34 is used for the current prediction unit. hevc_intra_flag[ x0 ][ y0 ] equal to 0 specifies that another intra mode is used for the current prediction unit. When not present, hevc_intra_flag[ x0 ][ y0 ] is inferred to be equal to 1.

sdc_flag[ x0 ][ y0 ] equal to 1 specifies that the residual of each partition of the current prediction unit is only represented by sdc_residual_flag[ x0 ][ y0 ][ i ], sdc_residual_sign_flag [ x0 ][ y0 ][ i ] and sdc_residual_abs_minus1[ x0 ][ y0 ][ i ]. sdc_flag[ x0 ][ y0 ] equal to 0 specifies that more residual may be present in the transform tree. When not present, sdc_flag[ x0 ][  y0 ] is inferred to be equal to 0.

The variable SdcFlag[ x0 ][ y0 ] is set to be equal to sdc_flag[ x0 ][ y0 ].

The variable DmmFlag[ x0 ][ y0 ] is derived as specified in the following:

- DmmFlag[ x0 ][ y0 ] =	!hevc_intra_flag[ x0 ][ y0 ]                      	(Error: Reference source not found-1)

depth_intra_mode[ x0 ][ y0 ] specifies the depth intra mode of the current prediction unit. depth_intra_mode[ x0 ][ y0 ] equal to 0 specifies that DepthIntraMode[ x0 ][ y0 ] is equal to INTRA_DEP_DMM_WFULL, and depth_intra_mode[ x0 ][ y0 ] equal to 1 specifies that DepthIntraMode[ x0 ][ y0 ] is equal to INTRA_DEP_DMM_CPREDTEX. When not present, if hevc_intra_flag[ x0 ][ y0 ] is equal to 0, DepthIntraMode[ x0 ][ y0 ] is inferred to be equal to INTRA_DEP_DMM_WFULL; otherwise, if hevc_intra_flag[ x0 ][ y0 ] is equal to 1, DepthIntraMode[ x0 ][ y0 ] is inferred to be equal to INTRA_DEP_NONE. Error: Reference source not found specifies the value of the variable depthIntraModeMaxLen depending on depthIntraModeSet and the value of the variable DepthIntraMode and the associated name depending on the on depth_intra_mode and depthIntraModeSet.

The variable SdcFlag[ x0 ][ y0 ] is derived as specified in the following:

- SdcFlag[ x0 ][ y0 ] =	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_SDC_PLANAR )  | |  	(Error: Reference source not found-2)
	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_SDC_DMM_WFULL )

The variable DmmFlag[ x0 ][ y0 ] is derived as specified in the following:

- DmmFlag[ x0 ][ y0 ] =	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_DMM_WFULL )  | |  	(Error: Reference source not found-3)
	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_DMM_CPREDTEX )

- Table Error: Reference source not found-1 – Specification of DepthIntraMode and associated name depending on depthIntraModeSet and depth_intra_mode and specification of and depthIntraModeMaxLen depending on depthIntraModeSet

|  | depthIntraModeSet | 0 | 1 | 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | depthIntraModeMaxLen | 1 | 3 | 3 |  |
| DepthIntraMode | Associated name | depth_intra_mode |  |  |  |
| 0 | INTRA_DEP_SDC_PLANAR | 0 | - | 0 |  |
| 1 | INTRA_DEP_NONE | 1 | 0 | 1 |  |
| 2 | INTRA_DEP_SDC_DMM_WFULL | - | - | 2 |  |
| 3 | INTRA_DEP_DMM_WFULL | - | 1 | 3 |  |
| 4 | INTRA_DEP_DMM_CPREDTEX | - | - | 4 |  |

## Enable DMM4 for 4x4 block

In 3D-HEVC, DMM4 is disabled for 4x4 block normatively. However, there is no such restriction for DMM1 or HEVC intra prediction modes. By enabling DMM4 for 4x4 block, the decoder does not have to check CU size or partition mode of the current CU when decoding depth intra mode. Depth mode parameter syntax table in subsection 2.3.1 can be simplified as follows, where the parts marked with strikethrough are removed.

- H.7.3.8.5.1 	Depth mode parameter syntax

| depth_mode_parameters( x0 , y0 , log2CbSize ) { | Descriptor |
| --- | --- |
| if ( log2CbSize < 6 ) |  |
| hevc_intra_flag[ x0 ][ y0 ] | ae(v) |
| if ( PartMode[ xC ][ yC ] = = PART_2Nx2N ) |  |
| sdc_flag[ x0 ][ y0 ] | ae(v) |
| if ( !hevc_intra_flag[ x0 ][ y0 ] && ( ( log2CbSize > 3 ) ¦¦ ( log2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N ) ) ) |  |
| depth_intra_mode[ x0 ][ y0 ] | ae(v) |
| if ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_DMM_WFULL ) |  |
| wedge_full_tab_idx[ x0 ][ y0 ] | ae(v) |
| if( DmmFlag[ x0 ][ y0 ] ¦ ¦ SdcFlag[ x0 ][ y0 ] ) { |  |
| dcNumSeg = ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_SDC_PLANAR ) ? 1 : 2 |  |
| depth_dc_flag[ x0 ][ y0 ] | ae(v) |
| if ( depth_dc_flag[ x0 ][ y0 ] ) |  |
| for( i = 0; i < dcNumSeg; i ++ ) { |  |
| depth_dc_abs[ x0 ][ y0 ][ i ] | ae(v) |
| if ( depth_dc_abs[ x0 ][ y0 ][ i ]) |  |
| depth_dc_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |

# Simulation results

Proposed method is integrated into HTM-9.0r1 and compared with it under common test condition [3] and all intra case.

## With improvement of delta DC

As shown in Table 1 and Table 2, proposed method can achieve -0.50% and -0.93% average coding gain on synthesized view for CTC and all intra case respectively.

Table 1: Comparison of proposed method with HTM-9.0r1 (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.15% | 0.17% | 0.05% | -0.22% | -0.33% | 101.9% | 102.9% |
| Kendo | 0.00% | -0.17% | -0.11% | -0.05% | -0.53% | -0.76% | 101.8% | 95.9% |
| Newspaper_CC | 0.00% | -0.14% | 0.05% | 0.00% | -0.41% | -0.80% | 107.7% | 102.8% |
| GT_Fly | 0.00% | 0.17% | 0.04% | 0.03% | -0.25% | -0.47% | 107.6% | 101.8% |
| Poznan_Hall2 | 0.00% | 0.06% | -0.05% | 0.01% | -0.24% | -0.36% | 95.4% | 97.4% |
| Poznan_Street | 0.00% | -0.01% | -0.11% | 0.01% | -0.18% | -0.46% | 103.4% | 101.1% |
| Undo_Dancer | 0.00% | -0.15% | 0.07% | -0.02% | -0.20% | -0.41% | 107.6% | 102.7% |
| Shark | 0.00% | 0.08% | 0.06% | 0.03% | -0.25% | -0.42% | 107.5% | 101.0% |
| 1024x768 | 0.00% | -0.05% | 0.04% | 0.00% | -0.38% | -0.63% | 103.8% | 100.5% |
| 1920x1088 | 0.00% | 0.03% | 0.00% | 0.01% | -0.22% | -0.42% | 104.3% | 100.8% |
| average | 0.00% | 0.00% | 0.01% | 0.01% | -0.28% | -0.50% | 104.1% | 100.7% |

Table 2: Comparison of proposed method with HTM-9.0r1 (All intra case)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.00% | 0.00% | 0.00% | -0.44% | -0.82% | 156.7% | 98.5% |
| Kendo | 0.00% | 0.00% | 0.00% | 0.00% | -0.57% | -1.01% | 168.4% | 95.5% |
| Newspaper_CC | 0.00% | 0.00% | 0.00% | 0.00% | -0.40% | -1.10% | 151.6% | 94.0% |
| GT_Fly | 0.00% | 0.00% | 0.00% | 0.00% | -0.52% | -1.00% | 133.0% | 84.5% |
| Poznan_Hall2 | 0.00% | 0.00% | 0.00% | 0.00% | -0.25% | -0.98% | 147.7% | 85.9% |
| Poznan_Street | 0.00% | 0.00% | 0.00% | 0.00% | -0.21% | -0.92% | 152.7% | 91.6% |
| Undo_Dancer | 0.00% | 0.00% | 0.00% | 0.00% | -0.27% | -0.73% | 134.7% | 80.2% |
| Shark | 0.00% | 0.00% | 0.00% | 0.00% | -0.53% | -0.90% | 134.9% | 87.9% |
| 1024x768 | 0.00% | 0.00% | 0.00% | 0.00% | -0.47% | -0.98% | 158.9% | 96.0% |
| 1920x1088 | 0.00% | 0.00% | 0.00% | 0.00% | -0.36% | -0.91% | 140.6% | 86.0% |
| average | 0.00% | 0.00% | 0.00% | 0.00% | -0.40% | -0.93% | 147.5% | 89.8% |

## Without improvement of delta DC

As shown in Table 3 and Table 4, proposed method can achieve -0.30% and -0.46% average coding gain on synthesized view for CTC and all intra case respectively.

Table 3: Comparison of proposed method with HTM-9.0r1 (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.11% | 0.08% | 0.01% | -0.21% | -0.18% | 118.5% | 114.2% |
| Kendo | 0.00% | -0.19% | -0.07% | -0.06% | -0.43% | -0.48% | 90.1% | 90.1% |
| Newspaper_CC | 0.00% | -0.18% | 0.01% | -0.03% | -0.37% | -0.39% | 98.0% | 99.3% |
| GT_Fly | 0.00% | 0.39% | 0.16% | 0.06% | -0.24% | -0.45% | 99.6% | 98.1% |
| Poznan_Hall2 | 0.00% | -0.13% | -0.04% | -0.02% | -0.25% | -0.22% | 93.8% | 101.3% |
| Poznan_Street | 0.00% | 0.02% | 0.01% | 0.01% | -0.16% | -0.22% | 94.9% | 95.9% |
| Undo_Dancer | 0.00% | -0.10% | 0.15% | 0.00% | -0.16% | -0.25% | 100.4% | 104.6% |
| Shark | 0.00% | 0.05% | 0.09% | 0.01% | -0.22% | -0.23% | 113.6% | 111.2% |
| 1024x768 | 0.00% | -0.09% | 0.00% | -0.03% | -0.34% | -0.35% | 102.2% | 101.2% |
| 1920x1088 | 0.00% | 0.04% | 0.07% | 0.01% | -0.21% | -0.27% | 100.5% | 102.2% |
| average | 0.00% | -0.01% | 0.05% | 0.00% | -0.25% | -0.30% | 101.1% | 101.8% |

Table 4: Comparison of proposed method with HTM-9.0r1 (All intra case)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.00% | 0.00% | 0.00% | -0.36% | -0.35% | 137.7% | 107.7% |
| Kendo | 0.00% | 0.00% | 0.00% | 0.00% | -0.48% | -0.48% | 119.1% | 88.4% |
| Newspaper_CC | 0.00% | 0.00% | 0.00% | 0.00% | -0.34% | -0.39% | 122.2% | 95.4% |
| GT_Fly | 0.00% | 0.00% | 0.00% | 0.00% | -0.49% | -0.66% | 116.0% | 89.1% |
| Poznan_Hall2 | 0.00% | 0.00% | 0.00% | 0.00% | -0.19% | -0.50% | 121.0% | 91.3% |
| Poznan_Street | 0.00% | 0.00% | 0.00% | 0.00% | -0.24% | -0.35% | 120.5% | 90.3% |
| Undo_Dancer | 0.00% | 0.00% | 0.00% | 0.00% | -0.26% | -0.42% | 115.0% | 85.4% |
| Shark | 0.00% | 0.00% | 0.00% | 0.00% | -0.47% | -0.52% | 114.4% | 94.7% |
| 1024x768 | 0.00% | 0.00% | 0.00% | 0.00% | -0.39% | -0.41% | 126.3% | 97.2% |
| 1920x1088 | 0.00% | 0.00% | 0.00% | 0.00% | -0.33% | -0.49% | 117.4% | 90.2% |
| average | 0.00% | 0.00% | 0.00% | 0.00% | -0.35% | -0.46% | 120.7% | 92.8% |

## With improvement of delta DC and enabling DMM4 for 4x4 block

As shown in Table 5 and Table 6, proposed method can achieve -0.49% and -0.95% average coding gain on synthesized view for CTC and all intra case respectively. When compared with subsection 3.1, it can be seen that enabling DMM4 for 4x4 block does not influence the coding performance, but it makes the depth mode parameter syntax table cleaner.

Table 5: Comparison of proposed method with HTM-9.0r1 (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.03% | 0.16% | 0.02% | -0.28% | -0.38% | 110.7% | 103.4% |
| Kendo | 0.00% | -0.31% | -0.09% | -0.06% | -0.51% | -0.71% | 99.8% | 92.5% |
| Newspaper_CC | 0.00% | -0.08% | 0.09% | 0.00% | -0.41% | -0.84% | 99.0% | 93.1% |
| GT_Fly | 0.00% | 0.22% | -0.09% | 0.03% | -0.25% | -0.45% | 95.9% | 91.8% |
| Poznan_Hall2 | 0.00% | -0.02% | -0.16% | -0.03% | -0.26% | -0.18% | 99.8% | 95.1% |
| Poznan_Street | 0.00% | -0.01% | -0.11% | 0.00% | -0.18% | -0.49% | 104.4% | 98.2% |
| Undo_Dancer | 0.00% | -0.09% | 0.02% | -0.01% | -0.20% | -0.43% | 109.0% | 103.5% |
| Shark | 0.00% | -0.01% | -0.05% | 0.00% | -0.27% | -0.43% | 114.3% | 107.6% |
| 1024x768 | 0.00% | -0.12% | 0.06% | -0.02% | -0.40% | -0.65% | 103.1% | 96.3% |
| 1920x1088 | 0.00% | 0.02% | -0.08% | 0.00% | -0.23% | -0.39% | 104.7% | 99.2% |
| average | 0.00% | -0.03% | -0.03% | -0.01% | -0.29% | -0.49% | 104.1% | 98.1% |

Table 6: Comparison of proposed method with HTM-9.0r1 (All intra case)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.00% | 0.00% | 0.00% | -0.44% | -0.84% | 170.7% | 101.1% |
| Kendo | 0.00% | 0.00% | 0.00% | 0.00% | -0.56% | -1.02% | 143.5% | 82.4% |
| Newspaper_CC | 0.00% | 0.00% | 0.00% | 0.00% | -0.39% | -1.15% | 145.3% | 85.0% |
| GT_Fly | 0.00% | 0.00% | 0.00% | 0.00% | -0.52% | -1.00% | 126.9% | 85.7% |
| Poznan_Hall2 | 0.00% | 0.00% | 0.00% | 0.00% | -0.25% | -0.97% | 171.8% | 97.6% |
| Poznan_Street | 0.00% | 0.00% | 0.00% | 0.00% | -0.20% | -0.93% | 141.3% | 91.9% |
| Undo_Dancer | 0.00% | 0.00% | 0.00% | 0.00% | -0.27% | -0.76% | 159.6% | 93.7% |
| Shark | 0.00% | 0.00% | 0.00% | 0.00% | -0.51% | -0.91% | 139.7% | 99.3% |
| 1024x768 | 0.00% | 0.00% | 0.00% | 0.00% | -0.46% | -1.00% | 153.2% | 89.5% |
| 1920x1088 | 0.00% | 0.00% | 0.00% | 0.00% | -0.35% | -0.91% | 147.9% | 93.7% |
| average | 0.00% | 0.00% | 0.00% | 0.00% | -0.39% | -0.95% | 149.8% | 92.1% |

# References

- https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-9.0r1/

- J. Lee, M. Park, and C. Kim, “3D-CE5 related: Simplified depth inter mode coding”, JCT3V-F0149, Geneva, Switzerland, 25 Oct. – 01 Nov. 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments”, JCT3V-F1100, Geneva, Switzerland, 25 Oct. – 01 Nov. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).