| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0123 |
| --- | --- |

| Title: | CE5 related: Simplification of 64x64 Intra SDC mode in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Qin Yu1 Hongbin Liu2 Ying Chen2 Siwei Ma1 1School of EECS, Peking University, Institute of Digital Media, Beijing, China 25775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 86-10-57760696 hongbinl@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Peking University and Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In current 3D-HEVC, the block size for Intra SDC mode can be up to 64x64. However, the maximum Intra prediction block size is 32x32 in HEVC. Although it is not clear whether enabling 64x64 Intra prediction without increasing of the transform size is a drawback for the 3D-HEVC design, an alternative way of realizing 64x64 Intra prediction, only for SDC modes is proposed. This contribution proposes a simplification method by changing a 64x64 block prediction to four 32x32 block predictions. Experimental results show that the proposed method does not have the impact on the performance.

# Introduction

In the current Intra SDC design in 3D-HEVC, the prediction block size can vary from 8x8 to 64x64. However, in HEVC, the maximum Intra prediction block size is 32x32.

In HEVC as well as HTM software, the Intra prediction module can be easily extended to 64x64. However, the only issue is that an Intra predicted block may need to be associated with a transform unit with a size not smaller than the block, but there is no 64x64 transform size in HEVC.

When SDC applies, transform is skipped, thus the issue of applying 64x64 Intra prediction is not a problem anymore.

Since the current scope of the CE is also concerning 64x64 Intra prediction in the context of SDC coding, we propose a solution to avoid 64x64 Intra prediction. The proposed method split a 64x64 block size (when coded with SDC) into four 32x32 blocks and use the same Intra 32x32 prediction as in HEVC.

- Proposal

In this proposal, we propose to split the prediction for 64x64 CU into four 32x32 blocks. The construction of prediction sample is illustrated in Figure 1. Figure 1 (a) shows the design in the current 3D-HEVC. Neighbouring reconstructed samples are utilized to predict the current 64x64 block. Figure 1 (b) shows the proposed prediction method. As illustrated, a 64x64 block is split into four 32x32 blocks and these 32x32 blocks are predicted in Z scan order. Except the first 32x32 block (which is predicted by using neighboring reconstructed samples) of the 64x64 block, the other 32x32 blocks depend on the predicted sample values of one or more 32x32 blocks of the same CU. Same padding process as in HEVC intra prediction applies when any neighboring samples are unavailable. The four 32x32 blocks share the same Intra prediction mode as signalled/derived for the current CU.

The proposed method applies to the current 3D-HEVC for the planar mode. However, the design can be made generic to all SDC modes, such as the Intra 64x64 modes in the cases when other prediction directions are supported as in [1].

Figure 1: Neighboring samples used for intra prediction of 64x64 blocks in SDC coding

# Experimental results

The proposed method was implemented on HTM-9.0r1 [2] . The results of this proposal are compared with those collected under CTC [3] as well as All-Intra configurations. Experimental results show that this method has no performance degradation. This method is also implemented on HTM-9.0r1 with the technique proposed in JCT3V-G0122 [4], which was originally proposed in [1].

Results on top of HTM-9.0r1 are shown in Table 1 and Table 2. As can be seen, proposed method has negligible influence on coding performance, i.e., 0.01% and 0.06% coding loss on synthesis view for CTC and All-Intra respectively.

Results on top of HTM-9.0r1 + JCT3V-G0122 are shown in Table 3 and Table 4. Proposed method has negligible influence on coding performance, i.e., 0.03% and 0.04% coding loss on synthesis view for CTC and All-Intra respectively.

Table 1: Simplification for 64x64 Intra SDC mode on HTM9.0r1 (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.14% | -0.01% | -0.02% | -0.01% | -0.05% | 105.0% | 103.7% | 105.4% |
| Kendo | 0.00% | -0.21% | -0.17% | -0.07% | 0.00% | -0.04% | 95.1% | 95.3% | 96.9% |
| Newspaper_CC | 0.00% | -0.12% | 0.07% | 0.00% | 0.00% | -0.03% | 99.1% | 100.2% | 99.6% |
| GT_Fly | 0.00% | 0.09% | -0.09% | 0.00% | 0.05% | 0.00% | 103.1% | 97.8% | 100.0% |
| Poznan_Hall2 | 0.00% | 0.16% | -0.03% | 0.05% | 0.08% | 0.17% | 94.3% | 98.9% | 96.7% |
| Poznan_Street | 0.00% | -0.37% | -0.16% | -0.06% | 0.01% | -0.02% | 101.3% | 97.6% | 103.2% |
| Undo_Dancer | 0.00% | 0.01% | 0.21% | 0.03% | 0.02% | -0.01% | 104.5% | 114.1% | 103.1% |
| Shark | 0.00% | 0.13% | 0.08% | 0.01% | 0.02% | 0.05% | 114.8% | 110.2% | 108.9% |
| 1024x768 | 0.00% | -0.16% | -0.04% | -0.03% | 0.00% | -0.04% | 99.7% | 99.7% | 100.6% |
| 1920x1088 | 0.00% | 0.00% | 0.00% | 0.01% | 0.04% | 0.04% | 103.6% | 103.7% | 102.3% |
| average | 0.00% | -0.06% | -0.01% | -0.01% | 0.02% | 0.01% | 102.2% | 102.2% | 101.7% |

Table 2: Simplification for 64x64 intra SDC mode on HTM9.0r1 (All-Intra)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.00% | 0.00% | 0.00% | 0.02% | 0.02% | 95.1% | 103.0% | 100.1% |
| Kendo | 0.00% | 0.00% | 0.00% | 0.00% | 0.03% | 0.02% | 109.5% | 100.4% | 105.3% |
| Newspaper_CC | 0.00% | 0.00% | 0.00% | 0.00% | 0.03% | 0.01% | 104.6% | 105.2% | 104.0% |
| GT_Fly | 0.00% | 0.00% | 0.00% | 0.00% | 0.10% | 0.09% | 108.4% | 94.9% | 107.2% |
| Poznan_Hall2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.18% | 0.18% | 111.0% | 104.5% | 106.2% |
| Poznan_Street | 0.00% | 0.00% | 0.00% | 0.00% | 0.09% | 0.08% | 114.1% | 103.9% | 112.0% |
| Undo_Dancer | 0.00% | 0.00% | 0.00% | 0.00% | 0.03% | 0.05% | 99.8% | 103.9% | 97.0% |
| Shark | 0.00% | 0.00% | 0.00% | 0.00% | 0.05% | 0.03% | 103.8% | 101.5% | 105.9% |
| 1024x768 | 0.00% | 0.00% | 0.00% | 0.00% | 0.03% | 0.02% | 103.1% | 102.9% | 103.1% |
| 1920x1088 | 0.00% | 0.00% | 0.00% | 0.00% | 0.09% | 0.09% | 107.4% | 101.7% | 105.6% |
| average | 0.00% | 0.00% | 0.00% | 0.00% | 0.07% | 0.06% | 105.8% | 102.2% | 104.7% |

Table 3: Simplification for 64x64 intra SDC mode on HTM9.0r1+ JCT3V-G0122 (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.31% | 0.03% | -0.04% | -0.05% | -0.02% | 113.1% | 107.5% | 108.9% |
| Kendo | 0.00% | -0.09% | -0.07% | -0.04% | -0.05% | 0.02% | 102.7% | 99.6% | 100.7% |
| Newspaper_CC | 0.00% | 0.09% | -0.06% | 0.00% | 0.01% | 0.06% | 94.4% | 94.4% | 94.8% |
| GT_Fly | 0.00% | 0.09% | 0.07% | 0.01% | 0.04% | 0.01% | 100.9% | 99.4% | 100.5% |
| Poznan_Hall2 | 0.00% | -0.17% | -0.07% | -0.05% | 0.03% | 0.07% | 105.7% | 100.8% | 102.8% |
| Poznan_Street | 0.00% | -0.02% | -0.09% | -0.01% | 0.00% | 0.02% | 95.2% | 92.8% | 96.1% |
| Undo_Dancer | 0.00% | 0.07% | -0.14% | 0.01% | 0.03% | 0.03% | 101.5% | 106.5% | 101.6% |
| Shark | 0.00% | 0.06% | -0.07% | 0.00% | 0.04% | 0.06% | 111.4% | 107.1% | 110.9% |
| 1024x768 | 0.00% | -0.10% | -0.03% | -0.03% | -0.03% | 0.02% | 103.4% | 100.5% | 101.5% |
| 1920x1088 | 0.00% | 0.01% | -0.06% | -0.01% | 0.03% | 0.04% | 102.9% | 101.3% | 102.4% |
| average | 0.00% | -0.04% | -0.05% | -0.02% | 0.01% | 0.03% | 103.1% | 101.0% | 102.0% |

Table 4: Simplification for 64x64 intra SDC mode on HTM9.0r1+ JCT3V-G0122 (All-Intra)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.00% | 0.00% | 0.00% | 0.02% | 0.02% | 104.9% | 105.2% | 103.1% |
| Kendo | 0.00% | 0.00% | 0.00% | 0.00% | 0.01% | 0.03% | 86.7% | 88.9% | 84.8% |
| Newspaper_CC | 0.00% | 0.00% | 0.00% | 0.00% | 0.03% | 0.01% | 99.0% | 96.0% | 94.5% |
| GT_Fly | 0.00% | 0.00% | 0.00% | 0.00% | 0.02% | 0.02% | 99.9% | 97.1% | 94.1% |
| Poznan_Hall2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.12% | 0.16% | 103.5% | 101.0% | 98.3% |
| Poznan_Street | 0.00% | 0.00% | 0.00% | 0.00% | 0.03% | 0.04% | 99.4% | 96.7% | 97.8% |
| Undo_Dancer | 0.00% | 0.00% | 0.00% | 0.00% | 0.02% | 0.03% | 112.8% | 113.7% | 108.5% |
| Shark | 0.00% | 0.00% | 0.00% | 0.00% | 0.05% | 0.04% | 95.2% | 99.6% | 96.9% |
| 1024x768 | 0.00% | 0.00% | 0.00% | 0.00% | 0.02% | 0.02% | 96.9% | 96.7% | 94.1% |
| 1920x1088 | 0.00% | 0.00% | 0.00% | 0.00% | 0.05% | 0.06% | 102.1% | 101.6% | 99.1% |
| average | 0.00% | 0.00% | 0.00% | 0.00% | 0.04% | 0.04% | 100.2% | 99.8% | 97.2% |

# Conclusion

This proposal discusses possible difference between the design of Intra prediction in HEVC and 3D-HEVC in terms of supporting 64x64 CUs. Since in such cases SDC is used and transform is skipped, it doesn’t seem to be a real problem to us. However, we provide also results on realizing 64x64 Intra prediction with Intra predictions of 32x32 blocks, similar as discussed during the development of HEVC version 1. Results show that the 64x64 Intra prediction can be easily replaced, but the relevant SDC methods, including those in the current 3D-HEVC and in JCT3V-G0122 can still be efficient.

# References

- H. Liu, Y. Chen and L. Zhang, “CE5 related: Generic SDC for all Intra modes in 3D-HEVC,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-F0126, 6th Meeting: Geneva, CH, 25 Oct.- 1 Nov. 2013

- 3DV-HTM version 9.0r1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-9.0r1/

- D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-F1100, 6th Meeting: Geneva, CH, 25 Oct.- 1 Nov. 2013.

- H. Liu, and Y. Chen “CE5: Generic SDC for all Intra modes in 3D-HEVC,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-G0122, 7th Meeting: San Jose, US, 11–17 Jan. 2014.

# Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).