| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0124 |
| --- | --- |

| Title: | CE5 related: Delta DC coding for SDC and DMM modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Qin Yu1 Hongbin Liu2 Ying Chen2 Li Zhang2 Siwei Ma1 1School of EECS, Peking University, Institute of Digital Media, Beijing, China 25775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Peking University and Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In 3D-HEVC, when Segment-wise DC Coding (SDC) is enabled (for either Intra coded or Inter coded blocks), or Depth Modeling Mode (DMM) is enabled, residual is coded with a single value, namely delta DC. A delta DC may need to be coded with a high number of context coded bins (in theory up to 13). However, in HEVC, maximum number of context coded bins for syntax elements is 9.That to say, the residual coding for the above mentioned modes may become the bottle-neck of the current entropy coding process. To improve the throughput of entropy coding to a level similar to HEVC, the number of context coded bins should be reduced. Four methods are proposed to reduce the number of context coded bins of delta DC without coding performance drop. Experimental results show that -0.01%, -0.03%, -0.02%, and 0.23% BD rate differences are observed respectively for the four proposed methods.

# Introduction

To preserve the depth’s characteristics, new modes including Segment-wise DC Coding (SDC) and Depth Modeling Mode (DMM) are introduced into intra mode coding. Meanwhile, SDC is also extended to inter mode coding (inter SDC).

For these modes, one or more (up to 4) DC residual values (delta DC) need to be coded and transmitted to the decoder. The syntax elements used for representing DC residual are shown in Table 1 and Table 2 for SDC/DMM and inter SDC respectively.

In SDC/DMM coding, the absolute value of the DC residual is signaled as depth_dc_abs which is in the range [0, 255]. In inter SDC coding, the absolute value (minus 1) of the DC residual is signalled as inter_sdc_resi_abs_minus1 which is in the range [0, 254].

Table 1: Syntax elements of DC residual for SDC/DMM

| if( DmmFlag[ x0 ][ y0 ] ¦ ¦ SdcFlag[ x0 ][ y0 ] ) { |  |
| --- | --- |
| dcNumSeg = ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_SDC_PLANAR ) ? 1 : 2 |  |
| depth_dc_flag[ x0 ][ y0 ] | ae(v) |
| if ( depth_dc_flag[ x0 ][ y0 ] ) |  |
| for( i = 0; i < dcNumSeg; i ++ ) { |  |
| depth_dc_abs[ x0 ][ y0 ][ i ] | ae(v) |
| if ( depth_dc_abs[ x0 ][ y0 ][ i ]) |  |
| depth_dc_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |

Table 2: Syntax elements of DC residual for inter SDC

| if( vps_inter_sdc_flag && PredMode[ x0 ][ y0 ] ! = MODE_INTRA && !skip_flag[ x0 ][ y0 ] ) |  |
| --- | --- |
| inter_sdc_flag | ae(v) |
| if( inter_sdc_flag ) { |  |
| puNum = ( PartMode = = PART_2Nx2N ) ? 1 : ( PartMode = = PART_NxN ? 4 : 2 ) |  |
| for( i = 0; i < puNum; i++ ) { |  |
| inter_sdc_resi_abs_minus1[ x0 ][ y0 ][ i ] | ae(v) |
| inter_sdc_resi_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |

## Number of bins coded with context in 3D-HEVC

Table 3 shows the binarization of syntax elements depth_dc_abs and inter_sdc_resi_abs_minus1. Binarized depth_dc_abs or inter_sdc_resi_abs_minus1 is a concatenation of a prefix bin string (truncated unary code) and (when present) a suffix bin string (0-th order Exp-Golomb code, i.e., EG0 code). The prefix bin string is coded with context model and the suffix bin string is bypass coded. From Table 3, we can see that total number of bins to be coded can be up to 28 (e.g., when value is equal to 255) for the worst case, and there are at most 13 bins (e.g., when value is equal to or larger than 12) that are coded with context model and the remaining ones are bypass coded.

Therefore, the delta DC coding will account for a high percentage of the overall context coded bins in this case.

Table 3: binarization of depth_dc_abs/inter_sdc_resi_abs_minus1

| Value | Prefix bin string (Truncated Unary code) |  |  |  |  |  |  |  |  |  |  |  |  | suffix bin string (EG0 code) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | 1 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |  |
| 3 | 1 | 1 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |  |
| 13 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |
| 14 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 100 |
| 15 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 101 |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 255 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 111111101110011 |

## Number of bins coded with context in HEVC version 1

In subclause 9.3.4.2.1 of HEVC version 1 specification [1], Table 9-37 shows the syntax coding of HEVC CU unit. From that table, we can see that last_sig_coeff_x_prefix and last_sig_coeff_y_prefix are the syntax elements which have the longest context coded bins. The number of context coded bins is 9.

However, for depth_dc_abs and inter_sdc_resi_abs_minus1, the maximum number of context coded bins is 13. In context-based adaptive binary arithmetic coding (CABAC), it is difficult to parallelize context coded bins. They are the bottle neck when doing parallel CABAC. To improve the throughput of the CABAC, the number of context coded bins should be reduced to a level similar to HEVC. Moreover, hardware is designed for the worst case, it is also important to reduce the worst case number of the context coded bins.

## Number of bins coded in HEVC version 1

By analyzing HEVC version 1 specification [1], we noticed that the syntax element abs_mvd_minus2 may have as many as 30 bins to be coded, although all of them are bypass coded.

## Relation between depth_dc_abs and inter_sdc_resi_abs_minus1

The depth_dc_abs and inter_sdc_resi_abs_minus1, although slightly different coded, are almost designed with the same CABAC coding mechanism and both are becoming bottlenecks for the high throughput of the CABAC coding.

Therefore, it makes sense to optimize them with the same method. Moreover, optimizing one syntax element and keeping the entropy coding of the other syntax element unchanged do not help reducing the worst case complexity.

- Proposal

To improve the throughput, four binarization and context coding methods are proposed for both the entropy coding of depth_dc_abs and inter_sdc_resi_abs_minus1.

## Method 1:

depth_dc_abs and inter_sdc_resi_abs_minus1 are binarized into a concatenation of a prefix bin string (truncated unary code) and (when present) a suffix bin string (0-th order Exp-Golomb code). As shown in Table 4, for value x (of depth_dc_abs or inter_sdc_resi_abs_minus1) equal to or larger than 9, the prefix bin string is consisted of nine ‘1’, and the suffix bin string is the binarized (x - 9) using 0-th order Exp-Golomb code; while for value x smaller than 9, the prefix bin string is the binarized x using unary code and the suffix bin string is not present. The prefix bins (up to 9) are coded with context model, while the remaining bins are bypass coded.

Table 4: Binarization of depth_dc_abs or inter_sdc_resi_abs_minus1

| Value | prefix bin string ( truncated unary code) |  |  |  |  |  |  |  |  | suffix bin string (EG0 code) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 0 |  |  |  |  |  |  |  |  |
| 2 | 1 | 1 | 0 |  |  |  |  |  |  |  |
| 3 | 1 | 1 | 1 | 0 |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |
| 8 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |  |
| 9 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |
| 10 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 100 |
| 11 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 101 |
| 12 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 11000 |
| … |  |  |  |  |  |  |  |  |  |  |
| 255 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 111111101110111 |

## Method 2:

depth_dc_abs and inter_sdc_resi_abs_minus1 are binarized into a concatenation of a prefix bin string (truncated unary code) and (when present) a suffix bin string (3-rd order Exp-Golomb code, i.e., EG3 code). As shown in Table 5, for value x (of depth_dc_abs or inter_sdc_resi_abs_minus1) equal to or larger than 9, the prefix bin string is consisted of nine ‘1’, and the suffix bin string is the binarized (x - 9) using 3-rd order Exp-Golomb code; while for value x smaller than 9, the prefix bin string is the binarized x using unary code and the suffix bin string is not present. The prefix bins (up to 9) are coded with context model, while the remaining bins are bypass coded.

Table 5: Binarization of depth_dc_abs or inter_sdc_resi_abs_minus1

| Value | prefix bin string ( truncated unary code) |  |  |  |  |  |  |  |  | suffix bin string (EG3 code) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 0 |  |  |  |  |  |  |  |  |
| 2 | 1 | 1 | 0 |  |  |  |  |  |  |  |
| 3 | 1 | 1 | 1 | 0 |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |
| 8 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |  |
| 9 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0000 |
| 10 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0001 |
| 11 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0010 |
| 12 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0011 |
| … |  |  |  |  |  |  |  |  |  |  |
| 255 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 111101111110 |

## Method 3:

depth_dc_abs and inter_sdc_resi_abs_minus1 are binarized into a concatenation of a prefix bin string (truncated unary code) and (when present) a suffix bin string (8-bit fixed length code). As shown in Table 6, for value x (of depth_dc_abs or inter_sdc_resi_abs_minus1) equal to or larger than 9, the prefix bin string is consisted of nine ‘1’, and the suffix bin string is the binarized (x - 9) using 8-bit fixed length code (due to the range of these two the syntax elements); while for value x smaller than 9, the prefix bin string is the binarized x using unary code and the suffix bin string is not present. The prefix bins (up to 9) are coded with context model, while the remaining bins are bypass coded.

Table 6: Binarization of depth_dc_abs or inter_sdc_resi_abs_minus1

| Value | prefix bin string (truncated unary code) |  |  |  |  |  |  |  |  | suffix bin string (8-bit fix length code) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 0 |  |  |  |  |  |  |  |  |
| 2 | 1 | 1 | 0 |  |  |  |  |  |  |  |
| 3 | 1 | 1 | 1 | 0 |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |
| 8 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |  |
| 9 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 00000000 |
| 10 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 00000001 |
| 11 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 00000010 |
| 12 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 00000011 |
| … |  |  |  |  |  |  |  |  |  |  |
| 255 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 11110110 |

## Method 4:

Delta DC value is constrained to be in the range [-30 30] to make depth_dc_abs and inter_sdc_resi_abs_minus1 no larger than 30.

depth_dc_abs and inter_sdc_resi_abs_minus1 are binarized using 0-th order Exp-Golomb code and all the bins are context coded, as shown in Table 7.

Table 7: Binarization of depth_dc_abs or inter_sdc_resi_abs_minus1

| Value | EG0 Part(coded with context model) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 |  |  |  |  |  |  |  |  |
| 1 | 1 | 0 | 0 |  |  |  |  |  |  |
| 2 | 1 | 0 | 1 |  |  |  |  |  |  |
| 3 | 1 | 1 | 0 | 0 | 0 |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |
| 28 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 |
| 29 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 |
| 30 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |

## Summary of the four methods

Table 8 provides maximum bins and maximum context coded bins for all these methods. From this table, we can see that, the number of context coded bins is no more than that of HEVC.

Table 8: Maximum number of bins and context coded bins

|  | HEVC | 3D-HEVC | Method 1 | Method 2 | Method 3 | Method 4 |
| --- | --- | --- | --- | --- | --- | --- |
| Maximum number of context coded bins | 9 | 13 | 9 | 9 | 9 | 9 |
| Maximum number of bins | 30 | 28 | 24 | 21 | 17 | 9 |
| Delta BD rate | n/a | 0.00% | -0.01% | -0.03% | -0.02% | 0.23% |

We prefer method 3 and method 2 due to the coding performance. Restricting of the range of the syntax elements, as proposed in method 4, introduces loss in CTC (can be larger in other cases, and it is considered to limit the flexibility of the design, therefore, we prefer not to introduce such restrictions at this stage.

Note that all proposed method are simulated by applying to both depth_dc_abs and inter_sdc_resi_abs_minus1, therefore, the complexity of the worst case of 3D-HEVC is really reduced.

# Experimental results

The proposed methods have been implemented into 3D-HTM v9.0r1 [2]. Simulations were conducted following common test conditions [3]. Table 9, 10, 11 and 12 show the performance of method 1, 2, 3 and 4 respectively. As shown in Table 9, 10 and 11, the proposed method 1, 2, 3 has almost no impact on the overall performance. 0.23% coding loss in aspect of synthesis view/ total bitrate has been observed for method 4.

Table 9: Coding performance of method 1 with respect to 3D-HTM anchor for 3-view case

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.02% | -0.01% | -0.01% | 0.00% | 0.01% | 104.2% | 109.8% | 104.6% |
| Kendo | 0.00% | -0.05% | -0.19% | -0.05% | -0.03% | -0.04% | 101.3% | 107.4% | 106.4% |
| Newspaper_CC | 0.00% | -0.09% | 0.02% | -0.02% | -0.04% | 0.05% | 94.0% | 104.8% | 97.1% |
| GT_Fly | 0.00% | -0.12% | -0.12% | -0.01% | -0.04% | -0.08% | 96.0% | 97.9% | 95.5% |
| Poznan_Hall2 | 0.00% | 0.12% | 0.00% | 0.03% | -0.01% | 0.03% | 91.4% | 100.5% | 94.1% |
| Poznan_Street | 0.00% | -0.11% | 0.02% | -0.01% | -0.02% | -0.01% | 92.3% | 101.6% | 94.9% |
| Undo_Dancer | 0.00% | -0.10% | 0.08% | 0.00% | -0.01% | -0.07% | 107.7% | 115.2% | 107.9% |
| Shark | 0.00% | 0.03% | -0.15% | -0.01% | -0.02% | 0.02% | 122.5% | 120.0% | 115.1% |
| 1024x768 | 0.00% | -0.05% | -0.06% | -0.03% | -0.02% | 0.01% | 99.9% | 107.3% | 102.7% |
| 1920x1088 | 0.00% | -0.04% | -0.03% | 0.00% | -0.02% | -0.02% | 102.0% | 107.1% | 101.5% |
| average | 0.00% | -0.04% | -0.04% | -0.01% | -0.02% | -0.01% | 101.2% | 107.2% | 102.0% |

Table 10: Coding performance of method 2 with respect to 3D-HTM anchor for 3-view case

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.02% | 0.06% | 0.00% | -0.02% | 0.03% | 106.7% | 109.5% | 108.1% |
| Kendo | 0.00% | -0.03% | -0.03% | -0.02% | -0.01% | -0.04% | 95.1% | 96.4% | 97.5% |
| Newspaper_CC | 0.00% | -0.19% | 0.03% | -0.03% | -0.05% | -0.04% | 100.9% | 105.3% | 101.5% |
| GT_Fly | 0.00% | -0.07% | -0.07% | -0.02% | -0.04% | -0.03% | 100.2% | 99.7% | 94.8% |
| Poznan_Hall2 | 0.00% | 0.06% | 0.00% | 0.02% | -0.01% | 0.03% | 97.4% | 100.0% | 96.7% |
| Poznan_Street | 0.00% | -0.01% | -0.05% | 0.00% | -0.02% | -0.05% | 90.9% | 99.9% | 92.2% |
| Undo_Dancer | 0.00% | -0.11% | 0.13% | 0.00% | -0.01% | -0.05% | 106.5% | 106.0% | 100.8% |
| Shark | 0.00% | -0.03% | -0.08% | -0.01% | -0.06% | -0.09% | 116.4% | 113.5% | 108.9% |
| 1024x768 | 0.00% | -0.08% | 0.02% | -0.02% | -0.03% | -0.02% | 100.9% | 103.7% | 102.4% |
| 1920x1088 | 0.00% | -0.03% | -0.01% | 0.00% | -0.03% | -0.04% | 102.3% | 103.8% | 98.7% |
| average | 0.00% | -0.05% | 0.00% | -0.01% | -0.03% | -0.03% | 101.8% | 103.8% | 100.1% |

Table 11: Coding performance of method 3 with respect to 3D-HTM anchor for 3-view case

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.02% | -0.08% | -0.02% | 0.01% | -0.01% | 107.0% | 110.7% | 109.7% |
| Kendo | 0.00% | -0.10% | -0.12% | -0.05% | -0.03% | -0.06% | 99.2% | 98.7% | 99.3% |
| Newspaper_CC | 0.00% | -0.02% | 0.04% | 0.00% | -0.04% | 0.05% | 99.5% | 101.6% | 100.4% |
| GT_Fly | 0.00% | 0.03% | -0.04% | 0.00% | 0.01% | -0.06% | 101.2% | 100.9% | 96.4% |
| Poznan_Hall2 | 0.00% | -0.11% | 0.13% | 0.02% | -0.02% | -0.03% | 96.1% | 100.0% | 95.5% |
| Poznan_Street | 0.00% | -0.12% | -0.13% | -0.03% | -0.02% | -0.04% | 96.3% | 96.8% | 96.6% |
| Undo_Dancer | 0.00% | -0.11% | 0.08% | -0.01% | -0.02% | -0.03% | 110.4% | 111.4% | 107.1% |
| Shark | 0.00% | 0.06% | -0.10% | 0.00% | -0.04% | 0.00% | 118.8% | 111.9% | 111.4% |
| 1024x768 | 0.00% | -0.04% | -0.05% | -0.02% | -0.02% | -0.01% | 101.9% | 103.6% | 103.1% |
| 1920x1088 | 0.00% | -0.05% | -0.01% | 0.00% | -0.02% | -0.03% | 104.6% | 104.2% | 101.4% |
| average | 0.00% | -0.05% | -0.03% | -0.01% | -0.02% | -0.02% | 103.6% | 104.0% | 102.1% |

Table 12: Coding performance of method 4 with respect to 3D-HTM anchor for 3-view case

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.09% | -0.06% | -0.01% | 0.00% | -0.01% | 108.9% | 112.4% | 107.0% |
| Kendo | 0.00% | -0.30% | -0.07% | -0.07% | 0.00% | 0.01% | 98.8% | 97.5% | 97.3% |
| Newspaper_CC | 0.00% | -0.10% | 0.00% | -0.02% | -0.04% | -0.01% | 102.3% | 101.5% | 99.0% |
| GT_Fly | 0.00% | 0.30% | -0.06% | 0.03% | 0.12% | 0.40% | 106.6% | 110.3% | 104.4% |
| Poznan_Hall2 | 0.00% | 0.15% | -0.16% | -0.01% | -0.05% | 0.06% | 97.6% | 98.2% | 93.6% |
| Poznan_Street | 0.00% | -0.07% | -0.08% | -0.03% | -0.02% | 0.25% | 97.1% | 103.6% | 98.4% |
| Undo_Dancer | 0.00% | 0.04% | 0.13% | 0.01% | 0.03% | 0.22% | 103.3% | 111.4% | 102.2% |
| Shark | 0.00% | 0.33% | 0.26% | 0.06% | 0.12% | 0.94% | 118.2% | 113.6% | 113.2% |
| 1024x768 | 0.00% | -0.10% | -0.04% | -0.03% | -0.01% | 0.00% | 103.3% | 103.8% | 101.1% |
| 1920x1088 | 0.00% | 0.15% | 0.02% | 0.01% | 0.04% | 0.38% | 104.6% | 107.4% | 102.4% |
| average | 0.00% | 0.05% | 0.00% | 0.00% | 0.02% | 0.23% | 104.1% | 106.1% | 101.9% |

# References

- B. Bross, W. Han, J. Ohm, G. Sullivan, Y. Wang and T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 10,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCTVC-L1003, 12th Meeting: Geneva, CH, 14–23 Jan. 2013.

- 3DV-HTM version 9.0r1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-9.0r1/

- D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-F1100, 6th Meeting: Geneva, CH, 25 Oct.- 1 Nov. 2013.

# Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).