| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0126 |
| --- | --- |

| Title: | CE2 related: Disparity derivation for depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Chen Li Zhang Hongbin Liu 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In 3D-HEVC, inter-view motion prediction for depth blocks is enabled. To support that, currently a disparity vector is generated by accessing neighboring reconstructed depth pixels of the current coding unit. Since the current method was considered to potentially introduce some delay, this proposal provides a simple method to fix this problem. A co-located depth block in the reference depth view is accessed and the maximum value of the four corner pixels is used to generate the disparity vector, therefore there is no need to access the spatial neighboring pixels of the current block. Simulation results show that the proposed method doesn’t provide any loss (-0.01% coding gain instead).

# Introduction

In the 6th JCT-3V meeting, inter-view motion vector prediction for depth coding [1] is adopted.

To support that, a single disparity vector is derived for each CU from a single depth value. The depth value is calculated from the three neighboring samples that are adjacent to the corners of the current CU. As shown in Figure 1, three neighboring sample positions namely above-left, above-right and bottom-left, as identified by red blocks in Figure 1, are used.

A single depth value from the three neighboring samples is calculated as:

Depth = ( 5*D[ xC-1 ][ yC-1 ] + 5*D[ xC-1 ][ yC + 2N-1 ] + 6*D[ xC + 2N-1 ][ yC-1 ] + 8 )>>4

Here D[x][y] represents the reconstructed depth value at location (x, y) and (xC, yC) represents the top-left corner of the current CU with a size of 2Nx2N.

Figure 1: Derivation of a disparity vector from the three (red) neighboring reconstructed samples.

Description of the CE encourages investigation of the current disparity vector derivation method for depth coding.

# Proposal

It is proposed that the depth value, to be used to convert to a disparity vector (DV), is derived from the reference depth view, by accessing only the four corner pixels of the block co-located with the current CU.

After the four corner pixels are identified, the maximum is chosen, similar as in BVSP, to be converted to DV.

The reference depth view is the first reference view as indicated in the slice level.

# Experimental results

Two different platforms are used to verify the coding performance and simulations are performed based on the common test conditions (CTC) [2].

The first test is based on HTMv9.0r1 [2] and corresponding results are show in Table 1. As it can be seen, there is a minor coding gain of 0.01% achieved with the proposed method.

Table 1: Results of the proposed method on HTMv9.0r1

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.02% | -0.04% | 103.9% | 106.7% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.03% | 0.00% | 97.0% | 96.7% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | -0.02% | 0.02% | 93.4% | 95.9% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | -0.02% | -0.04% | 106.0% | 101.7% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | -0.01% | 0.02% | 87.7% | 87.7% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.00% | -0.02% | 89.2% | 93.4% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | -0.02% | -0.02% | 104.9% | 112.2% |
| Shark | 0.0% | 0.0% | 0.0% | 0.0% | -0.04% | -0.04% | 112.5% | 110.3% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.01% | -0.01% | 98.1% | 99.8% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | -0.02% | -0.02% | 100.0% | 101.1% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | -0.01% | -0.01% | 99.3% | 100.6% |

The second test is based on the software of the CE proposal from MediaTek [4]. However in this test, we replace the NBDV process with just a zero DV. The zero DV is fed into the DoNBDV process and in theory should perform the same as the proposed method, with results shown in Table 1.

The corresponding results are show in Table 2: Results of the proposed method. As it can be seen, the coding gain (i.e., 0.02%) is slightly different from what is shown in Table 1.

The difference of the results is due to the reason that CE proposal [4] not only reuses the texture DoNBDV, but also makes other modifications, related to shifted inter-view merging candidate. More specifically, the following cases are treated different (with W and H denoting the width and height of current prediction unit) than the counterpart modules in the texture coding in the current 3D-HEVC:

- If the inter-view merging candidate is unavailable, (DV[ 0 ] + W/2, DV[ 1 ]+H/2) is used to identify a reference block to derive the shifted inter-view merging candidate;

- Otherwise (the inter-view merging candidate is available), if the inter-view merging candidate is uni-predictive, ( ((DV[ 0 ]+2)>>2) + W/2, ((DV[ 1 ] + 2) >> 2) +H/2 ) is used to identify a reference block to derive the shifted inter-view merging candidate;

- Otherwise (the inter-view merging candidate is available), if the inter-view merging candidate is bi-predictive, ( ((((DV[ 0 ]+2)>>2) + 2)>>2) + W/2,  ((((DV[ 1 ] + 2) >> 2) + 2) >> 2) +H/2 ) is used to identify a reference block to derive the shifted inter-view merging candidate;

If above modifications do not apply, the CE proposal with replacing NBDV results with zero DV should provide the same results as in this proposal.

Table 2: Results of the proposed method on CE software

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.00% | -0.05% | 98.3% | 98.2% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.02% | -0.03% | 106.0% | 106.2% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | -0.01% | 0.01% | 89.3% | 93.4% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | -0.02% | -0.03% | 97.8% | 100.4% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | -0.02% | 0.03% | 85.8% | 92.4% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.00% | -0.02% | 93.6% | 98.2% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | -0.01% | -0.01% | 98.4% | 102.3% |
| Shark | 0.0% | 0.0% | 0.0% | 0.0% | -0.03% | -0.04% | 107.7% | 109.5% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.00% | -0.02% | 97.9% | 99.3% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | -0.02% | -0.01% | 96.7% | 100.5% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | -0.01% | -0.02% | 97.1% | 100.1% |

# References

- X. Zhao, L. Zhang, Y. Chen, M. Karczewicz, “CE3: Inter-view motion vector prediction for depth coding”, JCT3V-F0125, Geneva, Switzerland, 25 Oct. – 01 Nov. 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments”, JCT3V-F1100, Geneva, Switzerland, 25 Oct. – 01 Nov. 2013.

- 3D-HTM 9.0r1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-9.0r1/

- Y.-L. Chang, Y.-W. Chen, J.-L. Lin, Y.-P. Tsai, S. Lei, “3D-CE2: results on interview motion prediction by DoNBDV for depth coding”, JCT3V-G0046, San José,  US, 11–17 Jan. 2014.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).