| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0128 |
| --- | --- |

| Title: | CE3 related: Motion storage optimization in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Chen 5775 Morehouse Drive San Diego, CA 92121 USA Qin Yu Siwei Ma School of EECS, Peking University, Institute of Digital Media, Beijing, China | Tel: Email: | 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated and Peking University |  |  |

_____________________________

# Abstract

In current 3D-HEVC, motion compression is achieved by two steps: compression to 8x8 level after each picture is decoded, and compression to 16x16 level after the whole access unit is decoded. The design was mainly based on earlier versions of HEVC version 1, where inside picture (access unit) motion compression to mainly reduce the line buffer applies to the 8x8 level. However, since the final HEVC utilizes a different approach to make an 8x8 size CU contain at most two motion vectors with so-called bi-prediction restriction, such a compression is not needed any more. In this document, it is proposed that the 3D-HEVC design should be aligned with the HEVC design for inside access unit motion storage optimization and the current 8x8 motion compression should be removed.

# Introduction

## HEVC motion storage optimization

In HEVC, there was intent to optimize the motion storage to mainly reduce the line buffer.

In earlier versions of HEVC WD design, the motion vectors can be maintained in a 4x4 level, meaning each 4x4 block within an 8x8 CU may contain two motion vectors (corresponding to RefPicList0 and RefPicList1 respectively) that are different from those of the other blocks. Later motion vectors are maintained in a 4x8 or 8x4 level. That means an 8x8 CU can contain up to four motion vectors. So motion compression has been proposed so that an 8x8 CU can contain only two motion vectors, by discarding motion vectors of part of the region of the 8x8 CU. Such a process is called 8x8 motion compression in this document.

However, in the final HEVC (version 1) standard, the motion compression is not needed anymore, due to the normative bi-prediction restriction for 4x8 or 8x4 PUs of the 8x8 CU. Such a bi-prediction restriction disallow a 4x8 or 8x4 PU to have the motion vector corresponding to RefPicList1 (denoted as mv1 for simplicity in this document) signaled (especially in AMVP mode) and trim out the mv1 for a merge candidate if the current PU has a size of 4x8 or 8x4.

## 3D-HEVC motion storage optimization

In 3D-HEVC, 8x8 motion compression was adopted into earlier versions of 3D-HEVC, for which the software doesn’t contain the bi-prediction restriction.

Therefore the current 3D-HEVC contains two motion storage optimizations for pictures within the same access unit.

- 8x8 motion compression.

- Bi-prediction restriction.

Note that only for the BVSP merge candidate the bi-prediction restriction doesn’t apply for historic reasons: the BVSP was based on 4x4 block sizes and when the smallest size was constrained to 4x8 or 8x4, the BVSP merge candidate was always uni-directional predicted in CTC.

Therefore, there are two motion storage optimization processes in 3D-HEVC for the same purpose, one is aligned with the latest HEVC version 1 design (Bi-prediction restriction) and the other is aligned with earlier HEVC design (8x8 motion compression) which was not in HEVC version 1 anymore.

- Proposal

In this proposal, we propose to remove the redundant inside AU motion storage optimization scheme: the 8x8 motion compression and use bi-prediction restriction to achieve the same purpose. Note that bi-prediction restriction is already in the current design (WD text as well as software).

The motion compression for pictures in DPB to a level of 16x16 is kept unchanged after the whole AU is coded.

Furthermore, in the HTM software, bi-prediction restriction is also extended to BVSP candidate, which has no impact in the case of CTC. Therefore, the bi-prediction restriction applies to all candidates in a merge list without conditionally checking the characteristics of each of them.

# Experimental results

The proposed method was implemented on HTM-9.0r1 [1]. The results of this proposal are compared with those collected under CTC [2]. Experimental results show that this method has no performance degradation, even some small gain, as shown in Table 1.

Table 1: Removal of 8x8 motion compression in HTM9.0r1 compared with CTC

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.2% | -0.3% | -0.1% | 0.0% | -0.09% | 108.6% | 109.5% | 107.5% |
| Kendo | 0.00% | -0.2% | -0.3% | -0.1% | -0.1% | -0.09% | 92.5% | 90.1% | 93.5% |
| Newspaper_CC | 0.00% | -0.2% | 0.0% | 0.0% | 0.0% | -0.05% | 98.5% | 102.3% | 102.7% |
| GT_Fly | 0.00% | -0.4% | -0.2% | 0.0% | -0.1% | -0.10% | 90.0% | 85.4% | 87.8% |
| Poznan_Hall2 | 0.00% | -0.1% | 0.1% | 0.0% | 0.0% | 0.02% | 88.0% | 93.6% | 88.5% |
| Poznan_Street | 0.00% | -0.3% | -0.2% | -0.1% | 0.0% | -0.07% | 93.2% | 94.0% | 93.6% |
| Undo_Dancer | 0.00% | -0.3% | -0.2% | -0.1% | -0.1% | -0.09% | 94.9% | 93.0% | 92.6% |
| Shark | 0.00% | -0.1% | -0.1% | 0.0% | 0.0% | -0.03% | 112.3% | 103.6% | 107.4% |
| 1024x768 | 0.00% | -0.2% | -0.2% | -0.1% | 0.0% | -0.07% | 99.9% | 100.6% | 101.2% |
| 1920x1088 | 0.00% | -0.2% | -0.1% | 0.0% | 0.0% | -0.05% | 95.7% | 93.9% | 94.0% |
| average | 0.00% | -0.2% | -0.2% | 0.0% | 0.0% | -0.06% | 97.3% | 96.4% | 96.7% |

# References

- 3DV-HTM version 9.0r1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-9.0r1/

- D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-F1100, 6th Meeting: Geneva, CH, 25 Oct.- 1 Nov. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).