| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0129 |
| --- | --- |

| Title: | AHG5: Comments on default NBDV view selection and ARP target reference picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Chen Li Zhang 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document contributes 1) a clean-up for the derivation of default reference view order index; 2) a bug fix for the derivation process of target reference index for ARP.

# Introduction

## Deviation of the default reference index for NBDV

In 3D-HEVC, NBDV might return a default disparity vector (zero), associated with a default view order index when the checked neighboring blocks do not contain a disparity motion vector. The current design needs to check both reference picture lists and compare the view order index one by one to derive a default view order index.

## Deviation of the target reference index for ARP

In 3D-HEVC, a target reference index RpRefIdxLX is chosen for each picture that can use ARP.

Firstly a temporal reference picture is identified corresponding to each reference picture list (RefPicList0 or RefPicList1).

Secondly, with such a temporal reference picture, the reference views in the reference picture list of the active inter-layer reference picture (as defined in the slice header) are inspected one by one. If the active inter-layer reference picture with view order index equal to refViewIdx  has a temporal reference picture (still present in DPB) with the same POC as the RpRefIdxLX of the current picture, the variable RefRpRefAvailFlagLX[ refViewIdx ] is set equal to 1. Otherwise, RefRpRefAvailFlagLX[ refViewIdx ] is set equal to 0. If the NBDV process returns a view order index (denoted by i) which corresponds to RefRpRefAvailFlagLX[ i ] equal to 0, ARP should not apply.

The current 3D-HEVC design in subclause H.8.3.9 (Derivation process for the target reference index for residual prediction) is as follows.

This process is invoked when the current slice is a P or B slice.

The variables RpRefIdxL0 and RpRefIdxL1 are set equal to –1, the variables RpRefPicAvailFlagL0 and RpRefPicAvailFlagL1 are set equal to 0.

The following applies for X in the range of 0 to 1, inclusive:

- When X is equal to 0 or the current slice is a B slice the following applies:

  - For i in the range of 0 to num_ref_idx_lX_active_minus1, inclusive, the following applies:

    - When PicOrderCnt( RefPicListX[ i ] ) is not equal to PicOrderCntVal and RpRefPicAvailFlagLX is equal to 0, the following applies:

        - RpRefIdxLX = i		(Error: Reference source not found-1)

        - RpRefPicAvailFlagLX = 1		(Error: Reference source not found-2)

      - [Ed. (GT): There might be pictures present in the DPB fulfilling the above conditions, but having e.g. a different value of DepthFlag compared to the current layer.].

- The variable RpRefPicAvailFlag is set equal to ( RpRefPicAvailFlagL0  | |  RpRefPicAvailFlagL1 ).

- When RpRefPicAvailFlag is equal to 1, the following applies for X in the range of 0 to 1, inclusive:

- When X is equal to 0 or the current slice is a B slice the following applies:

  - For i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, the following applies:

    - The variable refViewIdx is set equal to ViewIdx( RefPicListX[ i ] ).

    - The variable RefRpRefAvailFlagLX[ refViewIdx] is set equal to 0.

    - When RpRefPicAvailFlagLX is equal to 1 and there is a picture picA in the DPB with PicOrderCnt( picA ) equal to PicOrderCnt( RefPicListX[ RpRefIdxLX ] ), ViewIdx( picA ) equal to refViewIdx, DepthFlag( picA ) equal to 0 and marked as “used for reference”, RefRpRefAvailFlagLX[ refViewIdx ] is set equal to 1.

In this subclause, there is a clear typo in this bullet “–	The variable refViewIdx is set equal to ViewIdx( RefPicListX[ i ] ).”, since the i-th active inter-layer reference picture, rather than the i-th reference picture.

## Commonality of these two processes

Both of these two processes are mainly concerned on accessing the inter-layer reference picture which might be used for the coding of the current slice. Note that although one inter-layer reference picture is signaled in VPS, it might not be present for the current slice.

# Proposal

It is proposed that the derivation of default reference view order index should be simplified in a way similar as in the derivation process of the target reference picture in ARP. In addition, bug fixes are provided for the derivation process of the target reference picture in ARP.

The modified text is highlighted in green and deleted text is in red strikethrough.

- H.8.3.8	Derivation process for the default reference view order index for disparity derivation

This process is invoked when the current slice is a P or B slice.

The variable DefaultViewIdx is set equal to ViewOrderIdx[ RefPicLayerId[ 0 ] ].–1, and the following applies for curViewIdx in the range of 0 to ( ViewIdx – 1 ), inclusive:

- The following applies for X in the range of 0 to 1, inclusive:

  - When X is equal to 0 or the current slice is a B slice, the following applies for i in the range of 0 to NumRefPicsLX, inclusive:

    - When all of the following conditions are true, DefaultViewIdx is set equal to curViewIdx.

      - DefaultViewIdx is equal to –1.

      - ViewIdx( RefPicListX[ i ] ) is equal to curViewIdx.

      - PicOrderCnt( RefPicListX[ i ] ) is equal to PicOrderCntVal.

- H.8.3.9	Derivation process for the target reference index for residual prediction

This process is invoked when the current slice is a P or B slice.

The variables RpRefIdxL0 and RpRefIdxL1 are set equal to –1, the variables RpRefPicAvailFlagL0 and RpRefPicAvailFlagL1 are set equal to 0.

The following applies for X in the range of 0 to 1, inclusive:

- When X is equal to 0 or the current slice is a B slice the following applies:

  - For i in the range of 0 to num_ref_idx_lX_active_minus1, inclusive, the following applies:

    - When PicOrderCnt( RefPicListX[ i ] ) is not equal to PicOrderCntVal and RpRefPicAvailFlagLX is equal to 0, the following applies:

        - RpRefIdxLX = i		(Error: Reference source not found-3)

        - RpRefPicAvailFlagLX = 1		(Error: Reference source not found-4)

      - [Ed. (GT): There might be pictures present in the DPB fulfilling the above conditions, but having e.g. a different value of DepthFlag compared to the current layer.].[Ed. (CY): this is addressed below by “DepthFlag( picA ) equal to 0”.]

- The variable RpRefPicAvailFlag is set equal to ( RpRefPicAvailFlagL0  | |  RpRefPicAvailFlagL1 ).

- When RpRefPicAvailFlag is equal to 1, the following applies for X in the range of 0 to 1, inclusive:

- When X is equal to 0 or the current slice is a B slice the following applies:

  - For i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, the following applies:

    - The variable refViewIdx is set equal to ViewOrderIdx[ RefPicLayerId[ i ] ]ViewIdx( RefPicListX[ i ] ).

    - The variable RefRpRefAvailFlagLX[ refViewIdx] is set equal to 0.

    - When RpRefPicAvailFlagLX is equal to 1 and there is a picture picA in the DPB with PicOrderCnt( picA ) equal to PicOrderCnt( RefPicListX[ RpRefIdxLX ] ), ViewIdx( picA ) equal to refViewIdx, DepthFlag( picA ) equal to 0 and marked as “used for reference”, RefRpRefAvailFlagLX[ refViewIdx ] is set equal to 1.

# Discussions

Currently it is not clear on how the actual depth reference layers are signalled in the slice level for a texture picture, and similarly how the active texture reference layer (only the associated one) is signalled in the slice header for a depth picture.

The question is whether a depth reference layer can be present as an active reference layer of the current texture picture.

It is currently assumed that in 3D-HEVC, the active layers signalled in slice header by inter_layer_pred_layer_idc[ i ] only concern the layers that with the same property of being texture or depth. From the design principle point of view, such active layers are more for indicating the inter-layer reference pictures that will be put into the reference picture list of the current picture. Therefore, we prefer to keep the current assumption.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).