| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0130 |
| --- | --- |

| Title: | CE5 related: Unification of delta DC coding and signaling for inter SDC and intra SDC in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu Ying Chen 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 86-10-57760696 hongbinl@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In 3D-HEVC, segment-wise DC coding (SDC) is enabled for both Intra and Inter blocks, however, the signaling and delta DC coding methods are designed differently (although quite similar) for these two cases. In order to simplify the 3D-HEVC design, this contribution proposes to unify delta DC coding and the signaling for inter SDC and intra SDC. The delta DC coding method of intra SDC is also extended to be applied to inter SDC in the same way. Meanwhile, instead of implicitly signalling the intra SDC flag in the depth intra mode, the intra SDC flag is explicitly signalled in the same way as in inter SDC. It is reported that there is no coding loss on synthesized view in both CTC case and all intra case.

# Introduction

In 3D-HEVC, segment-wise delta DC coding (SDC) is applied to both intra prediction mode and inter prediction mode in depth coding. However, delta DC coding in intra SDC and inter SDC is designed differently as shown in Table 1 and Table 2.

In inter SDC, syntax element inter_sdc_resi_abs_minus1 which represents absolute value of delta DC minis 1 is coded; while in intra SDC, syntax element depth_dc_abs which represents absolute value of delta DC is coded.

Meanwhile, SDC flag is also designed differently for intra SDC and inter SDC. In inter SDC, a SDC flag inter_sdc_flag is signaled explicitly to indicate whether SDC coding is used. However, in intra SDC, SDC flag is implicitly included in depth_intra_mode.

Table 1: Coding unit syntax

| … |  |
| --- | --- |
| if( vps_inter_sdc_flag && PredMode[ x0 ][ y0 ] ! = MODE_INTRA && !skip_flag[ x0 ][ y0 ] ) |  |
| inter_sdc_flag | ae(v) |
| if( inter_sdc_flag ) { |  |
| puNum = ( PartMode = = PART_2Nx2N ) ? 1 : ( PartMode = = PART_NxN ? 4 : 2 ) |  |
| for( i = 0; i < puNum; i++ ) { |  |
| inter_sdc_resi_abs_minus1[ x0 ][ y0 ][ i ] | ae(v) |
| inter_sdc_resi_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| … |  |

Table 2: Depth mode parameter syntax

| depth_mode_parameters( x0 , y0 , log2CbSize ) { | Descriptor |
| --- | --- |
| depth_intra_mode[ x0 ][ y0 ] | ae(v) |
| if ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_DMM_WFULL ¦ ¦ DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_SDC_DMM_WFULL ) |  |
| wedge_full_tab_idx[ x0 ][ y0 ] | ae(v) |
| if( DmmFlag[ x0 ][ y0 ] ¦ ¦ SdcFlag[ x0 ][ y0 ] ) { |  |
| dcNumSeg = ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_SDC_PLANAR ) ? 1 : 2 |  |
| depth_dc_flag[ x0 ][ y0 ] | ae(v) |
| if ( depth_dc_flag[ x0 ][ y0 ] ) |  |
| for( i = 0; i < dcNumSeg; i ++ ) { |  |
| depth_dc_abs[ x0 ][ y0 ][ i ] | ae(v) |
| if ( depth_dc_abs[ x0 ][ y0 ][ i ]) |  |
| depth_dc_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |

In the 6th JCT-3V meeting, we proposed to generalize SDC for all intra modes in JCT3V-F0126 [1] , in which SDC flag is also explicitly signaled for intra SDC. As a follow-up, we propose JCT3V-G0122 [2] to report results of JCT3V-F0126 on top of latest HTM-9.0r1[3].

# Proposed method

## Description

As a clean-up, this contribution proposes to unify the delta DC coding and signaling for intra SDC and inter SDC, and it is designed on top of JCT3V-G0122.

First, delta DC coding method in intra SDC is applied to inter SDC.

Second, instead of implicitly signalling the intra SDC flag in the depth intra mode, the intra SDC flag is explicitly signalled in the same way as signalling the inter SDC flag. In this way, one SDC flag is explicitly signaled for both intra prediction mode and inter prediction mode.

## Signalling

The major syntax change is in the depth_mode_parameters( ) syntax table and the coding_unit() syntax table, other small syntax changes, as well as detailed semantics and decoding processes modifications are provided in the attached document with working draft text for this proposal. The working draft is on top of working draft of JCT3V-G0122.

New added parts of this proposal are highlighted in yellow and deleted parts of it are marked with strikethrough.

### Syntax and semantics

H.7.3.8.5 Coding unit syntax

| coding_unit( x0, y0, log2CbSize , ctDepth) { |  |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| … |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY ) && !predPartModeFlag ) |  |
| part_mode | ae(v) |
| if( SdcEnableFlag ) |  |
| sdc_flag[ x0 ][ y0 ] | ae(v) |
| … |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) { |  |
| if( vps_depth_modes_flag[ nuh_layer_id ] ) { |  |
| if ( log2CbSize < 6 ) |  |
| hevc_intra_flag[ x0 + i ][ y0 + j ] | ae(v) |
| if ( !hevc_intra_flag[ x0 + i ][ y0 + j ] && ( ( log2CbSize > 3 ) ¦¦ ( log2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N ) ) ) |  |
| depth_intra_mode[ x0 + i ][ y0 + j ] | ae(v) |
| if ( DepthIntraMode[ x0 + i ][ y0 + j ] = = INTRA_DEP_DMM_WFULL ) |  |
| wedge_full_tab_idx[ x0 + i ][ y0 + j ] | ae(v) |
| } |  |
| depth_mode_parameters( x0 + i , y0+ j , log2CbSize ) |  |
| if( hevc_intra_flag[ x0 ][ y0 ]) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] | ae(v) |
| } |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( hevc_intra_flag[ x0 ][ y0 ] ) { |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] ) |  |
| mpm_idx[ x0 + i ][ y0 + j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| } |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| … |  |
| if( vps_inter_sdc_flag && PredMode[ x0 ][ y0 ] ! = MODE_INTRA && !skip_flag[ x0 ][ y0 ] ) |  |
| inter_sdc_flag | ae(v) |
| if( inter_sdc_flag ) { |  |
| puNum = ( PartMode = = PART_2Nx2N ) ? 1 : ( PartMode = = PART_NxN ? 4 : 2 ) |  |
| for( i = 0; i < puNum; i++ ) { |  |
| inter_sdc_resi_abs_minus1[ x0 ][ y0 ][ i ] | ae(v) |
| inter_sdc_resi_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| if( sdc_flag[ x0 ][ y0 ] ¦¦ PredMode[ x0 ][ y0 ] = = MODE_INTRA ) |  |
| depth_dc_residual( x0 , y0 , log2CbSize ) |  |
| if( !pcm_flag[ x0 ][ y0 ] && !sdc_flag[ x0 ][ y0 ] ) { |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0])) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf && !inter_sdc_flag ) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

- H.7.3.8.5.1 Depth mode parameter syntax Depth DC residual value syntax

| depth_mode_parameters depth_dc_residual ( x0 , y0 , log2CbSize ) { | Descriptor |
| --- | --- |
| if ( log2CbSize < 6 ) |  |
| hevc_intra_flag[ x0 ][ y0 ] | ae(v) |
| if ( PartMode[ xC ][ yC ] = = PART_2Nx2N ) |  |
| sdc_flag[ x0 ][ y0 ] | ae(v) |
| if ( !hevc_intra_flag[ x0 ][ y0 ] && ( ( log2CbSize > 3 ) ¦¦ ( log2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N ) ) ) |  |
| depth_intra_mode[ x0 ][ y0 ] | ae(v) |
| if ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_DMM_WFULL ) |  |
| wedge_full_tab_idx[ x0 ][ y0 ] | ae(v) |
| pbOffset = ( PartMode[ x0 ][ y0 ] = = PART_NxN && PredMode[ x0 ][ y0 ] = = MODE_INTRA ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( k = 0; k < nCbS; k = k + pbOffset ) |  |
| if( DmmFlag[ x0 + k ][ y0 + j ] ¦ ¦ sdc_flagSdcFlag[ x0 ][ y0 ] ) { |  |
| dcNumSeg = ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_SDC_PLANAR ) ? 1 : 2 |  |
| dcNumSeg = PredMode[ x0 ][ y0 ] = = MODE_INTRA ? ( DmmFlag[ x0 + k ][ y0 + j ] ? 2 : 1 ) : ( ( PartMode[ x0 ][ y0 ] = = PART_2Nx2N ) ? 1 : ( PartMode[ x0 ][ y0 ] = = PART_NxN ? 4 : 2 ) ) |  |
| if ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) |  |
| depth_dc_flag[ x0 + k ][ y0 + j ] | ae(v) |
| if ( depth_dc_flag[ x0 + k ][ y0 + j ] ) |  |
| for( i = 0; i < dcNumSeg; i ++ ) { |  |
| depth_dc_abs[ x0 + k ][ y0 + j ][ i ] | ae(v) |
| if ( depth_dc_abs[ x0 + k ][ y0 + j ][ i ]) |  |
| depth_dc_sign_flag[ x0 + k ][ y0 + j ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |

- H.7.4.9.5 Coding unit semantics

- The variable SdcEnableFlag is derived as followsthe following applies:

  - SdcEnableFlag =

  - ( vps_inter_sdc_flag[ nuh_layer_id ] && PredMode[ x0 ][ y0 ] = = MODE_INTER ) ||

  - ( vps_depth_modes_flag[ nuh_layer_id ]

  - && PredMode[ x0 ][ y0 ] = = MODE_INTRA

- && PartMode[ x0 ][ y0 ] = = PART_2Nx2N )                                                                                             (H-16)

sdc_flag[ x0 ][ y0 ] equal to 1 specifies segment-wise DC coding of residual blocks is used for the current coding unit. sdc_flag[ x0 ][ y0 ] equal to 0 specifies segment-wise DC coding of residual blocks is not used for the current coding unit. When not present, sdc_flag[ x0 ][ y0 ] is inferred to be equal to 0.

…

The variable SdcFlag[ x0 ][ y0 ] is derived as specified in the following:

  - SdcFlag[ x0 ][ y0 ] =	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_SDC_PLANAR )  | |  	(Error: Reference source not found-1)
	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_SDC_DMM_WFULL )

inter_sdc_flag equal to 1 specifies simplified depth coding of residual blocks is used for the current coding unit. inter_sdc_flag equal to 0 specifies simplified depth coding of residual blocks is not used for the current coding unit. When not present, inter_sdc_flag is inferred to be equal to 0.

inter_sdc_resi_abs_minus1[ x0 ][ y0 ][ i ], inter_sdc_resi_sign_flag[ x0 ][ y0 ][ i ] are used to derive InterSdcResi[ x0 ][ y0 ][ i ] as follows:

  - InterSdcResi[ x0 ][ y0 ][ i ] = ( 1 − 2 * inter_sdc_resi_sign_flag[ x0 ][ y0 ][ i ] ) * 
		( inter_sdc_resi_abs_minus1[ x0 ][ y0 ][ i ] + 1 )	(Error: Reference source not found-2)

…

- H.7.4.9.5.1 Depth DC residual semantics

…

depth_dc_flag[ x0 ][ y0 ] equal to 1 specifies that depth_dc_abs[ x0 ][ y0 ][ i ] and depth_dc_sign_flag[ x0 ][ y0 ][ i ] are present. depth_dc_flag[ x0 ][ y0 ] equal to 0 specifies that depth_dc_abs[ x0 ][ y0 ][ i ] and  depth_dc_sign_flag[ x0 ][ y0 ][ i ] are not present. When not present, depth_dc_flag[ x0 ][ y0 ] is inferred to be equal to 1.

depth_dc_abs[ x0][ y0 ][ i ], depth_dc_sign_flag[ x0 ][ y0 ][ i ] are used to derive DcOffset[ x0 ][ y0 ][ i ] as follows:

  - DcOffset[ x0 ][ y0 ][ i ]  = 
( 1 − 2 *depth_dc_sign_flag[ x0 ][ y0 ][ i ] ) * ( depth_dc_abs[ x0 ][ y0 ][ i ]− dcNumSeg +2 )	(Error: Reference source not found-3)

# Simulation results

Proposed method is integrated into JCT3V-G0122 and compared with it under common test condition [4] and all intra case. As shown in Table 3 and Table 4, proposed method has -0.01% and 0.01% average coding performance change on synthesized view for CTC and all intra case respectively.

Table 3: Comparison of proposed method with JCT3V-G0122 (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.17% | -0.16% | -0.06% | -0.05% | -0.03% | 110.4% | 106.0% |
| Kendo | 0.00% | -0.01% | -0.03% | 0.02% | 0.13% | 0.04% | 97.1% | 97.3% |
| Newspaper_CC | 0.00% | -0.06% | -0.10% | -0.03% | -0.01% | -0.07% | 93.9% | 94.4% |
| GT_Fly | 0.00% | -0.04% | -0.02% | 0.00% | 0.00% | -0.03% | 93.1% | 92.7% |
| Poznan_Hall2 | 0.00% | -0.16% | -0.11% | -0.09% | -0.01% | -0.03% | 108.2% | 104.5% |
| Poznan_Street | 0.00% | 0.03% | 0.06% | 0.02% | 0.04% | 0.04% | 100.0% | 98.4% |
| Undo_Dancer | 0.00% | -0.08% | -0.17% | -0.02% | 0.03% | 0.00% | 94.1% | 100.0% |
| Shark | 0.00% | 0.03% | -0.03% | 0.00% | 0.04% | 0.01% | 103.4% | 105.0% |
| 1024x768 | 0.00% | -0.08% | -0.10% | -0.02% | 0.02% | -0.02% | 100.5% | 99.3% |
| 1920x1088 | 0.00% | -0.04% | -0.05% | -0.02% | 0.02% | 0.00% | 99.8% | 100.1% |
| average | 0.00% | -0.06% | -0.07% | -0.02% | 0.02% | -0.01% | 100.0% | 99.8% |

Table 4: Comparison of proposed method with JCT3V-G0122 (All intra case)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.00% | 0.00% | 0.00% | 0.01% | 0.01% | 104.9% | 105.2% |
| Kendo | 0.00% | 0.00% | 0.00% | 0.00% | 0.01% | 0.02% | 92.9% | 92.4% |
| Newspaper_CC | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | -0.01% | 100.8% | 99.6% |
| GT_Fly | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 107.8% | 105.2% |
| Poznan_Hall2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.02% | 0.04% | 107.5% | 103.9% |
| Poznan_Street | 0.00% | 0.00% | 0.00% | 0.00% | 0.01% | 0.01% | 91.1% | 90.5% |
| Undo_Dancer | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.01% | 112.1% | 109.2% |
| Shark | 0.00% | 0.00% | 0.00% | 0.00% | 0.01% | 0.00% | 112.0% | 109.6% |
| 1024x768 | 0.00% | 0.00% | 0.00% | 0.00% | 0.01% | 0.01% | 99.5% | 99.1% |
| 1920x1088 | 0.00% | 0.00% | 0.00% | 0.00% | 0.01% | 0.01% | 106.1% | 103.7% |
| average | 0.00% | 0.00% | 0.00% | 0.00% | 0.01% | 0.01% | 103.6% | 101.9% |

# References

- H. Liu, Y. Chen, L. Zhang, “CE5 related: Generic SDC for all Intra modes in 3D-HEVC”, JCT3V-F0126, Geneva, Switzerland, 25 Oct. – 01 Nov. 2013.

- H. Liu, Y. Chen, “CE5: Generic SDC for all Intra modes in 3D-HEVC”, JCT3V-G0122, San Jose, US, 11-17 Jan. 2014.

- 3DV-HTM version 9.0r1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-9.0r1/

- D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments”, JCT3V-F1100, Geneva, Switzerland, 25 Oct. – 01 Nov. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).