| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0130_text |
| --- | --- |

| Title: | Proposed text for JCT3V-G0130 based on 3D-HEVC Draft Text 2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu Ying Chen 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 86-10-57760696 hongbinl@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

All the changes are highlighted in yellow or red strikethrough.

# Syntax tables

- H.7.3.8.5	Coding unit syntax

| coding_unit( x0, y0, log2CbSize , ctDepth) { |  |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| … |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY ) && !predPartModeFlag ) |  |
| part_mode | ae(v) |
| if( SdcEnableFlag ) |  |
| sdc_flag[ x0 ][ y0 ] | ae(v) |
| … |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) { |  |
| if( vps_depth_modes_flag[ nuh_layer_id ] ) { |  |
| if ( log2CbSize < 6 ) |  |
| hevc_intra_flag[ x0 + i ][ y0 + j ] | ae(v) |
| if ( !hevc_intra_flag[ x0 + i ][ y0 + j ] && ( ( log2CbSize > 3 ) ¦¦ ( log2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N ) ) ) |  |
| depth_intra_mode[ x0 + i ][ y0 + j ] | ae(v) |
| if ( DepthIntraMode[ x0 + i ][ y0 + j ] = = INTRA_DEP_DMM_WFULL ) |  |
| wedge_full_tab_idx[ x0 + i ][ y0 + j ] | ae(v) |
| } |  |
| depth_mode_parameters( x0 + i , y0+ j , log2CbSize ) |  |
| if( hevc_intra_flag[ x0 ][ y0 ]) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] | ae(v) |
| } |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( hevc_intra_flag[ x0 ][ y0 ] ) { |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] ) |  |
| mpm_idx[ x0 + i ][ y0 + j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| } |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| … |  |
| if( vps_inter_sdc_flag && PredMode[ x0 ][ y0 ] ! = MODE_INTRA && !skip_flag[ x0 ][ y0 ] ) |  |
| inter_sdc_flag | ae(v) |
| if( inter_sdc_flag ) { |  |
| puNum = ( PartMode = = PART_2Nx2N ) ? 1 : ( PartMode = = PART_NxN ? 4 : 2 ) |  |
| for( i = 0; i < puNum; i++ ) { |  |
| inter_sdc_resi_abs_minus1[ x0 ][ y0 ][ i ] | ae(v) |
| inter_sdc_resi_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| if( sdc_flag[ x0 ][ y0 ] ¦¦ PredMode[ x0 ][ y0 ] = = MODE_INTRA ) |  |
| depth_dc_residual( x0 , y0 , log2CbSize ) |  |
| if( !pcm_flag[ x0 ][ y0 ] && !sdc_flag[ x0 ][ y0 ] ) { |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0])) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf && !inter_sdc_flag ) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

- H.7.3.8.5.1		Depth mode parameter syntax Depth DC residual value syntax

| depth_mode_parameters depth_dc_residual ( x0 , y0 , log2CbSize ) { | Descriptor |
| --- | --- |
| if ( log2CbSize < 6 ) |  |
| hevc_intra_flag[ x0 ][ y0 ] | ae(v) |
| if ( PartMode[ xC ][ yC ] = = PART_2Nx2N ) |  |
| sdc_flag[ x0 ][ y0 ] | ae(v) |
| if ( !hevc_intra_flag[ x0 ][ y0 ] && ( ( log2CbSize > 3 ) ¦¦ ( log2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N ) ) ) |  |
| depth_intra_mode[ x0 ][ y0 ] | ae(v) |
| if ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_DMM_WFULL ) |  |
| wedge_full_tab_idx[ x0 ][ y0 ] | ae(v) |
| pbOffset = ( PartMode[ x0 ][ y0 ] = = PART_NxN && PredMode[ x0 ][ y0 ] = = MODE_INTRA ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( k = 0; k < nCbS; k = k + pbOffset ) |  |
| if( DmmFlag[ x0 + k ][ y0 + j ] ¦ ¦ sdc_flagSdcFlag[ x0 ][ y0 ] ) { |  |
| dcNumSeg = ( DepthIntraMode[ x0 ][ y0 ] = = INTRA_DEP_SDC_PLANAR ) ? 1 : 2 |  |
| dcNumSeg = PredMode[ x0 ][ y0 ] = = MODE_INTRA ? ( DmmFlag[ x0 + k ][ y0 + j ] ? 2 : 1 ) : ( ( PartMode[ x0 ][ y0 ] = = PART_2Nx2N ) ? 1 : ( PartMode[ x0 ][ y0 ] = = PART_NxN ? 4 : 2 ) ) |  |
| if ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) |  |
| depth_dc_flag[ x0 + k ][ y0 + j ] | ae(v) |
| if ( depth_dc_flag[ x0 + k ][ y0 + j ] ) |  |
| for( i = 0; i < dcNumSeg; i ++ ) { |  |
| depth_dc_abs[ x0 + k ][ y0 + j ][ i ] | ae(v) |
| if ( depth_dc_abs[ x0 + k ][ y0 + j ][ i ]) |  |
| depth_dc_sign_flag[ x0 + k ][ y0 + j ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |

- H.7.4.9.5 Coding unit semantics

The variable SdcEnableFlag is derived as followsthe following applies:

- SdcEnableFlag =

- ( vps_inter_sdc_flag[ nuh_layer_id ] && PredMode[ x0 ][ y0 ] = = MODE_INTER ) ||

- ( vps_depth_modes_flag[ nuh_layer_id ]

- && PredMode[ x0 ][ y0 ] = = MODE_INTRA

&& PartMode[ x0 ][ y0 ] = = PART_2Nx2N )                                                                                             (H-16)

- sdc_flag[ x0 ][ y0 ] equal to 1 specifies segment-wise DC coding of residual blocks is used for the current coding unit. sdc_flag[ x0 ][ y0 ] equal to 0 specifies segment-wise DC coding of residual blocks is not used for the current coding unit. When not present, sdc_flag[ x0 ][ y0 ] is inferred to be equal to 0.

The variable SdcFlag[ x0 ][ y0 ] is derived as specified in the following:

- SdcFlag[ x0 ][ y0 ] =	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_SDC_PLANAR )  | |  	(Error: Reference source not found-1)
	( DepthIntraMode[ x0 ][ y0 ]  = =  INTRA_DEP_SDC_DMM_WFULL )

- inter_sdc_flag equal to 1 specifies simplified depth coding of residual blocks is used for the current coding unit. inter_sdc_flag equal to 0 specifies simplified depth coding of residual blocks is not used for the current coding unit. When not present, inter_sdc_flag is inferred to be equal to 0.

- inter_sdc_resi_abs_minus1[ x0 ][ y0 ][ i ], inter_sdc_resi_sign_flag[ x0 ][ y0 ][ i ] are used to derive InterSdcResi[ x0 ][ y0 ][ i ] as follows:

- InterSdcResi[ x0 ][ y0 ][ i ] = ( 1 − 2 * inter_sdc_resi_sign_flag[ x0 ][ y0 ][ i ] ) * 
		( inter_sdc_resi_abs_minus1[ x0 ][ y0 ][ i ] + 1 )	(Error: Reference source not found-2)

- …

hevc_intra_flag[ x0 ][ y0 ] equal to 1 specifies that intra modes with intraPredMode in the range of 0 to 34 is used for the current prediction unit. hevc_intra_flag[ x0 ][ y0 ] equal to 0 specifies that another intra mode is used for the current prediction unit. When not present, hevc_intra_flag[ x0 ][ y0 ] is inferred to be equal to 1.

depth_intra_mode[ x0 ][ y0 ] specifies the depth intra mode of the current prediction unit. depth_intra_mode[ x0 ][ y0 ] equal to 0 specifies that DepthIntraMode[ x0 ][ y0 ] is equal to INTRA_DEP_DMM_WFULL, and depth_intra_mode[ x0 ][ y0 ] equal to 1 specifies that DepthIntraMode[ x0 ][ y0 ] is equal to INTRA_DEP_DMM_CPREDTEX. When not present, if hevc_intra_flag[ x0 ][ y0 ] is equal to 0, DepthIntraMode[ x0 ][ y0 ] is inferred to be equal to INTRA_DEP_DMM_WFULL; otherwise, if hevc_intra_flag[ x0 ][ y0 ] is equal to 1, DepthIntraMode[ x0 ][ y0 ] is inferred to be equal to INTRA_DEP_NONE.

wedge_full_tab_idx[ x0 ][ y0 ] specifies the index of the wedgelet pattern in the corresponding pattern list when DepthIntraMode[ x0 ][ y0 ] is equal to INTRA_DEP_DMM_WFULL.

- H.7.4.9.5.1 Depth DC residual semantics

hevc_intra_flag[ x0 ][ y0 ] equal to 1 specifies that intra modes with intraPredMode in the range of 0 to 34 is used for the current prediction unit. hevc_intra_flag[ x0 ][ y0 ] equal to 0 specifies that another intra mode is used for the current prediction unit. When not present, hevc_intra_flag[ x0 ][ y0 ] is inferred to be equal to 1.

sdc_flag[ x0 ][ y0 ] equal to 1 specifies that the residual of each partition of the current prediction unit is only represented by sdc_residual_flag[ x0 ][ y0 ][ i ], sdc_residual_sign_flag [ x0 ][ y0 ][ i ] and sdc_residual_abs_minus1[ x0 ][ y0 ][ i ]. sdc_flag[ x0 ][ y0 ] equal to 0 specifies that more residual may be present in the transform tree. When not present, sdc_flag[ x0 ][  y0 ] is inferred to be equal to 0.

The variable SdcFlag[ x0 ][ y0 ] is set to be equal to sdc_flag[ x0 ][ y0 ].

The variable DmmFlag[ x0 ][ y0 ] is derived as specified in the following:

- DmmFlag[ x0 ][ y0 ] =	!hevc_intra_flag[ x0 ][ y0 ]                      	(Error: Reference source not found-3)

depth_intra_mode[ x0 ][ y0 ] specifies the depth intra mode of the current prediction unit. depth_intra_mode[ x0 ][ y0 ] equal to 0 specifies that DepthIntraMode[ x0 ][ y0 ] is equal to INTRA_DEP_DMM_WFULL, and depth_intra_mode[ x0 ][ y0 ] equal to 1 specifies that DepthIntraMode[ x0 ][ y0 ] is equal to INTRA_DEP_DMM_CPREDTEX. When not present, if hevc_intra_flag[ x0 ][ y0 ] is equal to 0, DepthIntraMode[ x0 ][ y0 ] is inferred to be equal to INTRA_DEP_DMM_WFULL; otherwise, if hevc_intra_flag[ x0 ][ y0 ] is equal to 1, DepthIntraMode[ x0 ][ y0 ] is inferred to be equal to INTRA_DEP_NONE.

wedge_full_tab_idx[ x0 ][ y0 ] specifies the index of the wedgelet pattern in the corresponding pattern list when DepthIntraMode[ x0 ][ y0 ] is equal to INTRA_DEP_DMM_WFULL.

depth_dc_flag[ x0 ][ y0 ] equal to 1 specifies that depth_dc_abs[ x0 ][ y0 ][ i ] and depth_dc_sign_flag[ x0 ][ y0 ][ i ] are present. depth_dc_flag[ x0 ][ y0 ] equal to 0 specifies that depth_dc_abs[ x0 ][ y0 ][ i ] and  depth_dc_sign_flag[ x0 ][ y0 ][ i ] are not present. When not present, depth_dc_flag[ x0 ][ y0 ] is inferred to be equal to 1.

depth_dc_abs[ x0][ y0 ][ i ], depth_dc_sign_flag[ x0 ][ y0 ][ i ] are used to derive DcOffset[ x0 ][ y0 ][ i ] as follows:

- DcOffset[ x0 ][ y0 ][ i ]  = 
( 1 − 2 *depth_dc_sign_flag[ x0 ][ y0 ][ i ] ) * ( depth_dc_abs[ x0 ][ y0 ][ i ]− dcNumSeg +2 )	(Error: Reference source not found-4)

# Decoding process

- H.8.5.4.1 General

…

- If inter_sdc_flag sdc_flag is equal to 0, the following applies, depending on the value of rqt_root_cbf, the following applies:

- ....

- Otherwise (inter_sdc_flagsdc_flag is equal to 1), the decoding process for simplified depth coded residual blocks as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the luma location ( xB0, yB0 ) set equal to ( 0, 0 ), the variable log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable nCbS set equal to nCbSL, and the (nCbSL)x(nCbSL) array resSamplesL as inputs, and the output is a modified version of the (nCbSL)x(nCbSL) array resSamplesL.

- …

- H.8.5.4.4 Decoding process for simplified depth coded residual blocks

...

- For x in the range of 0 to nCbS the following applies:

- For y in the range of 0 to nCbS the following applies:

  - The variable i is derived as specified in the following:

    - If x is less than xOff and y is less than yOff, i is set equal to 0.

    - Otherwise, if x greater than or equal to xOff and y is less than to yOff, i is set equal to 1.

    - Otherwise, if x less than xOff and y is greater than or equal to yOff, i is set equal to 2.

    - Otherwise, ( x is greater than or equal to xOff and y is greater than or equal to yOff), i is set equal to 3.

  - The value of resSamples[ x ][ y ] is set equal to DcOffsetInterSdcResi[ xCb ][ yCb ][ interSdcResiIdx[ i ] ]

- H.9.3.2.2 Initialization process for context variables

- The specifications in subclause 9.3.2.2 apply with the following modifications.

- All references to the process specified in subclauses7.3.8.1 through 7.3.8.11 are replaced with references to the process specified in subclauses Error: Reference source not found to Error: Reference source not found.

- Error: Reference source not found is appended to the end of Table 9-4.

- Error: Reference source not found to Error: Reference source not found are appended to the end of the subclause.

Table Error: Reference source not found-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit( ) depth_mode_parameters( ) | depth_intra_mode | Error: Reference source not found | 0..7 | 8..15 | 16..23 |
|  | wedge_full_tab_idx | Error: Reference source not found | 0 | 1 | 2 |
|  | depth_dc_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | depth_dc_abs | Error: Reference source not found | 0 | 1 | 2 |
|  | iv_res_pred_weight_idx | Error: Reference source not found |  | 0..3 | 4..7 |
|  | ic_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | inter_sdc_flag | Table Error: Reference source not found-2 | 0 | 1 | 2 |
|  | inter_sdc_resi_abs_minus1 | Table Error: Reference source not found-3 | 0 | 1 | 2 |
|  | inter_sdc_resi_sign_flag | Table Error: Reference source not found-4 | 0 | 1 | 2 |

…

Table Error: Reference source not found-2 – Values of initValue for inter_sdc_flag ctxIdx

| Initialization variable | ctxIdx of inter_sdc_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table Error: Reference source not found-3 – Values of initValue for inter_sdc_resi_abs_minus1 ctxIdx

| Initialization variable | ctxIdx of inter_sdc_resi_abs_minus1 |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table Error: Reference source not found-4 – Values of initValue for inter_sdc_resi_sign_flag ctxIdx

| Initialization variable | ctxIdx of inter_sdc_resi_sign_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

- H.9.3.3.1 General

- The specifications in subclause 9.3.3.1 apply with the following modifications.

- H.9.3.3.1 General is appended to the end of Table 9-32.

Table Error: Reference source not found-5 – Syntax elements and associated binarizations

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| coding_unit( ) | iv_res_pred_weight_idx | TR | cMax = 2, cRiceParam = 0 |
|  | ic_flag | FL | cMax = 1 |
|  | inter_sdc_flag | FL | cMax = 1 |
|  | inter_sdc_resi_abs_minus1 | UEG0 | [Ed. (GT) To be specified] |
|  | inter_sdc_resi_sign_flag | FL | cMax = 1 |
| … |  |  |  |

- H.9.3.4.2.1 General

- The specifications in subclause 9.3.4.2.1 apply with the following modifications:

- H.9.3.3.1 General is appended to the end of Table 9-37.

Table Error: Reference source not found-6 –Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >=5 |
| wedge_full_tab_idx | 0 | 0 | 0 | 0 | 0 | 0 |
| depth_dc_flag | 0 | na | na | na | na | na |
| depth_dc_abs | 0 | 0 | 0 | 0 | 0 | 0 |
| depth_dc_sign_flag | bypass | 0 | 0 | 0 | 0 | 0 |
| res_pred_flag | 0 | na | na | na | na | na |
| ic_flag | 0 | na | na | na | na | na |
| inter_sdc_flag | 0 | 0 | 0 | 0 | 0 | 0 |
| inter_sdc_resi_abs_minus1 | 0 | 0 | 0 | 0 | 0 | 0 |
| inter_sdc_resi_sign_flag | 0 | 0 | 0 | 0 | 0 | 0 |