| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0136 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 |  |  | Document: JCT3V-G0136 |
| --- | --- | --- | --- |
| Title: | MV-HEVC/SHVC HLS: Improvements of Video and Picture Parameter Sets |  |  |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Truong Cong Thang Tsuruga, Ikki-Machi, Aizu-Wakamatsu, Japan 965-8580 Jung Won Kang, Jinho Lee, Hahyun Lee, Jin Soo Choi 218, Gajeong-no, Yuseong, Daejeon, Korea | Tel: Email: | thang@u-aizu.ac.jp {jungwon, jinosoul, hanilee, jschoi}@ etri.re.kr |
| Source: | ETRI, University of Aizu |  |  |

_____________________________

# Abstract

This contribution proposes three improvements to the video and picture parameter sets. The first improvement is a semantic revision of sps_max_sub_layers_minus1. The second is providing a specific temporal_id_nesting_flag for each layer in VPS, so that an extractor/decoder may know the switching-up capability of each layer. The third is providing the information about the number of tiles in VPS, allowing a client to better know a CVS characteristics and thus to better prepare its decoding resources.

# Introduction

This contribution proposes some improvements to the video and picture parameter sets. The first improvement is a semantic revision of sps_max_sub_layers_minus1, which better matches with the information provided in VPS about the number of sub layers in each layer. The second improvement is providing a specific temporal_id_nesting_flag for each layer in VPS. This helps an extractor/decoder know exactly the switching-up capability of each layer. The third improvement is providing the information about the number of tiles in VPS, allowing a client to better know a CVS characteristics and thus to better prepare its decoding resources.

# Proposals

2.1 Semantic revision of sps_max_sub_layers_minus1

Recently, syntax element sub_layers_vps_max_minus1[ i ] is provided in VPS to describe the maximum number of sub layers in each layer. However, the semantics of sps_max_sub_layers_minus1 in SPS still refers directly to vps_max_sub_layers_minus1. It is better to refer to sub_layers_vps_max_minus1[ i ] so that, when sps_max_sub_layers_minus1 is not present, it will be considered to be equal to sub_layers_vps_max_minus1[ i ]. In this part, we propose to revise the semantics of sps_max_sub_layers_minus1 as follows.

- sps_max_sub_layers_minus1 plus 1 specifies the maximum number of temporal sub-layers that may be present in each CVS referring to the SPS. The value of sps_max_sub_layers_minus1 shall be in the range of 0 to 6, inclusive. When not present sps_max_sub_layers_minus1 is inferred to be equal to sub_layers_vps_max_minus1[ i ] of the i-th layer with nuh_layer_id equal to layer_id_in_nuh[ i ].

2.2 Adding vps_layer_temporal_id_nesting_flag in VPS

Currently, the vps_temporal_id_nesting_flag is used to indicate the possibility of switching up for all layers of a bitstream. In some cases, it is useful to let a decoder or an extractor know such possibility specifically for each layer without having to parsing the SPS’s. For this purpose, we propose to add one flag for each layer in the VPS. The syntax and semantics are as follows.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| vps_layer_temporal_id_nesting_present_flag | u(1) |
| if( vps_layer_temporal_id_nesting_present_flag ) |  |
| for( i = 0; i <= MaxLayersMinus1; i++ ) |  |
| vps_layer_temporal_id_nesting_flag[ i ] | u(1) |
| … |  |
| } |  |

- vps_layer_temporal_id_nesting_present_flag equal to 1 specifies that the syntax elements vps_layer_temporal_id_nesting_flag[ i ] are present. vps_layer_temporal_id_nesting_present_flag equal to 0 specifies that the syntax elements vps_layer_temporal_id_nesting_flag[ i ] are not present.

- vps_layer_temporal_id_nesting_flag[ i ] when sub_layers_vps_max_minus1[ i ] is greater than 0 specifies whether inter prediction is additionally restricted for pictures of the i-th layer. When sub_layers_vps_max_minus1[ i ] is equal to 0, vps_layer_temporal_id_nesting_flag[ i ] shall be equal to 1. When not present vps_layer_temporal_id_nesting_flag[ i ] is inferred to be equal to vps_temporal_id_nesting_flag.

And the semantics of sps_temporal_id_nesting_flag in SPS is revised accordingly as follows.

- sps_temporal_id_nesting_flag, when sps_max_sub_layers_minus1 is greater than 0, specifies whether inter prediction is additionally restricted for CVSs referring to the SPS. When vps_temporal_id_nesting_flag is equal to 1, sps_temporal_id_nesting_flag shall be equal to 1. When sps_max_sub_layers_minus1 is equal to 0, sps_temporal_id_nesting_flag shall be equal to 1. When not present sps_temporal_id_nesting_flag is inferred to be equal to vps_layer_temporal_id_nesting_flag[ i ] of i-th layer with nuh_layer_id equal to layer_id_in_nuh[ i ].

2.3 Tile information in VPS

It is obvious that a CVS could be very complex for some decoders to decode in real time. If the information about the number of tiles is provided to a decoder, the decoder could know if it is capable to organize its resource and then decode the CVS. That is, the information of tiles in VPS will be useful for capability negotiation. Moreover, if the pictures of a layer have a common tile setting, there is no need to repeat the described tile information in every PPS.

So, we propose to describe the common tile information (or settings) in VPS as follows. The new parts are highlighted in yellow.

| vps_vui( ){ | Descriptor |
| --- | --- |
| … |  |
| tiles_not_in_use_flag | u(1) |
| if( !tiles_not_in_use_flag ) { |  |
| for( i = 0; i <= MaxLayersMinus1; i++ ) { | u(1) |
| tiles_in_use_flag[ i ] |  |
| if( tiles_in_use_flag[ i ] ) { |  |
| default_tile_info_flag[ i ] | u(1) |
| if(default_tile_info_flag[ i ] ) { |  |
| num_tile_columns_minus1[ i ] | ue(v) |
| num_tile_rows_minus1[ i ] | ue(v) |
| loop_filter_across_tiles_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

- default_tile_info_flag[ i ] equal to 1 specifies that the pictures of the i-th layer have a default tile setting. default_tile_info_flag[ i ] equal to 0 specifies that the pictures of the i-th layer may or may not have a default tile setting.

- num_tile_columns_minus1[ i ] plus 1 specifies the number of tile columns partitioning the pictures of the i-th layer. When not present, the value of num_tile_columns_minus1[ i ] is inferred to be equal to 0.

- num_tile_rows_minus1[ i ] plus 1 specifies the number of tile rows partitioning the pictures of the i-th layer. When not present, the value of num_tile_rows_minus1[ i ] is inferred to be equal to 0.

Also, we revise the existing syntax and semantics related to tile information in PPS to have an updating mechanism of tile information when needed. The revisions are highlighted in yellow as below.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| tiles_enabled_flag | u(1) |
| if ( nuh_layer_id > 0 && tiles_enabled_flag ) |  |
| update_tiles_info_flag | u(1) |
| if(tiles_enabled_flag) { |  |
| if(nuh_layer_id == 0 ¦¦ update_tiles_info_flag) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| } |  |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height_minus1[ i ] | ue(v) |
| } |  |
| if( nuh_layer_id == 0 ¦¦ update_tiles_info_flag) |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| … |  |
| } |  |

- tiles_enabled_flag equal to 1 specifies that there is more than one tile in each picture referring to the PPS. tiles_enabled_flag equal to 0 specifies that there is only one tile in each picture referring to the PPS. When not present, the value of tiles_enabled_flag is inferred to be 0.

- update_tiles_info_flag: equal to 1 specifies that the tile information of the pictures referring to this PPS is updated. update_tiles_info_flag equal to 0 specifies that the tile information of the pictures referring to this PPS is not updated. When not present, the value of update_tiles_info_flag is inferred to be 0.

# Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

University of Aizu may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).