| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0139 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0139 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Header parameter set (HPS) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Hoda Roodaki Nokia Research Center Visiokatu 3, Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

It is asserted that in JCT-3V common test conditions, the overhead of enhancement-layer (EL) slice headers is on average about 3.4% when compared to EL bitrate only for both MV-HEVC and 3D-HEVC and about 1.0 and 1.2% (for MV-HEVC and 3D-HEVC, respectively) when compared to the total bitrate. The motivation of the contribution is to reduce the EL slice header overhead by a header parameter set (HPS) design, which enables the inheritance of slice header syntax elements from the HPS.

HPS was proposed earlier in JCTVC-J0109 for HEVC version 1. The HPS design in this contribution is asserted to be similar to that of JCTVC-J0109 with the addition that repetitive slice header patterns e.g. for an entire IRAP picture period can be included in the HPS and addressed either by slice_pic_order_cnt_lsb values or an indicated index hps_entry_idx in the slice header.

# Introduction

A header parameter set (HPS) design was presented in JCTVC-J0109 for inheritance of slice header syntax elements. JCT-VC decided to take no action on JCTVC-J0109, while one remark was that the HPS concept could be used in enhancement layers without needing to use it for the base layer.

This contribution provides statistics of the slice header bit count for MV-HEVC and 3D-HEVC common test conditions. It is concluded that slice headers take on average 3.4 % of the bitrate of the enhancement layers for both MV-HEVC and 3D-HEVC, when one slice per picture is coded. More details of the tests and results can be found in Section 3. As bit count share of the slice headers is not insignificant, this contribution proposes a header parameter set design with the main motivation to provide coding efficiency improvement for multi-slice sharing of header data.

# Overview of the proposal

The proposal is summarized as follows:

- The slice header syntax elements are categorized and represented by header parameter subset syntax structures as follows:

  - Reference picture set syntax elements, hps_rps( )

  - Inter-layer reference picture set syntax elements, hps_il_rps( )

  - Number of active reference pictures and reference picture list modification, hps_rplm( )

  - Prediction weights for weighted prediction, hps_pred_weight_table( )

  - Deblocking filter syntax elements, hps_dbf( )

  - Slice segment header extension, hps_slice_segment_header_extension( )

  - Other/miscellaneous syntax elements, hps_other_param( )

- The header parameter set RBSP contains:

  - Zero or more header parameter subset syntax structures of each type. The number of header parameter subset syntax structures can vary from one type to another. The structures of each type are indexed starting from 1.

  - Header parameter combination entries, each of which is a combination of header parameter subset indexes (one index per each type of header parameter subset syntax structures). Index 0 means that the respective header parameter subset is not included in the header parameter combination entry but rather is coded in the slice segment header.

  - Syntax elements that enable mapping of a picture to a header parameter set combination entry.

- The use of the slice header inheritance mechanism is conditioned on a PPS flag (pps_slice_header_inherit_enabled_flag) and a slice header flag (slice_header_inherit_flag).

- A picture is mapped to a header parameter set combination as follows:

  - The slice segment header contains slice_hps_id, which is the identifier of the HPS RBSP being used in the slice.

  - The HPS contains header parameter combination entries for N (= hps_num_poc_values_once) + M (= hps_num_poc_values_periodic) entries. An initial POC LSB (= hps_initial_pic_order_cnt_lsb) for the first as well as a POC delta (hps_poc_delta[ i ]) is specified for each of these entries. POC LSB values are mapped to header parameter combination entries accordingly, where the M entries are repeated until the entire value range of POC LSB values (from 0 to MaxPicOrderCntLsb – 1, inclusive) is taken. Figure 1 illustrates this mapping process.

Figure 1. An illustration of mapping POC LSB values to header parameter combination entries.

For example, in random access common conditions both N and M may be equal to 8 (the "GOP length") and an HPS may be sent for each IRAP picture (to enable starting of decoding from each IRAP picture). The N header parameter combination entries may differ from the M header parameter combination entries for example in that not all reference pictures may be available in the N entries and hence RPS can differ in the respective entries among the N and M entries.

  - Either slice_pic_order_cnt_lsb or hps_entry_idx (included in the slice segment header), as conditioned by hps_entry_idx_present_flag, is used to pick the header parameter combination entry within the HPS RBSP. When slice_pic_order_cnt_lsb is used, it used to index the created mapping shown above. hps_entry_idx specifies an index among the N + M header parameter combination entries.

It is asserted that the proposal has the following characteristics and benefits:

- The "forward-looking" mapping of header parameter combination entries to slice_pic_order_cnt_lsb values enables coding of many header parameter subset structures within the same HPS RBSP and facilitates compact signaling which HPS is in use (as fewer HPSs may be used and hence slice_hps_id can be shorter) and which header parameter combination entry is in use (as typically no additional syntax element would be needed).

- It is assumed that the encoder makes most decisions on slice header parameters based on the given configuration rather than based on the picture content. The header parameter subsets for which dynamic content-based parameter value selection is made can be excluded from the header parameter combination entries and hence coded conventionally in the slice segment headers themselves.

- The optional hps_entry_idx makes it possible to use the HPS mechanism when HPSs are sent out-of-band or when MaxPicOrderCntLsb is smaller than the interval of sending HPSs in-band (and taking them in use).

- HPS RBSPs can be repeated in the bitstream as many times as needed to obtain sufficient error resilience.

- The slice segment header syntax is kept intact until pic_order_cnt_lsb, inclusive, which enables POC-based AU boundary detection without HPS parsing and also slice_pic_order_cnt_lsb-based indexing of header parameter combination entries.

The detailed proposal is indicated with change marks in the accompanying specification text.

# Statistics of the slice header bit count

We used HTM 9.0 to implement the statistics collection of the slice header bit count. The C3 common test conditions (JCT3V-F1100) were used in the test. Both MV-HEVC and 3D-HEVC configurations were tested.

It is noted that one slice per picture was used. Currently HTM does not support slice-based operation and hence we could not perform a simulation that would better suit typical transmission environments, such as limiting the slice size according to the maximum transfer unit of the Ethernet link layer (1500 bytes – protocol headers of higher protocol stack layers). Consequently, the slice header overhead in packet-based transmission is likely to be higher than shown in the statistics below.

We derived the slice header bit count for only the enhancement layers, as the base layer was assumed to remain compatible with HEVC version 1. The overhead is derived against the EL bitrate only as well as against the total bitrate.

In conclusion, the overhead of EL slice headers is more than 3% when compared to EL bitrate only and about 1% when compared to the total bitrate. Table 1 and 2 include the collected MV-HEVC and 3D-HEVC statistics, respectively.

Table 1. Overhead of EL slice headers in MV-HEVC C3 configuration.

| Sequences | # Frames | EL slice header bit count / EL bit count | EL slice header bit count / total bit count |
| --- | --- | --- | --- |
| Balloons | 300 | 3.71% | 1.46% |
| Kendo | 300 | 3.65% | 1.52% |
| Newspaper | 300 | 4.02% | 1.55% |
| Shark | 300 | 2.65% | 0.62% |
| Poznan Street | 250 | 3.83% | 1.02% |
| Undo dancer | 250 | 1.79% | 0.55% |
| GT_Fly | 250 | 2.98% | 0.71% |
| Poznan Hall | 200 | 4.40% | 1.88% |
| average |  | 3.38% | 1.16% |

Table 2. Overhead of EL slice headers in 3D-HEVC C3 configuration.

| Sequences | # Frames | EL slice header bit count / EL bit count | EL slice header bit count / total bit count |
| --- | --- | --- | --- |
| Balloons | 300 | 4.11% | 1.32% |
| Kendo | 300 | 3.29% | 1.26% |
| Newspaper | 300 | 3.62% | 1.08% |
| Shark | 300 | 1.88% | 0.51% |
| Poznan Street | 250 | 2.82% | 0.61% |
| Undo dancer | 250 | 2.12% | 0.40% |
| GT_Fly | 250 | 3.60% | 0.98% |
| Poznan Hall | 200 | 5.80% | 1.59% |
| average |  | 3.41% | 0.97% |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).