| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0141v2 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0141v2 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On temporal enhancement layers |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution asserts that "diagonal" inter-layer prediction would be useful when an SHVC-coded temporal enhancement layer is provided for an AVC base layer and when an enhancement layer provides a temporal enhancement, possibly along with spatial or quality enhancement, relative to base layer, where the picture rate ratio is non-dyadic, e.g. 24 Hz base layer and 50 Hz enhancement layer.

When no conventional inter-layer prediction from the same access unit is used, it is proposed to enable the use of other pictures from a direct reference layer as a reference for prediction as follows:

- An additional short-term RPS syntax structure can be included in the slice segment header for a direct reference layer. The additional short-term RPS syntax structure specifies the pictures from the direct reference layer that are included in the initial reference picture list(s) of the current picture, but causes no change on the marking of the pictures.

- The decoding process for reference picture lists construction is modified to include reference pictures from the additional short-term RPS syntax structure for the current picture.

The specification text of version 2 of the contribution responds to the comments expressed in the first JCT-VC review on 10th January, 2014. The following changes were implemented in the proposed specification text:

- A gating flag in SPS multilayer extension specifies if diagonal inter-layer prediction is enabled in the slice header level.

- Diagonal inter-layer prediction can be used even if normal inter-layer prediction is used. (In version 1 of the contribution, it was specified that if normal inter-layer prediction is used, diagonal prediction is not used.)

- Motion vectors are constrained to be 0 when diagonal inter-layer prediction from a reference layer causing upsampling is used.

- Definitions were updated and it was checked that terms inter-layer reference picture, aligned inter-layer reference picture and diagonal inter-layer reference picture are used appropriately throughout the text.

An open issue is whether the number of diagonal prediction reference layers is limited to 1 or whether diagonal prediction is allowed from any number of direct reference layers. The specitication text presents the former option, and also includes editor's notes commenting how to allow a number of diagonal prediction reference layers greater than 1.

In version 1, the specification text changes were presented on top of the MV-HEVC draft text. Bullet 3 above require the specification text changes in version 2 of the contribution to be presented on top of the SHVC draft text.

# Problem Statement

The contribution asserts that the support for the following use cases is desirable and therefore proposes changes to support the use cases.

- HEVC temporal enhancement layer for AVC base layer

In this use case it is desirable to let the enhancement-layer pictures to predict from temporally non-aligned base-layer pictures in order to avoid redundant coding of "skipped" enhancement layer pictures (that would co-exist in the same access units as the base-layer pictures) and the DPB memory usage for such "skipped" enhancement layer pictures.

In this use case the spatial resolution of the layers is the same.

- Non-dyadic temporal enhancement

In this use case only very few pictures are temporally aligned between the base layer and enhancement layer and hence reside in the same access units. An example of a 24-Hz base layer using a GOP size of 6 and a 50-Hz enhancement layer using a GOP size of 12 or 13 is illustrated above. Due to the picture rates, only every 12th picture of the base layer is aligned with every 25th picture in the enhancement layer. Yet, to improve compression efficiency it would be desirable to let the enhancement layer to predict from non-aligned base-layer pictures, as illustrated with red arrows in the figure above (where every 6th or 7th picture in the enhancement layer is predicted from the base layer).

In this use case the spatial resolution of the layers may differ.

# Proposal

When the inter-layer RPS is empty, it is proposed to enable the use of other pictures from a direct reference layer as a reference for prediction as follows:

- An additional short-term RPS syntax structure can be included in the slice segment header for a direct reference layer. The additional short-term RPS syntax structure specifies the pictures from the direct reference layer that are included in the initial reference picture list(s) of the current picture, but causes no change on the marking of the pictures.

- The decoding process for reference picture lists construction is modified to include reference pictures from the additional short-term RPS syntax structure for the current picture.

Note that only one reference picture form the direct reference layer that causes resampling to be invoked can be included in the additional short-term RPS, as already constrained by the Scalable Main profile. If quality scalability is in use between the direct reference layer and the current layer, the number of pictures in the additional short-term RPS is not separately limited, but the constraint on NumPicTotalCurr (all pictures that may be used for reference) jointly limits the number of reference pictures from the current layer and the reference pictures included in the additional short-term RPS.

The detailed proposal is presented by change-marked text in the accompanying specification document.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).