| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0142 |
| --- | --- |

| Title: | On comparison of 3D-HEVC and MV-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Ye Yan University of Science and Technology of China | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution suggests aspects to be taken into account when preparing bitstreams for comparing the performance of MV-HEVC (with depth auxiliary picture layers) and 3D-HEVC.

# Proposals on the Planned Testing and Comparison

JCT-3V AHG9 has a mandate to prepare for a visual comparison between MV-HEVC (with depth map carriage) and 3D-HEVC at the 7th JCT-3V meeting. It is presumed that one target for such visual testing is to have evidence whether 3D-HEVC provides a sufficient RD performance improvement to justify the formal establishment of the project.

The following proposals are made on the planned testing:

- MV-HEVC (with depth coding) and 3D-HEVC should have the same level of rate-distortion and view synthesis optimization (RDO and VSO, respectively) for the prepared bitstreams.

Our understanding of the HTM software is that VSO is not supported for MV-HEVC with depth coding. Hence, either HTM needs to be updated for the support of VSO for MV-HEVC with depth coding or VSO has to be turned off for both MV-HEVC and 3D-HEVC.

- The codecs should be used with their best configuration.

- Spatial resolution of depth views.

MV-HEVC allows using a depth resolution that differs from texture resolution. It is known in MVC+D and 3D-AVC that the selection of depth resolution improves the performance compared to using full-resolution depth (see e.g. JCT3V-E0035). Our initial tests with MV-HEVC, with VSO turned off, conclude similarly that depth resolution equal to the luma texture resolution is not the optimal choice, see Section  for details.

- Range extension coding tools.

Depth pictures are coded with the auxiliary picture layer mechanism, which does not constrain the used coding tools. Specifically, the depth auxiliary picture layers can be coded with version 1 tools or with REXT tools. As the REXT amendment is to be finalized with the same schedule as the MV-HEVC amendment, REXT tools are readily available for depth coding.

- SHVC coding tools.

It is remarked that the auxiliary picture layer mechanism does not preclude using of inter-layer motion prediction (while it is disabled as an additional constraint currently). The use of inter-layer motion prediction was reported to provide about 1% BD-rate decrease in synthesis PSNR when allowing motion prediction from texture to depth (see Section 2.2 for details). Enabling of SHVC inter-layer motion prediction should be considered when comparing MV-HEVC and 3D-HEVC, as it is a tool that is readily available in MV-HEVC also for depth coding.

# Summary of initial results

The results have been provided here merely to support some of the arguments above rather than to give an accurate characterization of the current performance of MV-HEVC or HTM. They should be understood to be indicative only.

## Spatial resolution of depth views

The following results were obtained using SHM3 separately for texture and depth coding for 49-frame sequences. Quarter depth resolution (half both vertically and horizontally) is compared to full depth resolution in the results.

Table 1. MV-HEVC C3 configuration, quarter depth resolution compared to full depth resolution.

## SHVC inter-layer motion prediction from texture to depth

These results were obtained using SHM1 for full-length sequences. The results have been published in [Y. Yan, M. M. Hannuksela, and H. Li, "Multiview-Video-plus-Depth Coding and Inter-component Prediction in a High-level Syntax Extension of H.265/HEVC", Proc. of Picture Coding Symposium, Dec. 2013].

Table 2. MV-HEVC C3 configuration, inter-component motion prediction (ICMP) on compared to ICMP off.

| Sequence | Depth PSNR / depth bitrate | Synth PSNR / total bitrate |
| --- | --- | --- |
| Balloons | -2.3% | -0.5% |
| Kendo | -1.4% | -0.7% |
| Newspaper_CC | -0.9% | -0.3% |
| GT_Fly | -9.1% | -1.5% |
| Poznan_Hall2 | -5.0% | -1.5% |
| Poznan_Street | -2.9% | -0.4% |
| Undo_Dancer | -3.3% | -2.5% |
| 1024x768 | -1.6% | -0.5% |
| 1920x1088 | -5.1% | -1.5% |
| Average(all) | -3.6% | -1.0% |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

The authors are not aware of University of Science and Technology of China having any current or pending patent rights relating to the technology described in this contribution.