| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0147 |
| --- | --- |

| Title: | 3D-CE1.h: Simplification of sub-PU level temporal interview motion prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Young Su Heo, Min Seong Lee, Woo Woen Gwun, Yoon Jin Lee, Gwang Hoon Park Media Lab., Kyung Hee University 1, Seochun, Kiheung, Youngin, Gyonggi, 446-701, Korea | Tel: Email: | +82-31-201-3680 {medist, yomega00, woogwun, rainmake, ghpark} @khu.ac.kr |
| Source: | Kyung Hee University |  |  |

_____________________________

# Abstract

This contribution proposes early decision and hole-filling method for SPIVMP simplification. The experimental results show that average 0.0% BD-rate is observed while memory access for motion information is reduced and parallelism is supported for SPIVMP.

# Introduction

In sub-PU level inter-view motion prediction (SPIVMP), one set of temporary motion information is stored and updated for reference blocks which have unavailable motion information [1], [2]. For each sub-PU, if the reference block of the current sub-PU has available motion information, temporary motion information is updated with the motion information of the reference block. Otherwise, if the reference block does not have available motion information, the temporary motion information is copied to the current sub-PU. And this process is called the motion hole-filling process.

Figure 1. Flowchart of the current SPIVMP Scheme

# Problem statements

## Data dependency in motion hole-filling process

The motion hole-filling process uses temporary motion information for an unavailable reference block (motion hole). In the current 3D-HEVC, to fill the motion hole, SPIVMP process needs to find where the last available reference sub-PU position is. Therefore, temporary motion information must be updated every time after the end of each derivation process of sub-PU level motion prediction. It makes the dependency between sub-PUs so current SPIVMP is not friendly with parallel processing [3].

## Memory access

For the SPIVMP, when corresponding reference block of center sub-PU does not contain any available motion information, most of rest corresponding blocks which are belong to the current PU also don’t contain available motion information. In that case, temporal inter-view merging candidate is typically useless regardless of hole-filling. Therefore, skipping derivation process for motion information for SPIVMP might be better than accessing to the rest corresponding reference blocks.

However, the current derivation process always accesses to all corresponding reference blocks and add temporal inter-view merging candidate to the list even all corresponding reference blocks of sub-PUs are intra coded. It causes unnecessary external memory access.

# Proposed algorithm

Figure 2 shows the proposed simplification of derivation process for a sub prediction block temporal inter-view motion vector candidate.

First, the default motion information is derived from corresponding reference block of sub-PU which is located at center of current PU. The center position (centX, centY) is derived as following:

centX = Clip3(0, PicWidthInSampleL - 1, (nPSW/nSubPsW/2)*nSubPsW + nSubPsW/2 )

centY = Clip3(0, PicHeightInSampleL - 1, (nPSH/nSubPsH/2)*nSubPsH + nSubPsH/2 )

nPSW, nPSH, nSubPsW, nSubPsH is the width and height of the PU, and the width and height of the sub-PU, respectively.

If default motion information is not available, the derivation process is terminated. Therefore, number of memory access to the corresponding reference blocks can be reduced.

Otherwise, if default motion information is available, the sub-PU level derivation process is performed and unavailable motion informations from corresponding reference blocks are filled with the default motion information. Sub-PU level parallelism for SPIVMP is supported by using default motion information.

Figure 2. Flowchart of the proposed simplification for SPIVMP

# Experimental results

The proposed simplification is tested under HTM-9.0r1 based on the CTC [4]. The coding performance is shown in Table 1. For the experimental analysis, average 20% of center sub-PUs are intra coded and derivation process for temporal inter-view merging candidate is early terminated.

Table 1. Results of proposed simplification for SPIVMP (vs. HTM-9.0r1)

# Conclusion

This contribution proposes early decision and hole-filling method for SPIVMP simplification. The memory accessing is reduced by early decision according to availability of the default motion information and Sub-PU level parallelism is supported by using default motion information. The experimental results are almost same to the anchor.

# Reference

- L. Zhang, G. Tech, K. Wegner, Y. Chen and S. Yea, “Test Model 6 of 3D-HEVC and MV-HEVC,” The 6th JCT-3V Meeting, JCT3V-F1005, Geneva, CH, November 2013.

- J. An, K. Zhang, J.-L. Lin and S. Lei, “3D-CE3: Sub-PU level inter-view motion prediction”, The 6th JCT-3V Meeting, JCT3V-F0110, Geneva, CH, November 2013.

- X. Zhao, L. Zhang, Y. Chen and V. Thirumalai, “CE3 related: Simplifications to sub-PU level inter-view motion prediction”, The 6th JCT-3V Meeting, JCT3V-F0127, Geneva, CH, November 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV Core Experiments,” The 6th JCT-3V Meeting, JCT3V-F1100, November 2013.

# Patent rights declaration(s)

Kyung Hee University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).