| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0148 |
| --- | --- |

| Title: | CE1: Motion parameters stored for VSP-coded blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shinya Shimizu*, Shiori Sugimoto 1-1, Hikari-no-Oka, Yokosuka, Kanagawa, JAPAN | Tel: Email: | *+81-46-859-2703 shimizu.shinya@lab.ntt.co.jp |
| Source: | NTT Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to store sub-PU level motion parameters for the blocks where view synthesis prediction is applied instead of a PU level disparity parameter which is utilized to derive a virtual depth map. The motion parameters proposed to be stored are actually utilized to generate prediction signals. Experiments reportedly show that about 0.11% and 0.08% bitrate reductions for coded and synthesized views, respectively, without decoder complexity increase. The coding result with the deblocking filter on 8x8 blocks boundaries in VSP-coded blocks is also provided.

# Introduction

View synthesis prediction is specified as two-step disparity compensation in the current WD [1]. Figure 1 illustrates the procedure of the current view synthesis prediction. The first disparity compensation is PU-based processing to generate virtual depth map for the current PU. The second disparity compensation is sub-PU-based processing to generate actual predictor for the coding target picture.

Figure 1: View synthesis prediction in 3D-HEVC

In the current WD, the first disparity parameter (the light blue vector in Figure 1) is stored in the motion storage for the blocks where VSP is applied. We proposed to store the second disparity parameters (the red vectors in Figure 1) in JCT3V-F0176 [2]. This contribution provides the coding result based on HTM v9.0r1 and the proposed working draft. The coding result with the deblocking filter on boundaries of 8x8 blocks in VSP-coded blocks is also provided.

# Proposal

The proposed method is to store the second disparity parameters (the red vectors in Figure 1), which are actually utilized for prediction, in the motion storage instead of the first disparity parameters (the light blue vector in Figure 1), which are utilized to fetch virtual depth map as shown in Figure 2. The stored disparity parameters are subjected to the other processes including de-blocking filter, AMVP, TMVP, NBDV derivation, and MPI as specified in the current WD.

Figure 2: Disparity parameters stored in motion memory

The proposed draft is attached to this contribution. Since the view synthesis prediciton is performed in sub-PU level as the sub-PU inter-view motion prediction does, it is proposed to integrate view synthesis prediction with sub-PU inter-view motion prediction.

# Experiments

The proposed methods are built on the top of the reference software 3DV-HTM v9.0r1. And the experiments are conducted according to the common test condition [3].

TABLE 1 shows the simulation results by the proposed method. As it can be seen, the proposed method achieves about 0.1% bitrate reduction on average for both coded and synthesized views.

Since the proposed change is only the selection of disparity parameter to be stored and the disparity parameters proposed to be stored are already derived in the current process, the impact on the decoder complexity might be negligible. Furthermore, the proposed change might be better align with the other inter prediction modes and more reasonable in terms of the concept of motion storage.

TABLE 1: Simulation results

We also perform an additional simulation where the deblocking filter is applied to the sub-PU boundaries in VSP coded PUs. Since the base specification of HEVC prohibit applying the deblocking filter to internal edges in 8x8 block, the deblocking filter is applied to the 8x8 block boundaries in VSP coded Pus although sub-PU size is 8x4 or 4x8. TABLE 2 shows the simulation results with applying the deblocking filter, and there is almost no impact in term of the coding efficiency.

TABLE 2: Additional simulation results with applying the deblocking filter to 8x8 block boundary in VSP coded PUs

# Conclusion

In this document, it is proposed to store disparity parameters utilized to generate prediction signals for VSP coded blocks. The proposed method is better aligned to the other inter prediction method including sub-PU inter-view motion prediction and there is no decoder complexity increase. We recommend adopting the proposed method.

# Patent rights declaration(s)

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, “3D-HEVC Draft Text 2”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-F1001 version 4, December 2013, Geneva, Switzerland.

- S. Shimizu and S. Sugimoto, “CE1-related: Motion parameters stored at VSP-coded blocks”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-F0176, October 2013, Geneva, Switzerland.

- JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, “Common Test Conditions of 3DV Core Experiments”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-F1100, October 2013, Geneva, Switzerland.