| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0151r1 |
| --- | --- |

| Title: | AHG13: Results with quarter resolution depth map coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shinya Shimizu*, Shiori Sugimoto (NTT) 1-1, Hikari-no-Oka, Yokosuka, Kanagawa, JAPAN | Tel: Email: | *+81-46-859-2703 shimizu.shinya@lab.ntt.co.jp |
| Source: | NTT Corporation |  |  |

_____________________________

# Abstract

This contribution reports the result of quarter resolution depth map coding on MV-HEVC. In the simulation, VSO was activated for quarter resolution depth map coding with assuming the nearest neighbour upsampling would be performed during view synthesis. Experiments reportedly show that about 3.38% bitrate reduction and 6.22% bitrate increase for coded and synthesized views, respectively, with saving about 30% decoder runtime.

Revision 1 provides additional coding results with enabling the texture coding tools in 3D-HEVC except for VSP and DoNBDV. Relative to the common anchor, where all 3D-HEVC coding tools are used, bitrate increase of about 15% for synthesized views is reported with about 27% decoding runtime saving. When only the same coding tools are enabled, the quarter resolution depth map coding provides an bitrate increase of about 8.2% for synthesized view with about 28% decoding runtime saving.

# Introduction

The current MV-HEVC and 3D-HEVC supports only a full resolution depth map coding while MVC+D and 3D-AVC support a reduced resolution depth map coding. As already reported in [1], it is difficult to achieve better coding performance with reduced resolution depth map coding for 3D-HEVC. However, such capability would be beneficial since decoder complexity can be reduced in terms of memory usage, the number of operation, and throughput of pixel.

This contribution reports the coding result with quarter resolution depth map based on the latest version of HTM.

# Implementation

In case of MV-HEVC, it is decided to convey depth map as one of the auxiliary picture [2]. However, the latest HTM does not support the auxiliary picture coding. Therefore, in our implementation, depth maps are still coded as normal pictures. The spatial resolution of each picture is encoded in SPS.

There is no inter-component prediction for MV-HEVC; no modification is necessary except for VSO. The VSO in HTM requires full resolution depth map in order to estimate distortion after view synthesis, and this estimation of distortion is performed for each predictor and results of residual coding. Therefore, in order to reduce encoder complexity, depth map is upsampled by the zero-order hold upsampling in our implementation. It might be worth noting that mismatch between the assumed upsampling method in VSO and those actually used on the post-processing brings large loss in terms of view synthesis quality and there are many reports that more sophisticate upsampling also provide large improvement of view synthesis quality.

# Experiments

The proposed methods are built on the top of the reference software 3DV-HTM v9.0r1-F0122. And the experiments are conducted according to the common test condition [3]. The following three sets of simulations were conducted.

Test case 1:

Anchor: Full resolution depth map with no 3D-HEVC coding tool

Tested: Quarter resolution depth map with no 3D-HEVC coding tool

Test case 2:

Anchor: Full resolution depth map with all texture and depth coding tools (the common anchor)

Tested: Quarter resolution depth map with texture coding tools except for BVSP and DoNBDV

Test case 3:

Anchor: Full resolution depth map with texture coding tools except for BVSP and DoNBDV

Tested: Quarter resolution depth map with texture coding tools except for BVSP and DoNBDV (Same as test case 2)

It is noted that the texture coding tools enabled in the test case 2 and 3 require no change on the current WD.

TABLE 1 shows the simulation resultd. For the test case 1, which corresponds to MV-HEVC, as it can be seen, the proposed method achieves about 3.38% bitrate reduction and 6.22% bitrate increase for coded and synthesized views, respectively, with saving about 30% decoder runtime. For the Kendo sequence, the quarter resolution depth map coding also achieves more than 1% bitrate reduction.

The results for the test case 2 show that quarter resolution depth map coding with no change on the current 3D-HEVC specification gives about 15% bitrate increase with 27% decoder runtime saving. When comparing the results where the same coding tools are used for both anchor and tested (test case 3), the bitrate increase reduced to 8.20% with almost the same decoder runtime saving.

In addition to the flexibility, it might be beneficial to support reduced resolution depth map coding since the spatial resolution of existing depth sensors is lower than that of normal camera in general. If reduced resolution depth map coding is not allowed, new standard cannot accept direct inputs from such sensors.

TABLE 1: Simulation results

(a) Test case 1

(b) Test case 2

(c) Test case 3

Due to the lack of time, we couldn’t finish implementation of quarter resolution depth map coding for 3D-HEVC. In order to support full 3D-HEVC, we need to modify many coding tools, MPI, DMM, DoNBDV, and BVSP. However, as reported in JCT3V-D0215, such modifications might be simple.

# Conclusion

In this document, it is reported that a quarter resolution depth map coding gives significant decoder complexity reduction with reasonable bitrate increases for MV-HEVC/3D-HEVC. We recommend allowing reduced resolution depth map coding.

# Patent rights declaration(s)

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- S. Sugimoto, S. Shimizu, K. Kimata, “3D-HEVC: Report on Reduced Resolution Depth Map Coding in 3D-HEVC”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-D0215, April 2013, Incheon, Korea.

- JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, “MV-HEVC Draft Text 6”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-F1004, November 2013, Geneva, Switzerland.

- JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, “Common Test Conditions of 3DV Core Experiments”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-F1100, October 2013, Geneva, Switzerland.