| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0182r2 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0160r2 |
| --- | --- |

| Title: | AHG9: On Sub-bitstream extraction and rewriting process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Yan Ye 9710 Scranton Road, #250 San Diego, CA 92121 USA | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com |
| Source: | InterDigital Communications, Inc |  |  |

_____________________________

# Abstract

This contribution proposes parameter set syntax signaling modifications and constraints to simplify the sub-bitstream extraction and bitstream rewriting process.

Based on the track discussion, an indicator is added to enable the simply re-writing process in section 5, and additional informative sub-bitstream extraction process is added in section 6.

# Introduction

Sub-bitstream extraction is applied on the input bitstream with a target highest TemporalId value and a target layer identifier list, and the output sub-bitstream may include a particular number of layers and/or temporal sub-layers. It is specified that a conforming sub-bitstream shall contain one or more coded slice segment NAL units with nuh_layer_id equal to 0 and TemporalId equal to 0.

Figure 1 is an example of sub-bitstream extraction process on a multiple-layer bistream, and the output sub-bitstream contains two temporal sub-layers of two layers (layer-0 and layer-1).

- Sub-bitstream extraction example

One special case of sub-bitstream extraction process is to extract an independent layer from the scalable bitstream into a single layer bitstream conforming to HEVC v1, such process is called bitstream rewriting process.

Figure 2 is an example of bitstream rewriting process, where an independent non-base layer (layer-1) of a multiple layer scalable bitstream is extracted into a single layer bitstream with nuh_layer_id equal to 0.

- Bitstream rewriting example

The parameter sets such as VPS and SPS applied on particular layers of the bitstream may not be valid for the sub-bitstream after the extraction process or the single layer bitstream after the rewriting process. For instance, some of the parameter set syntax elements are structured based on the consecutive layer structure, or conditioned by the nuh_layer_id value of the parameter set NAL unit. Therefore, any change to the layer structure of the parameter sets due to the extraction and/or rewriting process may affect the presence of particular syntax elements in VPS and SPS.

In addition, in order to output a conforming single layer bitstream, the rewriting process may have to modify the nuh_layer_id value of the independent non-base layer extracted and its active SPS/PPS, and reformat the SPS/PPS structure. The current working draft does not address these issues adequately.

One way to carry out the extraction/rewriting process is to require an extractor such as a middle box to be able to parse all parameter set syntax elements, analyze changes required for the output layers’ parameters, and generate new parameter sets for the output sub-bitstream or single layer bitstream. This would not only increase the workload of the middle box, but also require the middle box to have the capability and knowledge of a scalable codec.

This contribution proposes parameter set syntax signaling modifications and constraints to simplify the sub-bitstream extraction and bitstream rewriting process.

# Layer set constraint

A layer set indicates a set of layers for the operation of the sub-bitstream extraction process with the target highest TemporalId equal to 6. Currently profiles are indicated in the VPS extension syntax for each output layer set instead of the entire bitstream, and parameters such as avg_bit_rate_idc, avg_pic_rate and DPB size are also signaled for each output layer set in VPS VUI.

A layer set may include all layers, or a number of dependent layers, or a mix of independent layers and dependent layers. One special layer set is the layer set including a single independent non-base layer if such kind of layer is available.  In order to facilitate the rewriting process at the middle box, a constraint is proposed to mandate that the scalable bitstream must include a layer set for each independent non-base layer in the bitstream.

- Each one of all independent non-base layers shall be exclusively included in a layer set of the bitstream.

# VPS for sub-bitstream extraction

VPS and its extension are designed for session negotiation and capability exchange of video conferencing and streaming applications. Some cross-layer associated syntax elements are signaled based on the consecutive layer index given the maximum number of layers (vps_max_layers_minus1) available; examples of these syntax elements include direct_dependency_flag[i][j], max_tid_il_ref_pics_plus1[i][j] and direct_dependency_type[i][j], where i and j indicate the i-th and j-th layer.

After the sub-bitstream extraction process, the original layer structure could be broken and the same layer index may map to  different actual layers in the sub-bitstream. In order to be able to regenerate the VPS for the extracted sub-bitstream in a straightforward manner, without requiring the middle-box to parse and re-generate the parameter set syntax elements, it is proposed to signal those cross-layer associated syntax in the VPS VUI for each layer set. This way, the sub-bitstream extraction process can directly refer to the corresponding parameter syntax elements of a desired layer set to re-generate the VPS for the extracted sub-bitstream.

Table 1 is the proposed cross-layer associated syntax structure per layer set to be included in the VPS VUI.

- Proposed layer set info syntax structure

| layer_set_info( ) { |  |
| --- | --- |
| for( s = 1; s < NumOutputLayerSets; s++ ) { |  |
| layerSetMaxLaysMinus1 = NumLayersInIdList[s] |  |
| for( i = 1; i <= layerSetMaxLayersMinus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| layerSet_direct_dependency_flag[s][i][j] | u(1) |
| if(max_tid_ref_present_flag ) |  |
| for( i = 0; i < layerSetMaxLayersMinus1; i++ ) |  |
| for( j = i + 1; j <= layerSetMaxLayersMinus1; j++ ) |  |
| if( layerSet_direct_dependency_flag[s][j][i] ) |  |
| layerSet_max_tid_il_ref_pics_plus1[s][i][j] | u(3) |
| if (!default_direct_dependency_type) { |  |
| for( i = 1; i <= layerSetMaxLayersMinus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( layerSet_direct_dependency_flag[s][i][j] ) |  |
| layerSet_direct_dependency_type[s][i][j] | u(v) |
| } |  |
| } |  |

layerSet_direct_dependency_flag[s][i][j] equal to 0 specifies that the layer with index j is not a direct reference layer for the layer with index I of s-th output layer set. layerSet_direct_dependency_flag[s][i][j] equal to 1 specifies that the layer with index j may be a direct reference layer for the layer with index I of s-th output layer set. When layerSet_direct_dependency_flag[s][i][j] is not present for i and j in the range of 0 to layerSetMaxLayersMinus1, it is inferred to be equal to 0.

layerSet_max_tid_il_ref_pics_plus1[s][i][j] equal to 0 specifies that within the CVS non-IRAP pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] are not used as reference for inter-layer prediction for pictures with nuh_layer_id equal to layer_id_in_nuh[ j ] of s-th output layer set. max_tid_il_ref_pics_plus1[s][i][j] greater than 0 specifies that within the CVS pictures with nuh_layer_id equal to layer_id_in_nuh[i] and TemporalId greater than max_tid_il_ref_pics_plus1[s][i][j] – 1 are not used as reference for inter-layer prediction for pictures with nuh_layer_id equal to layer_id_in_nuh[j] of s-th output layerset. When not present, max_tid_il_ref_pics_plus1[s][i][j] is inferred to be equal to 7.

layerSet_direct_dependency_type[s][i][j] indicates the type of dependency between the layer with nuh_layer_id equal layer_id_in_nuh[ i ] and the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] of s-th output layer set. layerSet_direct_dependency_type[s][i][j] equal to 0 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for both inter-layer sample prediction and inter-layer motion prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ]. layerSet_direct_dependency_type[s][i][j] equal to 1 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for inter-layer sample prediction but not for inter-layer motion prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ] of s-th output layer set. layerSet_direct_dependency_type[i][j] equal to 2 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for inter-layer motion prediction but not for inter-layer sample prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ] of s-th output layer set. The length of the layerSet_direct_dependency_type[s][i][j] syntax element is direct_dep_type_len_minus2 + 2 bits. Although the value of layerSet_direct_dependency_type[s][i][j] shall be in the range of 0 to 2, inclusive, in this version of this Specification, decoders shall allow values of layerSet_direct_dependency_type[s][i][j] in the range of 3 to 232 − 2, inclusive, to appear in the syntax. When not present, the value of layerSet_direct_dependency_type[s][i][j] is inferred to be equal to default_direct_dependency_type.

# SPS/PPS for re-writing process

For the case of bitstream re-writing, the value of nuh_layer_id of the independent non-base layer needs to be changed to 0. As a result, if the value of nuh_layer_id of the active SPS applied on the independent non-base layer is greater than 0, it needs to be changed to 0, and the syntax elements such as update_rep_format_flag and sps_rep_format_idx need to be removed and syntax elements such as sps_max_sub_layers_minus1, sps_temporal_id_nesting_flag and profile_tier_level() need to be copied from corresponding VPS syntax. The same re-writing process may also be applied to the PPS due to the syntax element pps_infer_scaling_list_flag.

To simplify the re-writing process such that the active SPS/PPS of the independent non-base layer will not need to be modified, a constraint is proposed as follows:

- A coded slice segment NAL unit of an independent non-base layer shall refer to a PPS with nuh_layer_id equal to 0.

- The value of nuh_layer_id of the SPS activated for an independent non-base layer shall be equal to 0

In this way, majority of the SPS/PPS syntax elements can be reused after the re-writing process without any modification except the representation format syntax values.

It is noted that, in order to enable SPS sharing between base and enhancement layers, in the current draft specifications the values of representationrepresentation format syntax elements chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, and bit_depth_chroma_minus8 are inferred from the VPS (rather than from the SPS) when a non-base layer refers to an SPS with nuh_layer_id equal to 0.

Two options are proposed here to simplify the representation format syntax mapping for the re-writing process, but still be able to allow SPS sharing.

## Option 1

One way is to mandate the values of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, and bit_depth_chroma_minus8 signaled in the active SPS applied on an independent non-base layer to be identical to the corresponding values inferred from the VPS. Therefore the same active SPS applied on the independent non-base layer can be applied on the extracted single layer bitstream without any modifications.

## Option 2

An alternative way is to signal the corresponding representation format syntax values in an SEI message. This way, the re-writing process can replace the values of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, and bit_depth_chroma_minus8 signaled in the original active SPS with the values signaled in the SEI message directly without the need to refer to VPS.

- Proposed SEI message of independent non-base layer representation format

| sei_rep_format(payloadSize){ | Descriptor |
| --- | --- |
| independent_layer_id | u(6) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| } |  |

independent_layer_id specifies the value of nuh_layer_id of the independent non-base layer.

pic_width_in_luma_samples, pic_height_in_luma_samples, chroma_format_idc, separate_colour_plane_flag, bit_depth_luma_minus8, and bit_depth_chroma_minus8 share the same semantics as those specified in SPS.

# Re-writing indicator signaling

The track discussion suggests, instead of mandating all independent non-base layers to obey the proposed constraints, to add an indicator whether a particular independent non-base layer can be rewritten since their SPSs and PPs obey the proposed constraints. Table 3 shows the suggested indicator in the VPS extension.

- Proposed re-writing indicator in VPS extension

| vps_extension( ) { | Descriptor |
| --- | --- |
| avc_base_layer_flag | u(1) |
| vps_vui_present_flag | u(1) |
| if( vps_vui_present_flag ) |  |
| vps_vui_offset | u(16) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask_flag[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask_flag[ i ] |  |
| } |  |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i <= MaxLayersMinus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| view_id_len | u(4) |
| if( view_id_len > 0 ) |  |
| for( i = 0; i < NumViews; i++ ) |  |
| view_id_val[ i ] | u(v) |
| for( i = 1; i <= MaxLayersMinus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| for( i = 1; i <= MaxLayersMinus1; i++ ) |  |
| if( NumDirectRefLayers[ layer_id_in_nuh[ i ] ] = = 0 ) |  |
| rewriting_enabled_flag[ i ] | u(1) |
| …… |  |
| } |  |

rewriting_enabled_flag[ i ] equal to 1 specifies that the following constraints apply to the layer with nuh_layer_id equal to layer_id_in_nuh[ i ], rewriting_enabled_flag equal to 0 specifies that the following constraints may or may not apply to the layer with nuh_layer_id equal to layer_id_in_nuh[ i ].

- Each coded slice segment NAL unit with nuh_layer_id value equal to layer_id_in_nuh[ i ] referring to the VPS shall refer to a PPS with nuh_layer_id value equal to 0.

- Each coded slice segment NAL unit with nuh_layer_id value equal to layer_id_in_nuh[ i ] referring to the VPS shall refer to a SPS with nuh_layer_id value equal to 0.

- The values of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, and bit_depth_chroma_minus8, respectively, of the active SPS for the layer with nuh_layer_id equal to layer_id_in_nuh[ i ] shall be the same as the values of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, and bit_depth_chroma_minus8, respectively, of the vps_rep_format_idx[ i ]-th rep_format( ) syntax structure in the active VPS.

# Sub-bitstream extraction process

An informative process to create a base layer during the sub-bitstream extraction process is presented as follows.

F. 11 Specification of base layer bitstream

A bitstream that conforms to one or more profiles as specified in Annex F shall contain a base layer bitstream that conforms to one or more of the profiles specified in Annex A. This base layer bitstream is derived by invoking the sub-bitstream extraction process as specified in clause F.10 with no input and the base layer bitstream being the output.

F. 11.1 Creation of conforming sub-bitstream extraction from a set of layers not containing the base layer (informative)

According to the sub-bitstream extraction process specified in F.10, the resulting sub-bitstream shall contain a base layer with one or more coded slice segment NAL units with nuh_layer_id equal to 0 and TemporalId equal to 0. When the resulting bitstream does not contain a base layer, the following procedure may be used to create a base layer during sub-bitstream extraction.

When TargetDecLayerIdList of a particular output layer set specified by the VPS does not contain a nuh_layer_id value equal to 0, let nuhLayerIdA be equal to the minimum nuh_layer_id value included in TargetDecLayerIdList, and apply the following operations in sequential order:

- Remove all VCL NAL units with TemporalId greater than tIdTarget or nuh_layer_id not among the values included in TargetDecLayerIdList.

- For each of the remaining VCL NAL units with nuh_layer_id equal to nuhLayerIdA, set the value of nuh_layer_id equal to 0.

- Remove all SEI NAL units (with nal_unit_type equal to 39 or 40) that have nuh_layer_id equal to 0 and that contain a non-nested buffering period SEI message, a non-nested picture timing SEI message, or a non-nested decoding unit information SEI message.

- For each SEI NAL unit containing a scalable nesting SEI message, the following operations are applied in sequential order:

- When none of the following is true for the scalable nesting SEI message, the SEI NAL unit is removed:

- nesting_op_flag is equal to 0 and all_layers_flag is equal to 1

- nesting_op_flag is equal to 0, all_layers_flag is equal to 0, and at least one of the values of nesting_layer_id[ i ] for all i in the range of 0 to nesting_num_layers_minus1, inclusive, is equal to the value of one of the nuh_layer_id value of TargetDecLayerIdList.

- nesting_op_flag is equal to 1, nesting_max_temporal_id_plus1[ i ] is equal to or less than tIdTarget, and the list of nesting_op_idx[ i ] for all i in the range of 0 to nesting_num_ops_minus1, inclusive, is a subset of TargetDecLayerIdList.

- When the SEI NAL unit is not removed, the following applies:

- if TargetDecLayerIdList contains only one nuh_layer_id value, the SEI NAL unit is replaced by an SEI NAL unit containing only the original nested SEI message not as part of a scalable nesting SEI message.

- Otherwise, when any of the following is true for the scalable nesting SEI message, a new SEI NAL unit containing only the nested SEI message not as part of a scalable nesting SEI message is generated and inserted immediately before the original SEI NAL unit in decoding order:

  - nesting_op_flag is equal to 0 and all_layers_flag is equal to 1

  - nesting_op_flag is equal to 0, all_layers_flag is equal to 0, and for the values of nesting_layer_id[ i ] for all i in the range of 0 to nesting_num_layers_minus1, inclusive, one is equal to nuhLayerA, and at least another one is equal to the value of one of the nuh_layer_id in TargetDecLayerIdList.

- When TargetDecLayerIdList contains only one value of nuh_layer_id, remove the all SEI NAL units with payloadType equal to xxx (to be defined for layers_not_present( ), inter_layer_constrained_tile_sets( ), bsp_nesting( ), bsp_initial_arrival_time( ), bsp_hrd( ), three_dimensional_reference_displays_info( ), and depth_representation_info_sei( )).

- Remove all non-VCL NAL units with TemporalId greater than tIdTarget or with nuh_layer_id greater than 0 but not among the values included in TargetDecLayerIdList.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).