| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, USA, 9 – 17 Jan. 2014 | Document: JCTVC-P0192v2 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, USA, 11 – 17 Jan. 2014 | Document: JCT3V-G0161v2 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On decoded picture buffer management |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Adarsh K. Ramasubramonian Ying Chen Qualcomm Incorporated 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-8345 yekuiw@qti.qualcomm.com 1-858-658-5801 aramasub@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

At JCT-VC#15 and JCT-3V#6 meetings in Geneva, the group agreed to specify a separate DPB capacity for each layer without sharing of DPB capacity across layers. This document proposes either to allow for DPB capacity sharing across layers to utilize the process for DPB memory optimization, or to remove the reference marking processes in subclause F.8.1.4 per discardable_flag and in subclause F.8.1.4.1 per VPS layer dependency signalling for specification clean-up.

V2 of JCTVC-P0192/JCT3V-G0161 provides the following changes in addition to the proposal alternative 1, in reposnding to comments received during the first review:

- Signal the maximum number of decoded pictures of each layer that need to be stored in the DPB (in addition to the maximum size of each sub-DPB).

- Use the added signalling in the DPB "bumping" process.

- A couple of bug fixes to the text in v1 of JCTVC-P0192/JCT3V-G0161.

# Introduction

The reference marking processes in subclause F.8.1.4 per discardable_flag and in subclause F.8.1.4.1 per VPS layer dependency signalling can reduce the memory size for the DPB when different layers may share their DPB capacities, because these processes could enable empty a picture store buffer during the decoding of an AU such that the same memory unit can be used for storage of decoded picture of a higher layer in the same AU. However, when each layer has its own sub-DPB and DPB capacity is not shared across layers, these marking processes are meaningless, as any picture (of a layer) that would be marked as "unused for reference" by these processes will be marked as "unused for reference" when starting decoding the next picture in that layer in decoding order. Marking at this moment is sufficient for emptying the picture store (if the picture is not needed for output, either) for storage of the next decoded picture in that layer (or any picture subsequent in decoding order).

During earlier discussions, it was mentioned that as per the HEVCv1 specification the moment at which the decoded pictures of the current access unit are being stored into the DPB is at the end of decoding the whole current access unit, and thus the above-mentioned marking processes could still help even DPB capacity is not shared across layers. However, that interpretation of the HEVCv1 specification is not aligned with the design intent, as it has been clearly specified that the current decoded picture is stored in the DPB rather than being stored in a temporary buffer that is not a part of the DPB as in AVC, and the decoder would start to store parts of the decoded picture of the current picture before the current picture is completely decoded. Of course a decoder implementation, if it does not care about optimal memory size requirement, could also choose to put partially decoded current picture not in the DPB, in which case it would need to have one more picture store of memory that is not part of the DPB.

Therefore, we propose either to really enable the reference marking processes in subclause F.8.1.4 per discardable_flag and in subclause F.8.1.4.1 per VPS layer dependency signalling to be utilized for memory optimization, or to remove the process for specification clean-up.

# Proposal alternative 1

The layers for which the decoded pictures are of the same size in bytes share their DPB capacities. In other words, these layers share the same sub-DPB. These layers are the layers that have the same spatial resolution, bit depths, and colour format.

Text changes for this proposal alternative are provided in the attachment, based on JCT3V-F1004v6, by the user name "Resolution-specific sub-DPB".

The attachment also includes some bug fixes originated from Gerhard Tech, with slight changes by the authors. The authors would like to thank Gerhard for those bug fixes.

# Proposal alternative 2

The marking process for sub-layer non-reference pictures not needed for inter-layer prediction specified in subclause F.8.1.4.1 is removed. The early marking of discardable pictures specified in subclause F.8.1.4 is also removed.

Text changes for this proposal alternative are also provided in the attachment, based on JCT3V-F1004v6, by the user name "Layer-specific sub-DPB".

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).