| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0204v2 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0165v2 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Sub-bitstream property SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Chen Ye-Kui Wang Adarsh K. Ramasubramonian Hendry 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-845-6589 cheny@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com 1-858-651-4722 fhendry@qti.qualcomm.com 1-858-658-5801 aramasub@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In order to correctly decode the target output layers, it is not necessary for each AU to be "complete", i.e. picture from each layer in the bitstream is present in the AU. This allows use cases like picture rate scalability in SHVC as well as optimization of the size of the bitstream by discarding some pictures that are not needed for decoding of pictures of the target output layers, e.g. pictures with discardable_flag equal to 1 in layers that are not target output layers. In JCT3V-F0091/JCTVC-O0273, multi-mode sub-bitstream extraction has been proposed to enable extraction of sub-bitstreams that contain "incomplete" AUs to be confirming bitstreams. At the previous meeting, it was suggested to consider an SEI message to convey properties of such sub-bitstreams instead of making them conforming. This document proposes such an SEI message.

In v2 of JCT3V-G0165/JCTVC-O0204, the signalling of picture rate information is removed as it was concluded not needed during the discussion at the joint HLS BoG. In addition, some other syntax cleanups were made.

# Introduction

In SVC, MVC, MVC+D, and HEVC version 1, the sub-bitstream extraction process is specified. Each extractable sub-bitstream is required to be a conforming bitstream.

One issue that has been considered in the design of the bitstream extraction process, e.g. in MVC development, is whether to target the best sub-bitstream self-completeness (such that each sub-bitstream further extracted from a sub-stream, recursively, can be a conforming bitstream) or the optimal size of a sub-bitstream (such that some or all pictures that are not needed for correct decoding of the target output layers do not have to be included in the extracted sub-bitstream).

Targeting best self-completeness makes the bitstream extraction process easier to specify. A self-complete sub-bitstream is still "fully extractable" and is more useful if the bitstream extraction is performed by a streaming server or an intermediate network element and each subset of the bitstream may be further requested by some end users. However, on the other hand, if the extracted sub-bitstream is directly sent to end users and is not supposed to be further extracted, better transmission bandwidth efficiency can be achieved by targeting the optimal size of the bitstream.

To enable either advantage to be applied depending on applications, the sub-bitstream extraction process can be designed to adaptively choose the best extraction operation.

Therefore, we proposed to support multi-mode bitstream extractions in JCT3V-E0087/JCTVC-N0267 and later JCT3V-F0091/JCTVC-O0273. However, as has been discussed at the 15th JCT-VC and 6th JCT-3V meetings, it was suggested not to specify the sub-bitstreams by the additional extraction modes to be conforming bitstreams, but rather to consider an SEI message for conveying the properties of such sub-bitstreams.

Therefore, in the document, we propose an SEI message to signal the properties of such sub-bitstreams.

# Proposal

## Sub-bitstream property SEI message

| sub_bitstream_property( payloadSize ) { | Descriptor |
| --- | --- |
| active_vps_id | u(4) |
| num_additional_sub_streams_minus1 | ue(v) |
| for( i = 0; i <= num_additional_sub_streams_minus1; i++) { |  |
| sub_bitstream_mode[ i ] | u(2) |
| output_layer_set_idx_to_vps[ i ] | ue(v) |
| highest_sublayer_id[ i ] | u(3) |
| avg_bit_rate[ i ] | u(16) |
| max_bit_rate[ i ] | u(16) |
| } |  |
| } |  |

## Sub-bitstream property SEI semantics

The sub-bitstream property SEI message, when present, provides the bit rate information for a sub-bitstream created by discarding those pictures in the layers that do not belong to the target output layers of the output layer sets specified by the active VPS and that do not affect the decoding of the target output layers.

When present, the sub-bitstream property SEI message shall be associated with an initial IRAP access unit, and the information provided by the SEI messages applies to the bitstream corresponding to the CVS containing the associated initial IRAP access unit.

active_vps_id identifies the active VPS. The value of active_vps_id shall be equal to the value of vps_video_parameter_set_id of the active VPS referred to by the VCL NAL units of the associated access unit.

num_additional_sub_streams_minus1 plus 1 specifies the number of the sub-bitstreams for which the bit rate information may be provided by this SEI message. The value of num_additional_sub_streams_minus1 shall be in the range of 0 to 210− 1, inclusive.

sub_bitstream_mode[ i ] specifies how the i-th sub-bitstream is generated. The value of sub_bitstream_mode[ i ] shall be equal to 0 or 1, inclusive. The values 2 and 3 are reserved for future use by ITU-T and ISO/IEC. When sub_bitstream_mode[ i ] is the greater than 1, decoders shall ingore the syntax elements output_layer_set_idx_to_vps[ i ], highest_sublayer_id[ i ], avg_bit_rate[ i ], and max_bit_rate[ i ].

When sub_bitstream_mode[ i ] is equal to 0, the i-th sub-bitstream is generated as specified by the following steps:

–	The sub-bitstream extraction process as specified in clause 10 is invoked with the bitstream corresponding to the CVS containing the sub-bitstream property SEI message, highest_sublayer_id[ i ], and LayerSetLayerIdList[ LsIdx4OptLs[ output_layer_set_idx_to_vps[ i ] ] ] as inputs. [Ed. (YK): This is in responding to Jill's suggestion to refer to the sub-bitstream extraction process rather than repeating here. We assume that the editorial fixes/improvements from Gerhard and YK, e.g. in JCTVC-P0192/JCT3V-G0161, would be integrated. If other edits would be done, then another variable name than LsIdx4OptLs with the same semantics should be used here.]

–	Remove all NAL units for which the nuh_layer_id is not included in TargetOptLayerIdList and either of the following conditions is true:

–	The value of nal_unit_type is not in the range of BLA_W_LP to RSV_IRAP_VCL23, inclusive, and max_tid_il_ref_pics_plus1[ LayerIdxInVps[ nuh_layer_id ] ][LayerIdxInVps[ layerId ] ] is equal to 0 for layerId values included in TargetOptLayerIdList.

–	 TemporalId is greater than the maximum value of max_tid_il_ref_pics_plus1[ LayerIdxInVps[ nuh_layer_id ] ][LayerIdxInVps[ layerId ] ] – 1 for all layerId values included in TargetOptLayerIdList.

When sub_bitstream_mode[ i ] is equal to 1, the i-th sub-bitstream is generated as specified by the above steps followed by:

–	Remove all NAL units with nuh_layer_id not among the values included in TargetOptLayerIdList and with discardable_flag equal to 1.

output_layer_set_idx_to_vps[ i ] specifies the index of the output layer set corresponding to the i-th sub-bitstream.

highest_sublayer_id[ i ] specifies the highest TemporalId of access units in the i-th sub-bitstream.

avg_bit_rate[ i ] indicates the average bit rate of the i-th sub-bitstream, in bits per second. The value is given by BitRateBPS( avg_bit_rate[ i ] ) with the function BitRateBPS( ) being specified as follows:

BitRateBPS( x ) = ( x & ( 214 − 1 ) ) * 10( 2 + ( x  >>  14 ) )		(F-X)

The average bit rate is derived according to the access unit removal time specified in clause Error: Reference source not found. In the following, bTotal is the number of bits in all NAL units of the i-th sub-bitstream, t1 is the removal time (in seconds) of the first access unit to which the VPS applies, and t2 is the removal time (in seconds) of the last access unit (in decoding order) to which the VPS applies. With x specifying the value of avg_bit_rate[ i ], the following applies:

–	If t1 is not equal to t2, the following condition shall be true:

( x & ( 214 − 1 ) )  = =  Round( bTotal ÷ ( ( t2 − t1 ) * 10( 2 + ( x  >>  14 ) ) ) )	(F-X+1)

–	Otherwise (t1 is equal to t2), the following condition shall be true:

( x & ( 214 − 1 ) )  = =  0		(F-X+2)

max_bit_rate[ i ] indicates an upper bound for the bit rate of the i-th sub-bitstream in any one-second time window of access unit removal time as specified in clause Error: Reference source not found. The upper bound for the bit rate in bits per second is given by BitRateBPS( max_bit_rate[ i ] ). The bit rate values are derived according to the access unit removal time specified in clause Error: Reference source not found. In the following, t1 is any point in time (in seconds), t2 is set equal to t1 + 1 ÷ 100, and bTotal is the number of bits in all NAL units of access units with a removal time greater than or equal to t1 and less than t2. With x specifying the value of max_bit_rate[ i ], the following condition shall be obeyed for all values of t1:

( x & ( 214 − 1 ) )  >=  bTotal ÷ ( ( t2 − t1 ) * 10( 2 + ( x  >>  14 ) ) )	(F-X+3)

-

-

-

- Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).