| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0167 |
| --- | --- |

| Title: | 3D-CE4: Crosscheck of Results on improved advanced residual prediction (JCT3V-G0064) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

- This contribution reports a cross-check of improvement ARP prediction. The experiment results perfectly match with those provided by the proponent. It is reported the BD-rate increase on average is 0.20 %, 0.20 % and 0.11 % in texture, video and synthesis respectively with 1/2 weight case and it is reported the BD-rate increase on average is 0.17 %, 0.15 % and 0.14 % in texture, video and synthesis respectively with 1/2 weight case.

# Test

Test send an index to indicate an ARP MV/DV candidate and weight (with weight case only) while HTM90 uses an index to indicate ARP weights (0, 1, 1/2).

| Index | Test (with weight) | Test (without weight) | HTM90 |
| --- | --- | --- | --- |
| 0 | No ARP | No ARP | No ARP (W = 0) |
| 1 | MV/DV index0 (W=1) | MV/DV index0 | W = 1 |
| 2 | MV/DV index1 (W=1) | MV/DV index1 | W = 1/2 |
| 3 | MV/DV index2 (W=1) | MV/DV index2 | - |
| 4 | MV/DV index0 (W=1/2) | - | - |
| 5 | MV/DV index1 (W=1/2) | - | - |
| 6 | MV/DV index2 (W=1/2) |  |  |

The DV index indicates which MV/DV candidate is used.

It seems MV / DV candidates are derived as following table.

|  | ARP MV cand | ARP DV cand |
| --- | --- | --- |
| DV candidate 0 | Left | Col (AltCol) |
| DV candidate 1 | Top | Left |
| DV candidate 2 | Col | Top |
| DV candidate 3 | Zero | NBDV (Left) |
| DV candidate 4 | MV + 4 | NBDV (Top) |
| DV candidate 5 | MV – 4 | Zero |
| DV candidate 6 | - | DV + 4 |
| DV candidate 7 | - | DV – 4 |

It seems pruning (unique check) is conducted each DV candidates and the first DV candidates.

# Experimental results

Table 1. adaptive ARP with ½ weighting (anchor: HTM9r1)

|  | video 0 | video 1 | video 2 | Video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.2% | -0.4% | -0.18% | -0.17% | -0.14% | 113.9% | 98.1% | 98.8% |
| Kendo | 0.0% | -1.3% | -1.4% | -0.64% | -0.61% | -0.39% | 110.9% | 97.0% | 96.9% |
| Newspaper_CC | 0.0% | -0.3% | -0.1% | -0.13% | -0.14% | -0.08% | 109.5% | 98.5% | 98.5% |
| GT_Fly | 0.0% | 0.0% | 0.0% | -0.07% | -0.06% | -0.04% | 106.6% | 96.8% | 97.5% |
| Poznan_Hall2 | 0.0% | -0.2% | 0.0% | -0.09% | -0.13% | 0.10% | 109.9% | 99.0% | 98.8% |
| Poznan_Street | 0.0% | -0.5% | -0.4% | -0.20% | -0.20% | -0.12% | 110.5% | 95.7% | 97.0% |
| Undo_Dancer | 0.0% | -0.4% | -0.4% | -0.18% | -0.17% | -0.16% | 113.4% | 98.3% | 97.4% |
| Shark | 0.0% | -0.2% | 0.0% | -0.11% | -0.12% | -0.03% | 106.9% | 98.4% | 99.1% |
| 1024x768 | 0.0% | -0.6% | -0.7% | -0.32% | -0.30% | -0.20% | 111.4% | 97.8% | 98.1% |
| 1920x1088 | 0.0% | -0.2% | -0.2% | -0.13% | -0.14% | -0.05% | 109.5% | 97.6% | 97.9% |
| average | 0.0% | -0.4% | -0.4% | -0.20% | -0.20% | -0.11% | 110.2% | 97.7% | 98.0% |

Table 2. adaptive ARP without ½ weighting (anchor: HTM9r1)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.8% | -0.6% | -0.28% | -0.25% | -0.29% | 103.0% | 98.5% | 100.1% |
| Kendo | 0.0% | -1.1% | -1.2% | -0.45% | -0.38% | -0.40% | 100.8% | 102.3% | 99.7% |
| Newspaper_CC | 0.0% | -0.3% | -0.2% | -0.09% | -0.08% | -0.09% | 100.6% | 101.3% | 99.5% |
| GT_Fly | 0.0% | -0.1% | -0.2% | -0.08% | -0.07% | -0.08% | 101.8% | 100.3% | 100.3% |
| Poznan_Hall2 | 0.0% | -0.1% | -0.2% | -0.05% | -0.04% | 0.03% | 102.7% | 99.3% | 100.0% |
| Poznan_Street | 0.0% | -0.3% | -0.4% | -0.07% | -0.06% | -0.09% | 102.9% | 99.7% | 99.7% |
| Undo_Dancer | 0.0% | -0.7% | -0.7% | -0.22% | -0.20% | -0.16% | 103.3% | 101.1% | 99.4% |
| Shark | 0.0% | -0.4% | -0.2% | -0.13% | -0.12% | -0.05% | 101.2% | 99.9% | 100.6% |
| 1024x768 | 0.0% | -0.8% | -0.7% | -0.27% | -0.24% | -0.26% | 101.5% | 100.7% | 99.8% |
| 1920x1088 | 0.0% | -0.3% | -0.3% | -0.11% | -0.10% | -0.07% | 102.4% | 100.1% | 100.0% |
| average | 0.0% | -0.5% | -0.5% | -0.17% | -0.15% | -0.14% | 102.0% | 100.3% | 99.9% |

# References

[1] K. Zhang, J. An, J.-L. Lin, Y.-L. Chang, S. Lei, “3D-CE4: Results on improved advanced residual prediction” JCT3V-G0064, JCT3V 7th Meeting: San José,  US, 11–17 Jan. 2014

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).