| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0169_r1 |
| --- | --- |

| Title: | 3D-CE3 related: Crosscheck of Simplification on NBDV derivation in 3D-HEVC (JCT3V-G0050) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

- This contribution reports a cross-check of simplification of NBDV prediction. The experiment results perfectly match with those provided by the proponent and the software was checked and confirmed that it follows the text. It is reported the BD-rate increase on average is 0.02 %, 0.01 % and 0.02 % in texture, video and synthesis respectively (Test1). In Test3, which modifies DV default values in addition to test 1, it is reported the BD-rate increase on average is 0.00 %, -0.01 % and -0.01 % in texture, video and synthesis respectively.

- Revision1 adds test result of NBDV case (where DepthRefinement is disabled), it is reported the BD-rate increase on average is 0.08 %, 0.07 % and 0.09 % in texture, video and synthesis respectively.

# Test

Test1 removes above block in NBDV derivation so that NBDV only uses temporal blocks and left block.

Test3 uses DepthToDvTable [128] as DV default value in addition to test1.

# Experimental results

Table 1. Test1 results (anchor: HTM9r1)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.3% | 0.03% | 0.02% | 0.00% | 101.6% | 100.9% | 100.9% |
| Kendo | 0.0% | 0.0% | -0.1% | -0.03% | -0.02% | -0.02% | 99.3% | 101.0% | 100.3% |
| Newspaper_CC | 0.0% | 0.0% | 0.3% | 0.05% | 0.02% | 0.08% | 99.4% | 100.5% | 100.0% |
| GT_Fly | 0.0% | 0.2% | 0.1% | 0.02% | 0.02% | 0.02% | 100.0% | 98.9% | 100.2% |
| Poznan_Hall2 | 0.0% | 0.3% | -0.2% | 0.02% | 0.01% | 0.04% | 100.0% | 101.3% | 100.1% |
| Poznan_Street | 0.0% | 0.1% | -0.1% | 0.02% | 0.02% | 0.01% | 100.3% | 99.3% | 99.5% |
| Undo_Dancer | 0.0% | 0.0% | 0.1% | 0.01% | 0.00% | 0.07% | 100.0% | 100.6% | 99.5% |
| Shark | 0.0% | 0.1% | 0.1% | 0.02% | 0.01% | 0.00% | 99.0% | 99.5% | 100.6% |
| 1024x768 | 0.0% | 0.0% | 0.2% | 0.02% | 0.01% | 0.02% | 100.1% | 100.8% | 100.4% |
| 1920x1088 | 0.0% | 0.1% | 0.0% | 0.02% | 0.01% | 0.03% | 99.9% | 99.9% | 100.0% |
| average | 0.0% | 0.1% | 0.1% | 0.02% | 0.01% | 0.02% | 99.9% | 100.3% | 100.1% |

Table 1. Test3 results (anchor: HTM9r1)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 101.0% | 101.2% | 100.4% |
| Kendo | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 98.9% | 100.9% | 100.5% |
| Newspaper_CC | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% | 98.6% | 99.9% | 99.7% |
| GT_Fly | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 99.7% | 100.7% | 100.2% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 99.8% | 99.4% | 99.8% |
| Poznan_Street | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 100.3% | 99.0% | 99.8% |
| Undo_Dancer | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 99.6% | 99.7% | 99.4% |
| Shark | 0.0% | -0.1% | -0.3% | 0.0% | 0.0% | 0.0% | 99.4% | 98.6% | 100.4% |
| 1024x768 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 99.5% | 100.7% | 100.2% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.8% | 99.5% | 99.9% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | -0.01% | 99.7% | 99.9% | 100.0% |

Table 1. Test1 without DepthDefine results (anchor: HTM9r1 without DepthRefine)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.4% | 0.5% | 0.13% | 0.12% | 0.19% | 99.7% | 95.6% | 96.5% |
| Kendo | 0.0% | 0.2% | 0.5% | 0.13% | 0.11% | 0.08% | 97.7% | 93.9% | 94.1% |
| Newspaper_CC | 0.0% | 0.4% | 0.3% | 0.11% | 0.08% | 0.11% | 99.8% | 96.6% | 97.9% |
| GT_Fly | 0.0% | 0.0% | 0.6% | 0.05% | 0.05% | 0.04% | 99.4% | 96.1% | 98.5% |
| Poznan_Hall2 | 0.0% | 0.1% | 0.2% | 0.06% | 0.04% | 0.08% | 99.9% | 98.7% | 98.8% |
| Poznan_Street | 0.0% | 0.2% | 0.0% | 0.04% | 0.04% | 0.06% | 99.7% | 99.1% | 98.1% |
| Undo_Dancer | 0.0% | 0.3% | 0.4% | 0.09% | 0.09% | 0.12% | 99.5% | 97.2% | 97.6% |
| Shark | 0.0% | 0.2% | 0.3% | 0.04% | 0.04% | 0.04% | 97.5% | 97.0% | 96.2% |
| 1024x768 | 0.0% | 0.3% | 0.4% | 0.12% | 0.10% | 0.13% | 99.1% | 95.4% | 96.2% |
| 1920x1088 | 0.0% | 0.2% | 0.3% | 0.06% | 0.05% | 0.07% | 99.2% | 97.6% | 97.9% |
| average | 0.0% | 0.2% | 0.3% | 0.08% | 0.07% | 0.09% | 99.2% | 96.8% | 97.2% |

Note: encoding time and decoding time is not so correct.

# References

[1] J.-L. Lin, Y.-W. Chen, K. Zhang, Y.-W. Huang, S. Lei, “3D-CE3 related: Simplification on NBDV derivation in 3D-HEVC” JCT3V-G0050, JCT3V 7th Meeting: San José,  US, 11–17 Jan. 2014

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).