| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0173 |
| --- | --- |

| Title: | 3D-CE5 related: Crosscheck of Conditional zero residual for segment-wise depth inter mode coding (JCT3V-G0058) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

- This contribution reports a cross-check of inter sdc coding. The experiment results perfectly match with those provided by the proponent. It is reported the BD-rate increase on average is 0.0 %, -0.1 % and 0.5 % in texture, video and synthesis respectively.

# Test

This test is a modified method which is implenented on top of JCT3V-G0101 and contains the following aspects:

- 2Nx2N only inter SDC (same as JCT3V-G0101)

- More accurate encoder RDO test (same as JCT3V-G0101)

- Absolute residual is decoded with (inter_sdc_resi_abs + 1) instead of inter_sdc_resi_abs in the case of merge mode. (JCT3V-G0058 only)

# Experimental results

Table 1. Test results (anchor: HTM9r1)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | -0.3% | 109.9% | 98.2% | 98.0% |
| Kendo | 0.0% | -0.2% | -0.1% | -0.1% | 0.1% | -0.4% | 108.6% | 99.7% | 98.2% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | -0.5% | 110.3% | 97.3% | 98.6% |
| GT_Fly | 0.0% | -0.3% | -0.3% | -0.1% | 0.1% | -0.6% | 109.6% | 98.6% | 100.0% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.2% | 0.0% | 0.3% | -0.6% | 110.2% | 99.1% | 99.7% |
| Poznan_Street | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | -0.2% | 109.7% | 97.6% | 99.6% |
| Undo_Dancer | 0.0% | -0.2% | -0.1% | 0.0% | 0.1% | -0.7% | 109.0% | 99.7% | 100.1% |
| Shark | 0.0% | -0.2% | -0.4% | -0.1% | 0.0% | -0.9% | 109.4% | 95.2% | 98.0% |
| 1024x768 | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | -0.4% | 109.6% | 98.4% | 98.3% |
| 1920x1088 | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | -0.6% | 109.6% | 98.1% | 99.5% |
| average | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | -0.5% | 109.6% | 98.2% | 99.0% |

# References

[1] Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei, J.-Y. Lee, M.-W. Park, “3D-CE5 related: Conditional zero residual for segment-wise depth inter mode coding” JCT3V-G0058, JCT3V 7th Meeting: San José,  US, 11–17 Jan. 2014

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).