| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0181 |
| --- | --- |

| Title: | CE5: Crosscheck on simplification on SDC planar coding (JCT3V-G0113) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Jian-Liang Lin No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yi-Wen Chen +886-3-5670766 ext. 26297 {yiwen.chen, jl.lin}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports cross-check result for the proposal of Hisilicon on the simplification on SDC planar coding. The proposed scheme is cross-checked and the results are matched with those provided by Hisilicon.

# Verification

Hisilicon provided the HTM-9.0r1-based source codes of their proposed method described in JCT3V-G0113 [1]. Three verifications are carried out by setting the following macros in the provided code:

case f0170-firstN

#define REMOVE_SDC_64                           0

#define SDC_PLANAR_SUBSAMPLE        1 & !REMOVE_SDC_64

#if SDC_PLANAR_SUBSAMPLE

#define SDC_PLANAR_FIRSTN                  1

#endif

case f0170-subsample

#define REMOVE_SDC_64                           0

#define SDC_PLANAR_SUBSAMPLE        1 & !REMOVE_SDC_64

#if SDC_PLANAR_SUBSAMPLE

#define SDC_PLANAR_FIRSTN                  0

#endif

case f0170-removeSDC64

#define REMOVE_SDC_64                          1

The simlations are conducted on the common test condiciton and all-intra conditions[2].

# Results

The BD-rates from our experimental results match those provided by Hisilicon. The detailed results can be seen in JCT3V-G0181_FisrtN_AI.xls, JCT3V-G0181_FisrtN_CTC.xls, JCT3V-G0181_RemoveSDC64_AI.xls, JCT3V-G0181_RemoveSDC64_CTC.xls, JCT3V-G0181_SbSample_AI.xls and JCT3V-G0181_SubSample_CTC.xls.

# Conclusion

The results presented by Hisilicon in JCT3V-G0113 are confirmed.

# References

- X. Zheng, Y. Lin, X. Chen, J. Zheng, “CE5: Test results on simplification on SDC planar coding,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-G0113, Jan., 2014.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-F1100, Oct, 2013.