| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0189 |
| --- | --- |

| Title: | CE5 related: Crosscheck on MediaTek's proposal "Simplified Intra Prediction for Depth Coding in 3D-HEVC (JCT3V-G0054)" |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | X. Zheng | Tel: Email: | +86-755-28425082 zhengxiaozhen@huawei.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

# Abstract

This contribution document reports the cross verification results of proposal G0054. The source code was implemented based on HTM-9.0r1. We compiled, inspected, and ran the code of these cases with CTC and all-intra configurations. The cross-check has been completed successfully and the results match those were provided by proponents of JCT3V- G0054.

# Test description and crosscheck results

This document reports the cross-check results of G0054. The encoder and decoder reconstructed YUV files generated from the above settings are matched. The experimental results of the rate-distortion are illustrated in Error: Reference source not found to Table 2. These results match the BD-rate results provided by proponents of JCT3V- G0054. Please note that the encoding and decoding time shown in the tables are not reliable.

Table 1: HTM9.0r1 anchor vs. proposed test1 in G0054 (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.01% | 0.01% | 0.05% | 124.5% | 135.3% | 117.5% |
| Kendo | 0.0% | -0.1% | 0.0% | -0.02% | -0.03% | 0.01% | 125.7% | 114.6% | 121.2% |
| Newspaper_CC | 0.0% | 0.0% | 0.1% | 0.03% | 0.00% | 0.03% | 125.4% | 110.8% | 120.5% |
| GT_Fly | 0.0% | 0.0% | -0.1% | -0.01% | -0.02% | -0.04% | 125.8% | 126.4% | 119.8% |
| Poznan_Hall2 | 0.0% | -0.1% | -0.1% | 0.00% | -0.04% | 0.02% | 125.7% | 126.6% | 117.8% |
| Poznan_Street | 0.0% | -0.1% | 0.0% | -0.01% | -0.01% | 0.06% | 125.7% | 130.7% | 120.2% |
| Undo_Dancer | 0.0% | -0.1% | -0.1% | -0.02% | -0.07% | -0.06% | 125.8% | 119.0% | 126.5% |
| Shark | 0.0% | 0.1% | 0.0% | 0.00% | -0.08% | -0.05% | 125.5% | 138.0% | 121.0% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.01% | 0.00% | 0.03% | 125.2% | 120.2% | 119.7% |
| 1920x1088 | 0.0% | -0.1% | -0.1% | -0.01% | -0.04% | -0.02% | 125.7% | 128.1% | 121.1% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | -0.03% | 0.00% | 125.5% | 125.2% | 120.6% |

Table 2: HTM9.0r1 anchor vs. proposed test of G0054 (all intra)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | 0.12% | 129.6% | 138.9% | 126.9% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | -0.02% | 0.04% | 130.2% | 142.8% | 132.6% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | -0.02% | -0.01% | 131.2% | 128.6% | 124.9% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | -0.04% | -0.22% | 130.2% | 124.1% | 122.4% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | 0.05% | 129.2% | 138.6% | 119.9% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.03% | 130.3% | 128.4% | 120.3% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | -0.06% | -0.13% | 129.3% | 124.2% | 117.2% |
| Shark | 0.0% | 0.0% | 0.0% | 0.00% | -0.13% | -0.38% | 130.2% | 126.0% | 123.1% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | -0.02% | 0.05% | 130.4% | 136.7% | 128.2% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | -0.05% | -0.13% | 129.9% | 128.3% | 120.6% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | -0.04% | -0.06% | 130.0% | 131.4% | 123.4% |