| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San Jose, US, 11 – 17 Jan. 2014 | Document: JCT3V-G0210 |
| --- | --- |

| Title: | 3D-CE2 related: Cross-check of JCT3V-G0055: a texture-partition-dependent depth partition for 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | J. Jung 38 Rue Général Leclerc 92130 Issy-les-Moulineaux France | Tel: Email: | +33145295366 joelb.jung@orange.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

The document reports cross-check results for the contribution JCT3V-G0055. It has been verified that common conditions are followed and that results provided by the proponent (MediaTek) are complete and match with the results reported in this document. The source code has also been checked, the software has been compiled and no issue is reported.

# Introduction

Quad-tree limitation (QTL) and predictive coding (PC)Error: Reference source not found proposed in JCT3V-A0044 reduce the encoding time and improve the coding performance for depth coding. With the quad-tree limitation, a coding depth CU cannot be split deeper than its collocated CU in the texture.

When AMP is enabled, the current QTL process set a strong restriction: 2Nx2N partition is the only allowed partition for a depth CU unless the collocated CU is coded in NxN partition.

JCT3V-G0055 proposes to allow depth partitions based on the texture partition direction. For a depth CU, the vertical/horizontal partitions are allowed only when the collocated texture CU at the same quad-tree level is coded with the vertical/horizontal partitions or NxN partition.

# Proposal verification

Anchor: HTM9.0r1 with common conditions.

Tested: anchor with proposed method

The software has been verified and compiled. It does not raise any issue.

Results obtained by MediaTek and Orange Labs are reported in the following tables.

Results obtained by MediaTek:

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | 0,0% | -0,1% | 0,0% | 0,0% | -0,1% | 99,9% | 96,5% | 97,2% |
| Kendo | 0,0% | -0,1% | -0,1% | 0,0% | 0,0% | -0,2% | 98,6% | 106,7% | 100,9% |
| Newspaper_CC | 0,0% | -0,1% | 0,1% | 0,0% | 0,0% | -0,1% | 98,0% | 97,1% | 100,2% |
| GT_Fly | 0,0% | 0,0% | 0,0% | 0,0% | 0,1% | -0,2% | 97,1% | 95,0% | 100,1% |
| Poznan_Hall2 | 0,0% | 0,0% | 0,1% | 0,0% | 0,1% | -0,2% | 97,7% | 99,1% | 97,0% |
| Poznan_Street | 0,0% | -0,1% | -0,1% | 0,0% | 0,0% | 0,0% | 99,1% | 94,6% | 96,8% |
| Undo_Dancer | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | -0,3% | 98,2% | 100,0% | 100,2% |
| Shark | 0,0% | 0,0% | -0,1% | 0,0% | 0,0% | -0,1% | 95,5% | 95,3% | 99,3% |
| 1024x768 | 0,0% | -0,1% | 0,0% | 0,0% | 0,0% | -0,2% | 98,8% | 100,1% | 99,4% |
| 1920x1088 | 0,0% | 0,0% | -0,1% | 0,0% | 0,0% | -0,2% | 97,5% | 96,8% | 98,7% |
| average | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | -0,2% | 98,0% | 98,0% | 99,0% |

Results obtained by Orange Labs

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | 0,0% | -0,1% | 0,0% | 0,0% | -0,1% | 98,3% | 96,1% | 94,4% |
| Kendo | 0,0% | -0,1% | -0,1% | 0,0% | 0,0% | -0,2% | 98,0% | 97,2% | 102,9% |
| Newspaper_CC | 0,0% | -0,1% | 0,1% | 0,0% | 0,0% | -0,1% | 100,4% | 88,3% | 58,5% |
| GT_Fly | 0,0% | 0,0% | 0,0% | 0,0% | 0,1% | -0,2% | 99,3% | 98,7% | 102,1% |
| Poznan_Hall2 | 0,0% | 0,0% | 0,1% | 0,0% | 0,1% | -0,2% | 99,7% | 115,8% | 102,6% |
| Poznan_Street | 0,0% | -0,1% | -0,1% | 0,0% | 0,0% | 0,0% | 97,8% | 95,7% | 92,5% |
| Undo_Dancer | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | -0,3% | 97,8% | 93,5% | 104,1% |
| Shark | 0,0% | 0,0% | -0,1% | 0,0% | 0,0% | -0,1% | 96,1% | 97,5% | 95,7% |
| 1024x768 | 0,0% | -0,1% | 0,0% | 0,0% | 0,0% | -0,2% | 98,9% | 93,9% | 85,3% |
| 1920x1088 | 0,0% | 0,0% | -0,1% | 0,0% | 0,0% | -0,2% | 98,1% | 100,2% | 99,4% |
| average | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | -0,2% | 98,4% | 97,8% | 94,1% |

Perfect match is achieved.

# Conclusion

This verification contribution confirms the results obtained by MediaTek.