| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0214 |
| --- | --- |

| Title: | CE1 related: Crosscheck on Modifications to motion hole filling in sub-PU level inter-view motion prediction (JCT3V-G0084) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hongbin Liu 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 86-10-57760696 hongbinl@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This proposal reports crosscheck results of modifications to motion hole filling in sub-PU level inter-view motion prediction (JCT3V-G0084) from Hisilicon. Simulation is conducted in CTC and two methods are tested. For method 1, there is some minor mismatch between crosscheck results and that provided in JCT3V-G0084, however, they re-run the simulation and their updated results matches with crosscheck results perfectly. For method 2, performance evaluation results perfectly matches with that provided in JCT3V-G0084.

# Introduction

In JCT3V-G0084, two methods are proposed to handle the motion hole in sub-PU level inter-view motion prediction.

- Method #1

For each sub-PU in the current PU, if the identified reference block does not contain available motion information, the followings are applied:

- If the sub-PU is the first one in the current PU, zero motion (reference index is set as 0, motion vectors for both List0 and List1 are set as (0, 0)) is set for this sub-PU;

- Otherwise, the corresponding motion information is inherited by the motion information of the previous sub-PU.

- Method #2

For each sub-PU in the current PU, if the identified reference block does not contain available motion information, the followings are applied:

- If the sub-PU is the first one in the current PU, DV information corresponding to current PU is set for this sub-PU;

- Otherwise, the corresponding motion information is inherited by the motion information of the previous sub-PU.

# Simulation results

Simulation is performed based on common test conditions and the coding performance is shown in Table 1 and Table 2.

Table 1: Comparison of method #1 with HTM-9.0r1 (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.06% | 0.11% | 0.03% | 0.01% | -0.07% | 100.1% | 103.5% |
| Kendo | 0.00% | -0.09% | 0.11% | 0.00% | 0.00% | 0.01% | 91.9% | 93.9% |
| Newspaper_CC | 0.00% | 0.07% | 0.14% | 0.04% | 0.04% | 0.06% | 94.8% | 95.6% |
| GT_Fly | 0.00% | 0.01% | -0.01% | 0.00% | -0.01% | 0.01% | 101.1% | 101.6% |
| Poznan_Hall2 | 0.00% | 0.20% | 0.23% | 0.09% | 0.08% | 0.15% | 91.3% | 96.8% |
| Poznan_Street | 0.00% | 0.02% | -0.04% | 0.00% | 0.00% | -0.01% | 100.5% | 100.1% |
| Undo_Dancer | 0.00% | -0.01% | 0.05% | 0.00% | 0.00% | 0.02% | 101.4% | 105.7% |
| Shark | 0.00% | 0.00% | -0.01% | 0.00% | -0.01% | 0.00% | 116.7% | 112.4% |
| 1024x768 | 0.00% | 0.01% | 0.12% | 0.02% | 0.01% | 0.00% | 95.6% | 97.7% |
| 1920x1088 | 0.00% | 0.05% | 0.04% | 0.02% | 0.01% | 0.03% | 102.2% | 103.3% |
| average | 0.00% | 0.03% | 0.07% | 0.02% | 0.01% | 0.02% | 99.7% | 101.2% |

Table 2: Comparison of method #2 with HTM-9.0r1 (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.05% | 0.08% | 0.01% | 0.01% | -0.05% | 111.5% | 111.7% |
| Kendo | 0.00% | -0.04% | 0.08% | 0.02% | 0.05% | 0.02% | 100.8% | 100.3% |
| Newspaper_CC | 0.00% | 0.06% | 0.07% | 0.02% | 0.02% | -0.01% | 91.2% | 97.3% |
| GT_Fly | 0.00% | -0.07% | -0.08% | -0.01% | -0.01% | -0.01% | 89.4% | 95.6% |
| Poznan_Hall2 | 0.00% | 0.25% | 0.25% | 0.08% | 0.04% | 0.10% | 92.1% | 96.8% |
| Poznan_Street | 0.00% | 0.09% | -0.05% | 0.01% | 0.01% | 0.01% | 91.5% | 94.4% |
| Undo_Dancer | 0.00% | -0.07% | -0.14% | -0.02% | -0.03% | -0.05% | 102.8% | 104.3% |
| Shark | 0.00% | 0.05% | -0.09% | 0.00% | -0.01% | -0.01% | 106.7% | 104.4% |
| 1024x768 | 0.00% | -0.01% | 0.08% | 0.02% | 0.03% | -0.01% | 101.2% | 103.1% |
| 1920x1088 | 0.00% | 0.05% | -0.02% | 0.01% | 0.00% | 0.01% | 96.5% | 99.1% |
| average | 0.00% | 0.03% | 0.02% | 0.01% | 0.01% | 0.00% | 98.2% | 100.6% |

# Conclusion

Crosscheck results perfectly match with that provided in JCT3V-G0084-v2.