| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0216 |
| --- | --- |

| Title: | CE5: Crosscheck on Segment-wise depth inter mode coding (JCT3V-G0101) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hongbin Liu 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 86-10-57760696 hongbinl@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This proposal reports crosscheck results of Segment-wise depth inter mode coding (JCT3V-G0101) from Samsung. Simulation is conducted in CTC and three simulation sets are tested. Performance evaluation results perfectly matches with that provided in JCT3V-G0101.

# Introduction

JCT3V-G0101 proposes to only apply inter SDC to CU coded with 2Nx2N partition mode, and they allow delta DC to be equal to zero in inter SDC. Meanwhile, they also improve the delta DC by selecting best delta DC from {deltaDC – 1, deltaDC, deltaDC + 1} or {deltaDC – 2, deltaDC – 1, deltaDC, deltaDC + 1, deltaDC + 2} at encoder through view synthesis optimization process. Here, deltaDC is the DC residual of inter SDC mode in 3D-HTM. Three simulation set are tested:

- Test #1

Only apply inter SDC to CU coded with 2Nx2N partition mode and allow delta DC to be equal to zero in inter SDC.

- Test #2

Test #1 + selecting best delta DC from {deltaDC – 1, deltaDC, deltaDC + 1}.

- Test #3

Test #1 + selecting best delta DC from {deltaDC – 2, deltaDC – 1, deltaDC, deltaDC + 1, deltaDC + 2}.

# Simulation results

Simulation is performed based on both common test conditions and the coding performance is shown in Table 1, Table 2 and Table 3 respectively.

Table 1: Comparison of test #1 with HTM-9.0r1 (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.17% | -0.14% | -0.06% | -0.08% | -0.10% | 117.2% | 111.5% |
| Kendo | 0.00% | -0.16% | -0.24% | -0.07% | -0.07% | 0.01% | 105.2% | 98.3% |
| Newspaper_CC | 0.00% | -0.06% | 0.06% | 0.00% | -0.03% | 0.00% | 90.6% | 90.3% |
| GT_Fly | 0.00% | 0.05% | -0.01% | 0.01% | 0.01% | 0.10% | 95.8% | 94.5% |
| Poznan_Hall2 | 0.00% | 0.15% | 0.09% | 0.05% | 0.03% | 0.22% | 89.7% | 92.5% |
| Poznan_Street | 0.00% | -0.13% | -0.02% | -0.02% | -0.04% | 0.01% | 95.8% | 98.7% |
| Undo_Dancer | 0.00% | -0.06% | -0.02% | -0.01% | -0.02% | 0.01% | 100.6% | 104.8% |
| Shark | 0.00% | 0.11% | 0.08% | 0.02% | 0.02% | 0.21% | 110.3% | 101.2% |
| 1024x768 | 0.00% | -0.13% | -0.11% | -0.04% | -0.06% | -0.03% | 104.3% | 100.0% |
| 1920x1088 | 0.00% | 0.03% | 0.03% | 0.01% | 0.00% | 0.11% | 98.4% | 98.3% |
| average | 0.00% | -0.03% | -0.02% | -0.01% | -0.02% | 0.06% | 100.6% | 99.0% |

Table 2: Comparison of test #2 with HTM-9.0r1 (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.05% | 0.06% | 0.01% | 0.06% | -0.25% | 122.1% | 114.9% |
| Kendo | 0.00% | -0.25% | -0.04% | -0.05% | 0.02% | -0.29% | 110.5% | 101.4% |
| Newspaper_CC | 0.00% | -0.10% | 0.03% | -0.01% | 0.13% | -0.24% | 111.4% | 102.6% |
| GT_Fly | 0.00% | -0.28% | -0.18% | -0.04% | 0.06% | -0.40% | 110.4% | 102.6% |
| Poznan_Hall2 | 0.00% | 0.25% | 0.02% | 0.06% | 0.22% | -0.36% | 90.4% | 92.7% |
| Poznan_Street | 0.00% | -0.02% | -0.10% | -0.01% | 0.08% | -0.11% | 105.2% | 100.8% |
| Undo_Dancer | 0.00% | -0.16% | -0.14% | -0.02% | 0.07% | -0.47% | 99.6% | 97.2% |
| Shark | 0.00% | -0.03% | -0.21% | -0.03% | 0.06% | -0.39% | 119.9% | 112.4% |
| 1024x768 | 0.00% | -0.10% | 0.02% | -0.02% | 0.07% | -0.26% | 114.7% | 106.3% |
| 1920x1088 | 0.00% | -0.05% | -0.12% | -0.01% | 0.10% | -0.34% | 105.1% | 101.1% |
| average | 0.00% | -0.07% | -0.07% | -0.01% | 0.09% | -0.31% | 108.7% | 103.1% |

Table 3: Comparison of test #3 with HTM-9.0r1 (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.09% | -0.07% | 0.00% | 0.10% | -0.35% | 128.8% | 116.6% |
| Kendo | 0.00% | -0.21% | -0.14% | -0.07% | 0.06% | -0.39% | 100.4% | 88.6% |
| Newspaper_CC | 0.00% | -0.04% | 0.03% | 0.00% | 0.21% | -0.40% | 106.9% | 97.7% |
| GT_Fly | 0.00% | -0.13% | -0.24% | -0.03% | 0.13% | -0.60% | 117.0% | 107.5% |
| Poznan_Hall2 | 0.00% | 0.16% | 0.15% | 0.06% | 0.30% | -0.47% | 111.4% | 106.0% |
| Poznan_Street | 0.00% | -0.10% | -0.09% | -0.03% | 0.10% | -0.16% | 111.5% | 103.4% |
| Undo_Dancer | 0.00% | -0.04% | -0.09% | -0.01% | 0.13% | -0.54% | 116.4% | 108.9% |
| Shark | 0.00% | -0.10% | -0.19% | -0.04% | 0.09% | -0.80% | 137.1% | 116.1% |
| 1024x768 | 0.00% | -0.06% | -0.06% | -0.02% | 0.12% | -0.38% | 112.0% | 101.0% |
| 1920x1088 | 0.00% | -0.04% | -0.09% | -0.01% | 0.15% | -0.51% | 118.7% | 108.4% |
| average | 0.00% | -0.05% | -0.08% | -0.01% | 0.14% | -0.46% | 116.2% | 105.6% |

# Conclusion

Crosscheck results perfectly match with that provided in JCT3V-G0101.