| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0218 |
| --- | --- |

| Title: | CE1: Crosscheck on Motion parameters stored for VSP-coded blocks (JCT3V-G0148) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hongbin Liu 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 86-10-57760696 hongbinl@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This proposal reports crosscheck results of Motion parameters stored for VSP-coded blocks (JCT3V-G0148) from NTT. Simulation is conducted in CTC. Performance evaluation results perfectly matches with that provided in JCT3V- G0148.

# Introduction

In JCT3V-G0148, NTT proposes to store motion parameter at sub-PU level for BVSP coded PUs. Meanwhile, they also test objective results of de-blocking sub-PU boundaries in BVSP coded PUs by identifying such sub-PU boundaries as edges in de-blocking process. Two simulation sets are tested.

- Test #1

Store motion parameter at sub-PU level for BVSP coded PUs.

- Test #2

Test #1 + de-blocking sub-PU boundaries in BVSP coded PUs

# Simulation results

Simulation is performed based on common test conditions and the coding performance is shown in Table 1 and Table 2.

Table 1: Comparison of test #1 with HTM-9.0r1 (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.32% | -0.07% | -0.07% | -0.07% | -0.08% | 99.9% | 103.1% |
| Kendo | 0.00% | -0.34% | -0.32% | -0.13% | -0.12% | -0.15% | 86.3% | 87.5% |
| Newspaper_CC | 0.00% | -0.17% | 0.07% | -0.02% | -0.02% | 0.00% | 90.0% | 93.1% |
| GT_Fly | 0.00% | -0.41% | -0.49% | -0.16% | -0.18% | -0.06% | 87.2% | 90.9% |
| Poznan_Hall2 | 0.00% | -0.29% | -0.16% | -0.06% | -0.09% | -0.09% | 90.4% | 95.4% |
| Poznan_Street | 0.00% | -0.26% | -0.19% | -0.07% | -0.06% | -0.07% | 83.3% | 89.6% |
| Undo_Dancer | 0.00% | -0.59% | -0.47% | -0.21% | -0.22% | -0.11% | 96.7% | 103.4% |
| Shark | 0.00% | -0.21% | -0.20% | -0.11% | -0.11% | -0.05% | 106.1% | 104.7% |
| 1024x768 | 0.00% | -0.28% | -0.11% | -0.07% | -0.07% | -0.08% | 92.1% | 94.6% |
| 1920x1088 | 0.00% | -0.35% | -0.30% | -0.12% | -0.13% | -0.08% | 92.7% | 96.8% |
| average | 0.00% | -0.33% | -0.23% | -0.10% | -0.11% | -0.08% | 92.5% | 96.0% |

Table 2: Comparison of test #2 with HTM-9.0r1 (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.25% | 0.03% | -0.04% | -0.05% | -0.04% | 99.9% | 102.2% |
| Kendo | 0.00% | -0.27% | -0.32% | -0.12% | -0.12% | -0.15% | 95.5% | 95.4% |
| Newspaper_CC | 0.00% | -0.17% | 0.07% | -0.02% | -0.02% | 0.00% | 90.6% | 96.3% |
| GT_Fly | 0.00% | -0.40% | -0.47% | -0.16% | -0.18% | -0.04% | 92.1% | 95.8% |
| Poznan_Hall2 | 0.00% | -0.32% | -0.16% | -0.08% | -0.10% | -0.06% | 87.2% | 93.6% |
| Poznan_Street | 0.00% | -0.31% | -0.13% | -0.07% | -0.06% | -0.08% | 87.4% | 93.1% |
| Undo_Dancer | 0.00% | -0.65% | -0.47% | -0.22% | -0.22% | -0.13% | 105.9% | 110.2% |
| Shark | 0.00% | -0.33% | -0.26% | -0.12% | -0.13% | -0.05% | 111.6% | 111.4% |
| 1024x768 | 0.00% | -0.23% | -0.07% | -0.06% | -0.06% | -0.06% | 95.3% | 98.0% |
| 1920x1088 | 0.00% | -0.40% | -0.30% | -0.13% | -0.14% | -0.07% | 96.8% | 100.8% |
| average | 0.00% | -0.34% | -0.21% | -0.10% | -0.11% | -0.07% | 96.3% | 99.7% |

# Conclusion

Crosscheck results perfectly match with that provide in JCT3V-G0148. Source code matches with working draft.