| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0222 |
| --- | --- |

| Title: | 3D-CE4: Cross check of Improved advanced residual prediction (JCT3V-G0064) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution provides a cross-check evaluation for the proposal of Improved advanced residual prediction from Mediatek. It is verified that the test results of the first two test sets presented in the proposal are identical to the proponent's one.

# Introduction

In [1], the motion vector candidate list is introduced for ARP. The proposal covers two parts: temporal/inter-view motion vector candidate list construction for ARP with/without half weighting factor, and aligned temporal DV (ATDV) as a new DV candidate. The software for the first part was studied, and there was no mismatch between the encoder and the decoder.

In the proposed first part, the candidate list with size equal to 3 is constructed and the candidate index is coded in the bitstream to indicate which candidate is used in the ARP process. To be more specifically, the possible candidates are derived as following table:

Table 1: motion vector canddiates used by ARP

| Candidate order | Temporal motion vector (TMV) candidates for temporal ARP | Disparity motion vector (DMV) candidates for inter-view ARP |
| --- | --- | --- |
| 0 | TMV from reference block | Up to 1 DMV from temporal neighbouring blocks used in NBDV |
| 1 | Scaled TMV from left block | DMV from left block |
| 2 | Scaled TMV from above block | DMV from top block |
| 3 | Scaled TMV from co-located block (bottom-right/center) | DoNBDV from left block |
| 4 | Zero | DoNBDV from above block |
| 5 | First TMV shifted by (4, 0) | DoNBDV from current block |
| 6 | First TMV shifted by (-4, 0) | Zero |
| 7 | - | First DMV shifted by (4, 0) |
| 8 | - | First DMV shifted by (-4, 0) |

Each of the above candidates is checked in order. It is inserted to the candidate list after pruning to the first candidates.

# Experimental results

Common test conditions as specified in [2] are used to verify the coding performance. The table below shows that the result from us exactly matches the result provided by the proponent. It is confirmed that the coding results completely matches the results from the proponent.

Table 2: The experimental result against the anchor (HTM-9.0r1) (with 2 weighting factors)

Table 3: The experimental result against the anchor (HTM-9.0r1) (with 3 weighting factors)

Note the encoding time may be not accurate since the data was collected using the linux clusters.

# Conclusion

The first two test sets of experimental results in the proposal completely match from each other. The cross-check for the first two test sets is confirmed.

# Reference

- K. Zhang, J. An, J.-L. Lin, Y.-L. Chang, S. Lei, “3D-CE4: Results on improved advanced residual prediction” JCT3V-G0064, JCT3V 7th Meeting: San José, US, 11–17 Jan. 2014.

- D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-F1100, 6th Meeting: Geneva, CH, Oct. 2013.