| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0290v3 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0233v3 |
| --- | --- |

| Title: | Joint BoG report on High Level Syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jill Boyce | Email: | jill@vidyo.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG met 11 Jan 2014.    The BoG plans to meet again for revisits, and to review results of side activities.

The BoG met again on 13 Jan 2014, as reflected in the v2 version of this document.

The BoG met again on 14 Jan 2014, as reflected in the v3 version of this document.

# Recommendations

The BoG recommends the following:

- Remove profile_ref_minus1 from the VPS extension, from JCTVC-P0048

- Move video signal information syntax structure earlier in the VPS VUI, from JCTVC-P0076

- Not signal the sps_max_num_reorder_pics[], sps_max_latency_increase_plus1[], and sps_max_dec_pic_buffering_minus1[] syntax elements in the SPS when nuh_layer_id > 0, from JCTVC-P0155.

- Add PPS extension type flags for conditional presence of syntax extensions per extension type, aligned with the SPS extension type flags, from JCTVC-P0166.  Further align the SPS extension type flags syntax between RExt and MV-HEVC/SHVC.

- Modification of derivation of variable NumActiveRefLayerPics, subject to review of the revised text by interested experts, from JCTVC-P0079

- Restrict that end of bitstream NAL shall have nuh_layer_id equal to 0, from JCTVC-P0130

- Add constraint restricting pictures marked as discardable from being present in the temporal or inter-layer RPS, from JCTVC-P0130

The BoG recommends the following activities take place:

- Revisit JCTVC-P0110 in the track, to select between two options to enable no default output layer sets

- Revisit JCTVC-P0262 in the track

- Side activity launched to consider modifications to the VPS extension to remove unnecessary syntax elements and change syntax elements to ue(v) coding, consistent with abandoning a design goal of avoiding ue(v) decoding in the VPS extension

- Side activity launched to classify VPS extension syntax elements per extension(s), to consider per-extension type syntax, including reordering syntax elements to cluster per extension type

The BoG recommends the following to the editors:

- Improve or add definitions in the MV-HEVC and SHVC specifications for layer sets, target output layers, output layer sets, and consider adding explanatory notes

- Delegate to the editors aspects raised by the following contributions: JCTVC-P0052, JCTVC-P0078, JCTVC-P0155, JCTVC-P0181, JCTVC-P0130

The BoG also met 13 Jan.

The BoG recommends the following:

- Add flag in VUI to indicate that all IRAP pictures are IDRs and that all layer pictures in an AU are IDR aligned, from JCTVC-P0068 proposal 1.

- Several minor modifications to the VPS syntax, consistent with eliminating the previous intention to avoid ue(v) parsing in the VPS, as represented in JCTVC-P0306.  Several additional questions are suggested to be discussed in the track:

  - Is the VPS VUI extension offset necessary?  Should there be a mechanism for additional extension of the VPS extension before the VPS VUI?  Consider syntax provided by Y.-K. Wang.

  - Is the VPS extension offset necessary in the VPS?  If so, how to address the start code emulation issue raised in JCTVC-P0125?

The BoG recommends the following actions:

- Revisit in the track JCTVC-P0132 and JCTVC-P0300, regarding alt output layer flag

The BoG also met 14 Jan.

The BoG recommends the following:

- Change alt output layer flag to be signaled within the loop of output layer sets, from JCTVC-P0300

- Add alpha channel information SEI message, from JCTVC-P0123

- Add sub-bitstream property SEI message, from JCTVC-P0204

The BoG recommends the following actions:

- Revisit JCTVC-P0133

- Review JCTVC-P0261, which was not reviewed in BoG, since presenter was unavailable

# Document review details

### High-level syntax and semantics cleanups

#### Video parameter set

JCTVC-P0046 MV-HEVC/SHVC HLS: Additional layer set [T. Ikai, T. Tsukuba, T. Yamamoto (Sharp)]

This contribution presents an additional layer set definition. In the current spec, layer sets are defined by a syntax layer_id_included_flag in VPS. There more number of layers, more bits is needed. Considering subset dependency structure in SHVC and arranged many views in MV-HEVC, the layer sets tend to have similar structure. Thus, the layer set can be defined by a represent layer named key layer. This contribution proposes an additional layer set indication by key layer concept.

Proposes to more efficiently code the layer sets, by moving layer set definitions from the VPS to the VPS extension, by considering the layer dependencies (which are in the VPS extension).  This would mean that HRD parameters associated with the layer sets couldn’t be made available to the v1 decoder.  However, the encoder would be free to use the existing design.

It was noted that the existing profiles do not allow large numbers of views or scalable layers, so the proposed bit savings for the existing profiles would be small.  However, if future profiles were defined which allow more layers, this approach seems promising, and could be incorporated later in a 2nd VPS extension.  Further study.

JCTVC-P0048 MV-HEVC/SHVC HLS: Syntax clean-up of profile, tier and level information [T. Tsukuba, T. Yamamoto, T. Ikai (Sharp)]

This contribution proposes a syntax clean-up of profile, tier and level information in VPS extension by removing a syntax element of profile_ref_minus1. It is asserted that profile_ref_minus1 doesn’t achieve its intention (bit rate saving).

The proposal asserts bit savings in the typical case, and simplification (by removing a syntax element), without losing flexibility.

Agree to remove profile_ref_minus1.

JCTVC-P0052 MV-HEVC/SHVC HLS: VPS extension clean-up [Y. Cho, B. Choi, M. W. Park, J. Y. Lee, H.-C. Wey, C. Kim (Samsung)]

This contribution proposes modification of video parameter set (VPS) extension syntax structure for its intended usage.  It includes several proposals.

Proposal 1. 	Semantics change to profile_ref_minus1.  No action because of decision to remove the syntax element.

Proposal 2.  Asserts that in the current design, if the VPS extension offset is used to avoid parsing the full base VPS, there is a parsing dependency in the VPS extension on NumLayersInIdList.   Related to P0076 proposal #1.

Question if it is really necessary to allow this parsing of these VPS extension syntax elements without parsing of the VPS.  Should we reconsider the design philosophy of avoidance of ue(v) decoding?

Proposal 3. Semantics of sub_layer_dpb_info_present_flag in dpb_size( ) syntax structure

Seems to be editorial.  Delegate to the editors.

JCTVC-P0070 MV-HEVC/SHVC HLS: On video parameter set extension [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

Video parameter set (VPS) is extended in parallel for supporting a particular extension for each profile. The existing syntax elements in the current VPS extension are present in the vps_multilayer_common_extension, while the syntax elements for 3D-HEVC are presented in the vps_3d_extension. Each extension of VPS can be independently included or excluded in video_parameter_set_rbsp, similarly to SPS extension.

Revisit after side activity led by B. Choi to categorize the contents of the VPS extension according to which extension types would require them.

Further discussion after side activity to categorize the contents of the VPS extension by extension type.

It was questioned whether for the VPS extensions, this parallel extension mechanism would avoid the need to parse syntax elements from other unrelated extensions.  While in the SPS parallel extension mechanism, the values of the flags may be restricted by profile.  However, in the VPS, multiple profiles can be represented, including not yet defined profiles, so it is not possible to restrict the values of the flags now.

It was noted that there was considerable overlap between the MV-HEVC and SHVC, so little advantage to dividing them.  Auxiliary pictures also had much commonality.

No action.

JCTVC-P0076 MV-HEVC/SHVC HLS: On VPS extension and VPS VUI [H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI)]

This contribution proposes VPS extension and VPS VUI changes to access ouput layer sets and video signal information without entropy decoding for session negotiation. Proposed changes include that layer sets information are signalled in the VPS extension and the signalling of video signal information in the VPS VUI is moved up to be accessible without entropy decoding.

Proposal 1 related to other reviewed contributions.

Proposal 2 video signal info syntax structure proposed to be moved earlier in VPS VUI.  Agreed.

JCTVC-P0110 MV-HEVC/SHVC HLS: On default output layer sets [K. Ugur, M. M. Hannuksela (Nokia)]

It is asserted that the default output layer set mechanism in the current SHVC/MV-HEVC design is not suitable for various use-cases, such as ROI and view scalability. In addition, for common configurations of SHVC and MV-HEVC, it is asserted that the default output layer set mechanism does not bring any coding efficiency benefit. For these reasons, it is proposed to remove the default output layer set functionality from the SHVC/MV-HEVC design.

The default output layer set in the current design is required to be a valid output layer set, which leads to some overhead.

It was also suggested to add a value to the default_one_target_output_layer_idc to indicate that there are no default output layer sets.

It was remarked that JCTVC-O0109 was related.

Agreed that it should be possible to have no default output layer sets.  BoG suggests to revisit in the track to select between two options: 1) remove the default_one_target_output_layer_idc syntax element, or 2) add an idc value that indicates no defaults.

JCTVC-P0125 MV-HEVC/SHVC HLS: On VPS extension offset and VPS VUI offset [A.K. Ramasubramonian, Hendry, Y.-K. Wang (Qualcomm)]

This contribution discusses some situations where insertion of emulation prevention bytes can cause problems with setting of the values of syntax elements vps_extension_offset and vps_vui_offset. To prevent such situation, it is proposed to take the last bit away from the two syntax elements and mandate the value of the bit to be equal to 1.

In the first revision of this contribution, it is identified that the example related to syntax element vps_extension_offset is not correct and thus the proposed syntax changes to solve the problem is not needed. Changes to the semantics of vps_extension_offset and changes to the syntax and semantics of vps_vui_offset are proposed.

General discussion about the need for avoiding ue(v) coding in the VPS and the need for the vps_vui_offset.

Side activity led by A. Ramasubramonian to study VPS extension to consider if the VPS extension could be cleaned up, given a relaxation of the guideline to avoid ue(v) coding in the VPS extension.  Consider if syntax elements could be removed, or changed from fixed length to ue(v) coding.  Revisit if there is no decision based upon the side activity to remove the vps_vui_offset syntax element.

JCTVC-P0132 MV-HEVC/SHVC HLS: On alt_output_layer_flag [A.K. Ramasubramonian, Y.-K. Wang, Hendry, Y. Chen (Qualcomm)]

At the 15th JCT-VC/6th JCT-3V meetings, the syntax element alt_output_layer_flag was introduced into the SHVC/MV-HEVC specifications. This document highlights several problems that are associated with this flag, including in output of pictures when there are multiple views, multiple target output layers or multiple resolutions across layers, and considerable complexity for the encoders to enable this flag. It is proposed that this flag be removed; if not removed, it is proposed that the identified problems to be fixed.

It was suggested that a middle box removing pictures in an enhancement layer could make the base layer violate the DPB constraints.

Revisit after full specification text is available for the proposal of removing alt_output_layer_flag, and for options to address the identified problem without removal of the alt_output_layer_flag.

Further discussed in the BoG, and provided text was reviewed. See notes under JCTVC-P0300.

JCTVC-P0300 MV-HEVC/SHVC HLS: On alt_output_layer_flag [M. M. Hannuksela (Nokia)] [late] [miss]

The contribution responds to the comments on the alt_output_layer_flag design presented in JCTVC-P0132/JCT3V-G0133.

Two options are presented to modify the current alt_output_layer_flag design:

- Keep the current alt_output_layer_flag syntax.

Derive AltOutputLayerFlag[ outputLayerSetIdx ] for each output layer set. AltOutputLayerFlag[ outputLayerSetIdx ] is equal to alt_output_layer_flag, if there is only one output layer in the output layer set and the output layer has reference layers, and otherwise AltOutputLayerFlag[ outputLayerSetIdx ] is equal to 0.

- Include alt_output_layer_flag for each output layer set that has only one output layer.

Derive AltOutputLayerFlag[ outputLayerSetIdx ] for each output layer set. AltOutputLayerFlag[ outputLayerSetIdx ] is equal to alt_output_layer_flag[ outputLayerSetIdx ], if there is only one output layer in the output layer set and the output layer has reference layers, and otherwise AltOutputLayerFlag[ outputLayerSetIdx ] is equal to 0.

An alternative output layer is proposed to be selected among the direct and indirect reference layers of the output layer. In the current MV-HEVC/SHVC text an alternative output layer could be any layer with smaller nuh_layer_id as that of the output layer. It is asserted that limiting the selection of alternative output layers to the direct and indirect reference layers of the output layer provides better support for including auxiliary picture layers in the same output layer set as quality/spatial scalability layers containing primary pictures.

Additionally, a VPS VUI indication is proposed to indicate whether a bitstream that is formed by removing, from another conforming bitstream, any coded picture that is not used as a reference for prediction for any other picture and is not the only coded picture of an access unit remains conforming.

It was suggested that as an alternative, a VUI indication could be used to indicate not  displaying all of the indicated output layers.

Discussion about if the encoder could easily calculate worst-case DPB parameters in a shared DPB to be able to encode the bitstream such that when individual discardable pictures were removed the DPB constraints were not violated.

If this concern could be addressed, the group was inclined to not use an additional VUI flag, but to require that if the alt output layer flag was set, the bitstream already satisfies the above constraint.  A note in the spec should be added to clarify this.  Option 2 was favored, for per layer set signaling of alt output layer flag.

Revisit after side discussion about calculation of worst-case DPB.

Discussed in BoG after side discussion and modification of the contribution to address the concerns raised.  The VPS VUI indication flag was removed.  Constraints were added.  Definitions were added. Notes were added to clarify when the flag could be set safely without risk of a sub-bitstream extraction resulting in a non-conforming bitstream.

Agree to change alt output layer flag to be signaled within the loop of output layer sets, with above modifications as included in the v2 version of the contribution.

JCTVC-P0136 MV-HEVC/SHVC HLS: Improvements of Video and Picture Parameter Sets [Truong Cong Thang (UoA), Jung Won Kang, Jinho Lee, Hahyun Lee, Jin Soo Choi (ETRI)]

This contribution proposes three improvements to the video and picture parameter sets. The first improvement is a semantic revision of sps_max_sub_layers_minus1. The second is providing a specific temporal_id_nesting_flag for each layer in VPS, so that an extractor/decoder may know the switching-up capability of each layer. The third is providing the information about the number of tiles in VPS, allowing a client to better know a CVS characteristics and thus to better prepare its decoding resources.

Proposal 1 related to sps_max_sub_layers_minus1. Has some parsing issues. Related proposal in JCTVC-P0155.

Proposal 2 related to vps_temporal_id_nesting_flag.  Includes inference rule for sps_temporal_id_nesting_flag which it was suggested could be problematic when SPS sharing is used.  It was suggested that restrictions would be needed so that flexibility only goes in one direction.

Question of interaction with current restriction that if an enhancement layer has an STSA or TSA the reference layers shall also be STSA or TSA, respectively.

Proposal 3, signaling of tile parameters in the VPS.

Has some relation to min_spatial_segmentation_idc, which is sent in the SPS VUI.

Should consider if such signaling should be in the VPS extension rather than the VPS VUI.  Should consider what constraints the value would place on the tile parameters in the PPS.  It was suggested that min and max tile parameters rather than actual values may be more useful.

Further study encouraged.

JCTVC-P0078 MV-HEVC/SHVC HLS: On output_layer_flag [H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI)]

This contribution proposes the following issues for improvement of MV-HEVC Working Draft 6 and SHVC Working Draft 4.

- In decoding process described in sub-clause 8.1, output_layer_flag[lsIdx][i] has to be changed to ouput_layer_flag[TargetOptLayerSetIdx][j]

- For default output layer sets, the inferred value of output_layer_flag[i][j] dependent on syntax element default_one_target_output_layer_idc has to be specified in the working draft text.

Proposal 1 agreed is an editorial fix and is deferred to the editors.  May be addressed in JCTVC-P0192.

Proposal 2 agreed is an editorial fix and is deferred to the editors.  May be addressed in JCTVC-P0192.

JCTVC-P0262 Support for out-of-band signaling in VPS to enable future layer additions [Ajay Luthra, Sam Narasimhan]

The current VPS structure in HEVC requires a change to the VPS (in the underlying video stream) to signal layer specific parameters when a new scalable layer is added to pre-compressed SHVC content. Even though removal of layers is supported currently (with an SEI) addition of new layers without a change to the VPS in underlying video elementary stream is currently not possible. One of the examples of the use case is where lower layer corresponds to 60 fps and higher layer corresponds to 120 fps. Earlier contributions (JCTVC-N0048, JCTVC-K0206 and ISO/IEC WG 11/N 12956) highlighted adding new layers at re-distribution points without a change to the parameters (VPS, SPS, PPS) in the lower layers as a required feature to be included in SHVC specification.

In order to enable this, we propose to add a flag (VPS external means flag) to the VPS syntax to indicate  parameters (such as profile, level, layer sets) for new layers are transmitted via external means rather than in the VPS. MPEG-2 transport streams provide the capability to signal these parameters through a ‘descriptor’ associated with the new layer-specific video stream carried in a separate PID. The proposal uses one of the ‘reserved’ bits in the VPS for this flag.  Based on the setting of VPS external means flag the layer specific parameters are either signaled in-band in the VPS or are sent through external means. With this addition, VPS does not have to be altered at re-multiplexing or re-distribution points when a new layer is added.

It was suggested to have a flag to indicate that video parameter sets are always in band, as a way to notify that sometimes the VPS will be sent out of band.

It was suggested to allow a second VPS extension to include information from further layers.

Revisit in the track.

#### Sequence and picture parameter sets (2)

JCTVC-P0155 MV-HEVC/SHVC HLS: On Sequence Parameter Set [S. Deshpande (Sharp)]

In this document a modification is proposed to syntax and semantics of SPS. It is proposed that the DPB parameters sps_max_num_reorder_pics[], sps_max_latency_increase_plus1[], and sps_max_dec_pic_buffering_minus1[] be signalled in SPS only when nuh_layer_id is equal to 0. Corresponding change is also proposed to the semantics. It is asserted that this aligns the signalling of DPB parameters in SPS with the current sub-DPB operation based on VPS signalled DPB parameters for output layer sets.

Related to JCTVC-P0130 proposal 3.

Agreed to not signal the sps_max_num_reorder_pics[], sps_max_latency_increase_plus1[], and sps_max_dec_pic_buffering_minus1[] syntax elements when nuh_layer_id > 0.

It was suggested to not actually infer the values but to update the section of spec text that would refer to it in the extension to refer to the vps parameters.  Delegate to editors whether to include an inference rule or update the spec text section.

JCTVC-P0181 MV-HEVC/SHVC HLS: On Picture Parameter Set [Y. He, Y. Ye (InterDigital)]

This contribution proposes to constrain the values of nuh_layer_id, TemporalId and pps_pic_parameter_set_id of picture parameter set NAL units to ensure that picture parameter set can be uniquely identified during PPS activation process.

It was remarked that if a slice with nuh_layer_id > 0 refered to a PPS_id which had been sent with a higher value of nuh_layer_id, that would be a non-conforming bitstreams.

It was suggested that the current specification already disallows some of the use cases described in the contribution as problematic.  It was suggested that the editors study the text to see if clarity could be improved.

JCTVC-P0166 Some Syntax Modifications for HEVC Range Extension [Y. Yu, L. Wang Arris)]

Proposal 1 chaired by Y.-K. Wang.

This contribution presents several syntax modifications for HEVC range extension [1]. The first part of this contribution proposes a conditional parsing for PPS extension syntax to allow decoders not to parse unsupported PPS extension syntax elements. In the second part of this contribution, it is proposed to present all transform skip related syntax for range extension at PPS level and all these syntax are conditionally parsed if transform_skip_enabled_flag is enable. In the third part of this contribution, we propose a solution to avoid several unnecessary intra copy block flag check in the current specification. This modification does not involve any coding performance change.

It was noted that it is anticipated that 3D-HEVC will have a PPS extension, and the RExt design currently does have a PPS extension.

It was remarked that it would be desirable to follow the same design principle as used for the SPS extension in the PPS extension. It was remarked that the exact syntax is not aligned between RExt and MV-HEVC/SHVC for the SPS extension currently. Alignment of the RExt, and MV-HEVC/SHVC syntax for the SPS extension should be done, and the PPS extension syntax should follow the same syntax.

Agreed to add this PPS extension syntax, as noted above.

Proposal 2 involves a functional change, which is considered to be outside the scope of the HLS BoG.  Allocate this topic to a different BoG or track.

Proposal 3 is considered to be outside the scope of the HLS BoG.  Allocate this topic to a different BoG or track.

#### Miscellaneous HLS topics (6)

JCTVC-P0047 MV-HEVC/SHVC HLS: On sub-bitstream extraction [T. Tsukuba, T. Yamamoto, T. Ikai (Sharp)]

This contribution proposes a solution to address the issue on sub-bitstream extraction process in SHVC / MV-HEVC. The issue is that VPS with nuh_layer_id equal to 0 is removed in sub-bitstream extraction when a target layer id list does not include nuh_layer_id equal to 0 and thus the extracted sub-bitstream is un-decodable. The proposal is to always include VPS with nuh_layer_id equal to 0 in a sub-bitstream regardless of a target layer id list. It is asserted that the proposed changes can solve the issue.

It was suggested that the current specification requires that any valid sub-bitstream must include the layer with nuh_layer_id equal to 0, so this proposal would not have any impact.

However, if a change was made, e.g. for legal sub-bitstreams of independent non-base layers such as proposed in JCTVC-P0182, where it was not required to include layer 0 in the layer set, this contribution could be relevant.  The revision of JCTVC-P0182 includes a similar proposal regarding inclusion of the VPS in the sub-bitstream.

JCTVC-P0068 MV-HEVC/SHVC HLS: On parameter improvements [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

Syntax elements related to cross layer IRAP alignment, profile-tier-level, view_id, dependency type, phase alignment are improved for SHVC, MV-HEVC and future extensions.

Proposal 1 cross_layer_irap_aligned_flag changed to an idc.  When value 2, says that IDRs are the only IRAP type and that they are already aligned across layers.   Also moved from VPS VUI to VPS extension, and removes slice header syntax.  The use case where this would be useful is not well defined.

Suggestion for a VPS VUI flag to indicate that all IRAP picture are IDRs and that they are all aligned.  Revisit proposal 1, after text for this variation is available.

Proposal 2 suggests changing direct_dep_type_len_minus2 from ue(v) to u(3).

It was remarked that  direct_dep_type_len_minus2 can have 32 values and hence should be u(5).  No action.

Proposal 3 regards view_id_len.  No action.

Proposal 4 profile_tier_level signaling.  No action.

Propoal 5. cross_layer_phase_alignment_flag. No action.

Further discussion in the BoG.

Agreed for the VUI flag to indicate that all IRAP pictures are IDRs and that they are all aligned, to be reflected in the v3 version of the document.

JCTVC-P0079 MV-HEVC/SHVC HLS: comments on MV-HEVC WD 6 and SHVC WD 4 [H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI)]

This document discuss the following issues in MV-HEVC draft text 6 and SHVC draft text 4 and proposes corresponding solutions: 1) derivation of the variable NumActiveRefLayerPics, 2) profile, tier and level information and 3) semantics of vps_number_layer_sets_minus1.

For proposal 1, the second proposed syntax was preferred to the first proposed syntax because it appeared to be simpler.  It was suggested that updated specification text using the second proposed syntax be provided where the refLayerPicFlag[ 0 ] derivation is removed since it appears to no longer be necessary.

Agreed to the second syntax option of the proposal 1 as described above,   Provided text was reviewed by the BoG and was found to be satisfactory.For proposal 2, It was noted that while MV-HEVC or SHVC enhancement layers would not be allowed to use a version 1 profile which the base layer used, auxiliary pictures may be able to use a version 1 profile, depending on decisions about how to signal auxiliary picture profiles.

For proposal 3, defer until side activity reports.  May not be necessary since syntax element may be removed.

JCTVC-P0130 MV-HEVC/SHVC HLS: Miscellaneous HLS topics [A.K. Ramasubramonian, Hendry, Y.-K. Wang, Y. Chen, V. Seregin (Qualcomm)]

This document proposes several changes to the current MV-HEVC/SHVC design. The proposed changes include (a) to restrictict that end of bitstream NAL units shall have nuh_layer_id equal to 0, (b) an indication of cross-layer temporal RPS alignment, (c) removal of a parsing dependency in the SPS syntax, (d) disallowing the presence of discardable pictures in temporal and inter-layer RPS subsets, and (e) fixing a bug in the derivation of inter-layer RPS subsets.

For proposal 1, agreed to restrict end of bitstream NAL unit to nuh_layer_id equal to 0.

For proposal 2, it is most related to 3D-HEVC, so suggested for further study.

For proposal 3, see the notes of JCTVC-P0155.

For proposal 4, adds constraint language to the decoding process: “There shall be no picture in the RPS that has discardable_flag equal to 1” for temporal prediction and “There shall be no picture that has discardable_flag equal to 1 in RefPicSetInterLayer0 or RefPicSetInterLayer1” for inter-layer prediction.

Seems to be consistent with the existing design philosophy.  Without this restriction, if a discardable picture was discarded, a subsequent picture might wrongly conclude that there was a picture loss which could cause a decoding error.  It was remarked that the proposed restrictions help with loss detection.

Agreed to add these two restrictions of proposal 4.

For proposal 5, delegated to editors.

JCTVC-P0306 Side activity A. K. Ramasubramonium

It was suggested that the VPS VUI could be a separate NAL unit with a NUT value, but it was pointed out that there are parsing dependencies in the VPS VUI on the VPS extension.

It was suggested that the VPS VUI could be considered as a VPS extension type.

Question about necessity for VPS VUI offset.  It was noted that there are already parsing dependencies in the VPS VUI on syntax elements on the VPS extension.  It was noted in JCTVC-P0125 that there is a start code emulation problem with the current design of VPS VUI offset.

Additional extension capability could be added after the current VPS extension and before the VPS VUI, which would make having the VPS VUI extension offset more valuable.   It was suggested that the offset may only be needed if the additional extension were used, rather than just the current condition based on if the VPS VUI is present.  Revisit after Y.-K. Wang to provide syntax to add an extension mechanism at the end of the VPS extension and before the VPS VUI.

Question about necessity for VPS extension offset in base VPS.  Reserved bits already exist in the HEVCv1 VPS.  Start code emulation may also be a problem for the VPS extension offset syntax element.

    - Other SEI and VUI (8)

JCTVC-P0120 AHG9: Display hint SEI message for ROI layers [K. Ugur, M. M. Hannuksela (Nokia)] [late]

This contribution proposes a new SEI message for SHVC with the aim of providing information on the intended display behaviour when enhancement layer is used for region of interest coding purposes.  The proposed SEI message indicates how the player should compose one or more enhancement layers with the base layer (e.g.  BL and EL pictures should be overlaid when displayed).

Some non-proponent support expressed. It was noted that such an SEI message could be added at any point during the standardization process, and does not need to be aligned with the first release of SHVC.

Further study encouraged, particularly to describe the use case where distinguishing between the different display conditions for particular applications is useful.

JCTVC-P0123 RExt HLS (AHG5 and 9): SEI message for alpha channel information [M. Naccari, M. Mrak (BBC)]

Alpha channel signals are used in professional (studio) video coding applications and their usage may become popular also for frame composition at the receiver side. Briefly, the alpha channel complements the information contained in the main bitstream by providing, for each image pixel, its degree of transparency. Support for alpha channel in range extensions of HEVC is provided using auxiliary pictures which are transmitted along with primary coded pictures. HEVC will transmit auxiliary pictures as a scalable layer with a particular identifier to specify the type of auxiliary picture (e.g. alpha channel, depth map, chroma enhancement layer, etc.). According to this transmission modality, picture dependent data can be transmitted using SEI messages. This contribution proposes a SEI message to communicate some information related to alpha channels and their usage in frame composition.

Proposes to upsample with signaling of arbitrary upsampling filter coefficients. The proponent noted the upsampling support was requested at the previous BoG discussion for JCTVC-O0132. It was noted that it might be possible to reuse the SHVC resampling filter.

It was noted that the bit depth is already available in the SPS of the auxiliary picture.

Agreed to the SEI message, as modified to remove the upsampling related syntax elements, to be uploaded as v3 version.

JCTVC-P0131 MV-HEVC/SHVC HLS: Layer up-switching information SEI message [A.K. Ramasubramonian, Hendry, Y.-K. Wang (Qualcomm)]

This document proposes an SEI message that signals the information that is asserted to assist layer up-switching. The SEI message signals, when the current layer is the highest layer being decoded, the locations in higher layers where layer up-switching can be performed.

The type of coded picture patterns which would benefit from this information are probably not common.

Counting access units requires maintaining state in the middle box.

It was suggested that access unit counts would be affected by loss or individual discardable pictures.

JCTVC-P0133 MV-HEVC/SHVC HLS: On recovery point and region refresh SEI messages [Hendry, Y.-K. Wang, A. K. Ramasubramonian (Qualcomm)]

This contribution discusses the use of the recovery point SEI message and the region refresh SEI message in the context of multi-layer bitstreams, and proposes some updates to the semantics of these two SEI messages in order to enable them to be used in the multi-layer context without ambiguity.

No syntax changes are proposed.  Semantics changes are backwards compatible with single layer HEVCv1 version.

It was noted that there is more than one possible desirable behavior for this message.  The current semantics are unclear, and there was support to clarify the behavior.

It was suggested to modify the semantics of the recovery point SEI message to indicate layer switching capability when the SEI message applies only to the enhancement layer.  If the SEI message applies to an enhancement layer and all of its direct and indirect reference layers, then the SEI message indicates a random access capability.

Revisit after revised text is available for the above suggestion.

Further study for the region refresh SEI message semantics section to consider how to associate with recovery point SEI message.

JCTVC-P0204 MV-HEVC/SHVC HLS: Sub-bitstream property SEI message [Y. Chen, Y.-K. Wang, A. K. Ramasubramonian, Hendry (Qualcomm)]

In order to correctly decode the target output layers, it is not necessary for each AU to be "complete", i.e. picture from each layer in the bitstream is present in the AU. This allows use cases like picture rate scalability in SHVC as well as optimization of the size of the bitstream by discarding some pictures that are not needed for decoding of pictures of the target output layers, e.g. pictures with discardable_flag equal to 1 in layers that are not target output layers. In JCT3V-F0091/JCTVC-O0273, multi-mode sub-bitstream extraction has been proposed to enable extraction of sub-bitstreams that contain "incomplete" AUs to be confirming bitstreams. At the previous meeting, it was suggested to consider an SEI message to convey properties of such sub-bitstreams instead of making them conforming. This document proposes such an SEI message.

There was some non-proponent support.

It was suggested that the pic_rate_present_flag syntax is not necessary because it is already available in the VPS VUI.  That would allow removal of the bit_rate_present_flag.

It was also suggested to change some fixed length syntax elements to ue(v), and to change the loop count to include a minus1.

Agreed to adopt SEI message, with text revised according to the above suggestions, as expressed in the v2 version.

JCTVC-P0261 Extension of the pic_struct element in HEVC [Alexandros Tourapis, David Singer, Krasimir Kolarov, Steve Saunders] [late]

Presenter not available.