| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0239 |
| --- | --- |

| Title: | JCT3V BoG Report on ATM Software Improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Dong Tian (Coordinator) 201 Broadway, 8th floor Cambridge, MA | Tel: Email: | tian@merl.com +1-617-621-7556 |
| Source: | Mitsubishi Electric Research Labs (MERL) |  |  |

_____________________________

# Abstract

In this BoG, the problems with ATM software Ver 10.0 (SVN REV 334) are identified. A plan to improve the software is drafted for considerations by the JCT3V group. With the plan, it is expected to solve the identified software problems and release an improved version that capable of generating all 3D-AVC conforming bitstreams before Friday, March 14, 2014.

# Introductions

The BoG discussion took place from 0800 on Jan 13 during 7th JCT3V meeting. The attendees and affiliations are listed in Table 1.

Table 1: Attendees & Affiliations

| Attendees | Affiliations |
| --- | --- |
| Dmytro Rusanovskyy | LGE |
| Miska Hannuksela | Nokia |
| Jin Young Lee | Samsung |
| Shinya Shimizu | NTT |
| YuLin Chang | MediaTek |
| Jian-Liang Lin | MediaTek |
| Olgierd Stankiewicz | Poznan University |
| Ying Chen | Qualcomm |
| Dong Tian | MERL |

The mandates of the BoG:

1) Clarify existing issues to generate conforming bitstreams.

2) Draft a software improvement plan with each work item assigned.

# Identified software issues

## DMVP and VSP Coupling Issue

It is reported that VSP does not functional properly and camera parameters are not signaled when DMVP is turned OFF.

Jin Young (Samsung) would take the lead to resolve the DMVP and VSP Coupling problem.

## Inter-view Predictions

By "PIP" inter-view test conditions (CTC), the views are coded in the following sorted order,

- The center view is coded as I view;

- The left view is coded as P view, with center view placed in LIST0;

- The right view is coded as P view, with center view placed in LIST0.

By "IPP" inter-view test conditions, the views are coded in the following sorted order,

- The left view is coded as I view;

- The center view is coded as P view, with left view placed in LIST0;

- The right view is coded as P view, with left and center views placed in LIST0.

By "IBP" inter-view test conditions, the views are coded in the following sorted order,

- The left view is coded as I view;

- The right view is coded as P view, with left view placed in LIST0;

- The center view is coded as B view, with left view placed in LIST0 and right view placed in LIST1.

For "IBP" inter-view prediction, the generated bitstream by the current codec could not be properly decoded. The BVSP direct 8x8 shall be disabled according to the adoption F0268.

Dmytro (LGE) would take the lead to resolve the IBP problem.

## Adaptive Luminance Compensation

ALC has problems with IPP interview prediction, and it may crash the decoder.

ALC also has problems with IBP interview prediction, and it may crash the decoder. ALC is not intended for B slices.

It was observed that ALC implementations depend on DMVP implementations. By turning OFF DMVP, the decoder will crash with ALC ON for PIP, IPP and IBP interview predictions.

Jin Young (Samsung) would take the lead to resolve the ALC related problems.

## High Level Syntax

Syntax elements that exist in the SW but don’t exist in the specification text:

- depth_info_present_flag

- inside_view_mvp_flag

- disp_flag and psip_flag

- texture_voidx_delta

- inloop_jvf_idc and thres_for_jvf

Syntax elements that exist in both SW and specification text but have been modified from recent adoptions:

- nonlinear_depth_representation_num and nonlinear_depth_representation_model (have been moved from seq_parameter_set_3davc_extension to  depth_parameter_set_rbsp)

Other mismatches:

- In the latest specification text JCT3V-F1002, seq_parameter_set_3davc_extension is present only if the profile_idc=139 but in the SW it is present for both profile_idc=139 and 138.

- In the SW, mvc_vui_parameters_present_flag and mvcd_vui_parameters_extension are present after seq_parameter_set_3davc_extension. However, in the F1002 they should be present before seq_parameter_set_3davc_extension (just at the end of the seq_parameter_set_mvcd_extension)

Miska (Nokia) would take the lead to resolve the HLS problems.

## Flexible Depth Resolution Issues

No support for different vertical and horizontal resolution ratio between texture and depth.

No support the input resolution which is not a multiple of 16.

Flexible depth resolution support with VSO – currently only compile-time switch works. Should be implemented as run-time switch.

Nokia has already fixed the issues and need to be crosschecked.

Miska (Nokia) would take the lead to resolve the Flexible Depth Resolution problem.

## VSP Related  Issues (JCT3V-F0268)

Among the issues that were originally reported in JCT3V-F0268, six of them need actions on the current software. The spec text had already been updated according to the solution in the following table, and the software need to be updated accordingly.

Table 2: Issues and solutions related to 3D-AVC software repoted in JCT3V-F0268

| Item | Problems | Suggested solution | Actions |
| --- | --- | --- | --- |
| 3 (DMVP/VSP) | In J.8.2, the depth information are specified only when DMVP is enabled without considering BVSP mode. | In J.8.2, the highlighted part should be added. When DepthFlag is equal to 0 and dmvp_flag or slice_vsp_flag is equal to 1, the variables DepthRefPicList0, DepthRefPicList1 for B slices, and DepthCurrPic are specified as follows. ... | To confirm the SW is up-to-date (Dmytro) |
| 4 (DMVP/VSP) | If DMVP is disabled while BVSP is enabled, 8.4.1 is invoked instead of J.8.2.1. | In J.8.2, the highlighted part should be added. – If nal_unit_type is equal to 21 and DepthFlag is equal to 0, the following applies: – If mb_alc_skip_flag is equal to 1 or mb_alc_flag is equal to 1, subclause Error: Reference source not found is invoked. – Otherwise, if dmvp_flag or slice_vsp_flag is equal to 1, subclause Error: Reference source not found is invoked. – Otherwise, subclause 8.4.1 is invoked. – Otherwise, ... | To confirm the SW is up-to-date (Dmytro) |
| - 5 (VSP Skip Unit Size) | - To decode one VSP PSkip mode, - Sub-clause J.8.2.1.3 (i.e., Derivation process for luma motion vectors for VSP skipped macroblocks in P and SP slices) is invoked four times, however, it always return the same disparity vector. | The depth-based disparity value derivation process in subclause Error: Reference source not found is invoked with depthPic equal to DepthCurrPic, textureX equal to x1 + dx1+ shiftedX, textureY equal to y1 + dy1, tBlWidth equal to 168, tBlHeight equal to 168, ... | SW need to be updated (Dmytro, Jian-Liang) |
| - 6 (VSP Direct 8x8) | - BVSP B_Direct_8x8 mode is not supported in 3D-AVC. | In J.8.2.1, the following changes should be made: vspFlag = !( sub_type[ mbPartIdx ] = = B_Direct_8x8 ¦ ¦ ( mb_type = = B_Skip && MbVSSkipFlag = = 0 ) ¦ ¦ ( ( mb_type = = B_Direct_16x16 ¦ ¦ sub_type[ mbPartIdx ] = = B_Direct_8x8 ) && !mb_direct_type_flag ) ) | SW need to be updated (Dmytro, Jian-Liang) |
| - 7 (VSP Direct 16x16 /DoNBDV) | - To decode one VSP B_Direct_16x16, the MB is set to have four partitions and for each partition, sub-partition size is set to 4x4. However, 8x8 BVSP should be applied. There is no need to invoke the motion vector derivation process 4*4 times. | - Set the sub-partition size equal to 8x8, by the following modified text: In subclause J.8.2, the highlighted part should be added. – Otherwise (mb_type is equal to B_Skip or B_Direct_16x16, or mb_type is equal to B_8x8 and sub_mb_type[ mbPartIdx ] is equal to B_Direct_8x8), the following apply: – If either MbVSSkipFlag or mb_direct_type_flag is equal to 1, subMbPartIdx is set to 0 and partWidth and partHeight are derived as: partWidth = 8 partHeight = 8 – Otherwise (both MbVSSkipFlag and mb_direct_type_flag is equal to 0), subMbPartIdx proceeds over values 0..3, and partWidth and partHeight are derived as: partWidth = 4 partHeight = 4 | Before changes: DoNBDV is on 4x4 blocks. After changes: DoNBDV is on 8x8 SW need to be updated (Ying Chen) |
| 8 (VSP mbPartIdx) | For BVSP coded 16x8/8x16/8x8 (excluding B_direct_8x8), the whole partition share the same motion vector. | In subclasue J.8.2, the highlighted part should be added. The range of the macroblock partition index mbPartIdx is derived as follows: – If mb_type is equal to B_Skip or B_Direct_16x16 or if both mb_type is equal to P_Skip and MbVSSkipFlag is equal to 1, mbPartIdx proceeds over values 0..3. – Otherwise (mb_type is not equal to B_Skip or B_Direct_16x16), set variable vspPartFlag equal to 0, and the following apply: - if one of bvsp_flag_l0[ mbPartIdx ] or bvsp_flag_l1[ mbPartIdx ] is equal to 1, vspPartFlag is set to 1 and mbPartIdx proceeds over values 0..3; - otherwise, mbPartIdx proceeds over values 0..NumMbPart( mb_type ) − 1. For each value of mbPartIdx, the variables partWidth and partHeight for the width and height of each macroblock partition or sub-macroblock partition in the macroblock are derived as follows: – If mb_type is not equal to P_8x8, P_8x8ref0, B_Skip, B_Direct_16x16, or B_8x8, subMbPartIdx is set equal to 0 and the following applies: – If mb_type is equal to P_Skip and MbVSSkipFlag is equal to 1, or vspPartFlag is equal to 1, partWidth and partHeight are derived as: partWidth = 8 (J-1) partHeight = 8 (J-2) – Otherwise, partWidth and partHeight are derived as: partWidth = MbPartWidth( mb_type ) (J-3) partHeight = MbPartHeight( mb_type ) (J-4) | To be confirmed from Qualcomm/Nokia. |

## General improvements

It is realized that there are a lot of obsolete code/MACRO left in the source code. It would be preferred to do some general improvements after the critical items being finished.

# Software improvement plan

The suggested software improvement plan for AhG 4 is listed in Table 3. At the end of Stage I, the software would be able to be used to generate most conforming bitstreams. And at the end of Stage II, the software would be able to be used to generate all conforming bitstreams.

Table 3: Software improvement plan (2 + 4 = 6 weeks)

| Work Item | Volunteer | Cross Checker/Helper | Days |
| --- | --- | --- | --- |
| Stage I (12 + ? days) |  |  |  |
| ALC, Part I (Sec. 2.3) | Samsung (Jin Young Lee) | Dmytro Rusanovskyy | 1 |
| DMVP and VSP decoupling (Sec. 2.1) | Samsung (Jin Young Lee) | Dmytro Rusanovskyy | 3 |
| High Level Syntax (Sec. 2.4) | Nokia (Miska Hannuksela) | Olgierd Stankiewicz | 3 |
| Flexible depth resolution issues (Sec. 2.5) | Nokia (Miska Hannuksela) | Olgierd Stankiewicz | 2 |
| VSP Direct 16x16 /DoNBDV (7 in F0268) | Qualcomm (Ying Chen) | Dong Tian | 3 |
| VSP mbPartIdx (8 in F0268) | Qualcomm (Ying Chen) | Miska Hannuksela | ? |
| VSP Skip Unit Size (5 in F0268) | LGE (Dmytro Rusanovskyy) | Jian-Liang Lin | 2 |
| VSP Direct 8x8 (6 in F0268) | LGE (Dmytro Rusanovskyy) | Jian-Liang Lin | 2 |
| DMVP/VSP (3 and 4 in F0268) | LGE (Dmytro Rusanovskyy) | Jian-Liang Lin | 2 |
| Stage II (20 days) |  |  |  |
| IBP (Sec. 2.2) | LGE (Dmytro Rusanovskyy) | Dong Tian | 10 |
| ALC, Part II (Sec. 2.3) | Samsung (Jin Young Lee) | Dong Tian | 10 |

The software work would start from Monday, January 20, 2014. Some clean up work may be done after the work items in Table 3 are finished. The final software is to be released before Friday, March 14, 2014.