| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G1011 |
| --- | --- |

| Title: | Report of Subjective Test Results from the Call for Proposals on 3D Video Coding Technology |  |  |
| --- | --- | --- | --- |
| Status: | Approved |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vittorio Baroncini, FUB Touradj Ebrahimi, EPFL Anthony Vetro, Mitsubishi Electric | Email: | vittorio@fub.it touradj.ebrahimi@epfl.ch avetro@merl.com |
| Source: | JCT-3V, MPEG Test, MPEG Video |  |  |

_____________________________

- Introduction

This document describes the results of the subjective quality assessment test for AVC-based coding of multiview video and depth. A description of the test conditions and subjective testing methodology as provided in [1] are also provided.

- Test Conditions

  - Input Test Sequences

The multiview test sequences with associated depth data, and corresponding input views that were used for the experiments are specified in the table below.

| Seq. ID | Test Sequence | 2-view input | 3-view input |
| --- | --- | --- | --- |
| S01 | Poznan_Hall2 | 7-6 | 7-6-5 |
| S02 | Poznan_Street | 5-4 | 5-4-3 |
| S03 | Undo_Dancer | 1-5 | 1-5-9 |
| S04 | GT_Fly | 9-5 | 9-5-1 |

  - Encoder Configuration

Two AVC-based codec configurations were considered for the subjective test:

- MVC+D: anchor codec for multiview and depth without block-level changes to decoding process [2]

- 3D-AVC: enhanced compression of multiview and depth with modifications to block-level decoding process for dependent texture views [3]

The encoder configuration settings for both codec are consistent with the CTC as given in JCT3V-E1100, which are also outlined below:

- Inter-view coding structure

  - 3 view case: center-left-right (in coding order)

  - 2 view case: right-left (in coding order)

  - P-I-P inter-view prediction

- Temporal prediction structure: GOP 8, intra every 24 frames (random access at ~1sec)

- Full resolution texture coding

- Anchor software: 3DV-ATM v9.0 (http://mpeg3dv.research.nokia.com/svn/mpeg3dv/tags/)

- Encoder configurations are provided as part of the “Starter-kit” (./configs )

The QP settings of the base view for both texture and depth, as well as the depth resolution for each sequence, are summarized in the below table:

| Seq. ID | Test Sequence | QP Settings | Depth Resolution |
| --- | --- | --- | --- |
| S01 | Poznan_Hall2 | 26, 31, 36, 41 | Quarter |
| S02 | Poznan_Street | 26, 31, 36, 41 | Quarter |
| S03 | Undo_Dancer | 31, 38, 41, 46 | Full |
| S04 | GT_Fly | 26, 31, 36, 41 | Quarter |

  - Output Views

Based on the reconstructed multiview video and associated depth data, which were encoded according to the test conditions specified in this section, two sets of stereo pairs were generated as given in the below table. The first stereo pair consists of 2 synthesized views, while the second stereo pair consists of one original and one synthesized view. Horizontal parallax adjustment was applied to each test sequence according to the shift parameter given in the below table.

| Seq. ID | Test Sequence | Input Views | Output Stereo #1 (2 synthesized views) | Output Stereo #2 (1 original, 1 synthesized) | Shift parameter |
| --- | --- | --- | --- | --- | --- |
| S01 | Poznan_Hall2 | 7-6-5 | (6.25-5.75) | (6-5.5) | 12 |
| S02 | Poznan_Street | 5-4-3 | (4.25-3.75) | (4-3.5) | 16 |
| S03 | Undo_Dancer | 1-5-9 | (4-6) | (5-7) | 20 |
| S04 | GT_Fly | 9-5-1 | (6-4) | (7-5) | 8 |

The view synthesis algorithm that was used for generating synthesized views is the “VSRS-1D-Fast” software (https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware).

- Evaluation Procedure

The different codec configurations were evaluated through formal subjective testing on stereoscopic displays. The evaluation was performed based on the output stereo views as specified in the table of section 2.3.

  - Evaluation Methodology

The ACR test method was used with 5 quality levels, where 5 indicates the highest quality and 1 indicates the lowest quality. The tests were carried out with naïve viewers. The ACR method foresees a timing of the Basic Test Cell (BTC) as shown in figure here below.

Example of ACR test method Basic Test Cell

The message showing the sentence “Vote N” will stand for 5 seconds. The subjects were asked to judge the overall quality of the video to be evaluated, and to express these judgments in terms of the wordings used to define the subjective scale. The test subjects were allowed to express their vote during that time period only.

The quality votes were expressed by the test subjects using paper scoring sheets.

The scoring sheets were done of one numbered box for each BTC (Basic Test Cell); the viewing subjects were instructed to write a number (from 1 to 5) in the box numbered as for the message they have just seen on the screen (see example below).

| 1 |  | 2 |  | 3 |  | 4 |  | 5 |  | 6 |  | 7 |  | 8 |  | 9 |  | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11 |  | 12 |  | 13 |  | 14 |  | 15 |  | 16 |  | 17 |  | 18 |  | 19 |  | 20 |

Example of ACR  Scoring Sheet

The numbering of the boxes allow an easy synchronization with the message present on the screen in a way to avoid uses of wrong scoring boxes. To allow the viewing subjects to manage scoring errors (if any) they were instructed to put a circle around a box filled out with a wrong score (if they realize the error during the voting time) and to write the desired score aside the wrongly filled box.

The whole test was organized in separate test sessions. The design of the test sessions was done as follows: the first part of each session is made of a “stabilization” phase; the stabilization phase included 4 BTC selected from those in that session, taking care of choosing one with high quality one with low quality and two of mid quality. This allowed subjects to create their quality scale in a more clear way before the actual test begins. The presence, at the beginning of each test session, of a stabilization phase is not known to the test subjects.

To reduce contextual effects, the presentation order of the BTCs was randomized applying different permutation for each (group of) subject(s), while the same test sequence was never shown in consecutive BTCs.

Considering the particular care that a 3D visual test requires, the length of the test sessions did not exceed 15 minutes. Moreover, each test session was followed by a resting phase with duration equal or superior to that of the test session. The length and the design of the test sessions were determined by the Test Coordinator in agreement with other experts.

  - Raw Data Processing

Mean Opinion Scores (MOS) Standard Deviation (SD) and Confidence Interval (CI) with 95% precision were computed on the raw data. Overlapping of CI values, centred on the MOS values, indicate that, in a rather acceptable way, no Statistical Significant Difference exists between two test points. Tables with the MOS and CI values for each test points are provided together with graphs. Alternative analysis methods such as T-Student test has also be considered.

  - Test Subjects selection, screening and training

The tests were conducted using naïve subjects only; the subjects were selected among students aged 18 to 30 years old; all test subjects were pre-screened for visual acuity, colour blindness and stereo vision, according to what foreseen by the current standardization literature.

The screening of the test subjects consisted of two steps: a first visual screening (acuity, colour, stereo vision) and a post screening. The post screening was done by means of statistical analysis of the test results, to exclude outliers.

The training phase comprised a short test session using different sequences to those of the actual test but with similar coding conditions to those of the actual test; a limited number of BTCs were used for this phase. The goal of the “training” session was to allow subjects to learn about the task and the timing of the voting. During the training session the test administrators were present and helped subjects in their behaviour; as the training sessions are done, any questions and doubts from subjects were answered. Particular care has been given to explain to the viewers the meaning of the five grades of the numerical scale and to verify their correct use of the scoring scale, looking at the scoring sheets produced during the training session.

  - Test laboratories and equipment

Two test laboratories, EPFL and FUB, provided their support to run the test. The test areas in these laboratories are wide rooms with no reflecting ceiling walls and floor, and are insulated by any external audible and visual pollution. The dimension of the test areas allowed for the following:

- Deep enough to seat subjects at 3H for a 46’ monitor (around two meters)

- Additional room to put the monitor at least at 1 meter form the wall behind the monitor

- Wide enough to seat up to 3 subjects in comfortable way

The wall behind the monitor was illuminated with a uniform light source, placed behind the monitor in a way no light hits on the viewing subjects; the light level did not exceed the 5% of the monitor peak luminance (when functioning as stereo monitor); the light source has to be at 6500 K° and driven with an high frequency switching device (in any case not at power frequency; e.g. 50 or 60 HZ). The ambient illumination was provided only by the light illuminating the wall behind the monitor.

In EPFL, a 46” Hyundai stereo display with passive glasses was used for the subjective evaluation and the tests were conducted with 22 subjects.

In FUB, a 47” LG stereo display with passive glasses was used for the subjective evaluation and the tests were conducted with 24 subjects.

- Results

The results of the subjective tests from both test laboratories are provided in this section.

  - EPFL Results

  - FUB Results

- Conclusions

The subjective test results indicate that the 3D-AVC codec configuration achieves comparable quality relative to the MVC+D codec configuration for all QP settings with approximately 20% less bit rate, on average, as concluded in the objective test results that are reported in JCT3V-F0094 [4].

- References

A. Vetro, T. Ebrahimi, V. Baroncini, FUB, "3D Video Subjective Quality Assessment Test Plan," JCT3V-F1011, Geneva, CH, November 2013.

T. Suzuki, M. M. Hannuksela, Y. Chen, S. Hattori, G. Sullivan, “MVC Extension for Inclusion of Depth Maps Draft Text 6,” JCT3V-C1001, Geneva, CH, January 2013.

M. M. Hannuksela, Y. Chen, T. Suzuki, J.-R. Ohm, G. Sullivan, “3D-AVC Draft Text 8,” JCT3V-F1002, Geneva, CH, November 2013.

D. Tian, A. Vetro, S. Shimizu, “AHG 9: 3D-AVC Coding Results,” JCT3V-F0094, Geneva, CH, November 2013.

___________________