| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G1101 |
| --- | --- |

| Title: | Description of Core Experiment 1 (CE1) on Merge List Construction |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin Jin Heo | Tel: Email: Email: | +886-3-5670766 ext. 25250 jl.lin@mediatek.com jin78.heo@lge.com |
| Source: | CE coordinators |  |  |

_____________________________

- Abstract

This document defines Core Experiment 1 on merge list construction to be performed for the 8th JCT-3V meeting.

- Introduction

The goal of this Core Experiment (CE) is to investigate the methods on merge list construction proposed at the 7th JCT-3V meeting. Tools under test will be evaluated according to their impact on compression efficiency and implementation complexity

- Participants

  - CE1

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| Samsung | Min Woo Park Jin Young Lee | m.w.park@samsung.com jinyoung79.lee@samsung.com | P/C |
| MediaTek | Jian-Liang Lin Yi-Wen Chen Kai Zhang Jicheng An Yu-Lin Chang Yu-Wen Huang | jl.lin@mediatek.com yiwen.chen@mediatek.com kai.zhang@mediatek.com jicheng.an@mediatek.com yulin.chang@mediatek.com yuwen.huang@mediatek.com | P/C |
| Zhejiang University | Yichen Zhang | felixzyc@gmail.com | P/C |
| Sharp | Tomohiro Ikai | ikai.tomohiro@sharp.co.jp | P/C |
| Qualcomm | Hongbin Liu Li Zhang Ying Chen | hongbinl@qti.qualcomm.com lizhang@qti.qualcomm.com cheny@qti.qualcomm.com | C |
| LG | Sunmi Yoo Taesup Kim Junghak Nam Sehoon Yea | sunmi.yoo@lge.com taesup.kim@lge.com junghak.nam@lge.com sehoon.yea@lge.com | C |
| NTT | S. Sugimoto S. Shimizu | sugimoto.shiori@lab.ntt.co.jp shimizu.shinya@lab.ntt.co.jp | C |
| HiSilicon | Yongbing Lin | yblin@huawei.com | C |

- Tools under Test

  - CE1

There are totally two topics to be investigated in CE1. Three categories are listed in the following:

- Merge candidate list simplification.

- Coding efficiency improvement by removal of redundant merge list candidates.

### Merge candidate list simplification

- JCT3V-G0100 3D-CE3: Additional depth-based DV candidate [Samsung]

This contribution proposes to replace the shift DV candidates with the depth-based DV (JCT3V-F0144). It is also propose to remove the two shift DV candidates in the Merge mode list. Removal of additional candidates such as bottom-right memory access should be considered more thoroughly in the CE.

- JCT3V-G0034 CE3-related: Worst case complexity reduction for merge candidate construction [Sharp]

This contribution proposes to reduce worst case complexity for merge candidate derivation process in 3D-HEVC. It is asserted that this complexity increase in the worst case causes narrower bottle neck (in software) or increased implementation cost (in hardware). This proposal omits additional 3D-HEVC candidates in 8x4 and 4x8 PU to reduce the worst case complexity.

- JCT3V-G0099 3D-CE3 related: Simplification of 3D merge list construction [Samsung]

Test1: The proposed method only performs the pruning processes between IVMV-A1/B1 in the texture coding and MPI-A1/B1 in the depth coding, respectively. Hence, the total number of the pruning processes is reduced from 11 to 4.

Test2: Instead of performing the pruning processes between IVMV-A1/B1 in the texture coding and MPI-A1/B1 in the depth coding, respectively, it is proposed to perform the pruning processes between IVMV and the first two available candidates in the HEVC merge list in the texture coding and MPI the first two available candidates in the HEVC merge list in the depth coding, respectively.

- JCT3V-G0104 CE3 related: Result on clean ups of VSP merging candidates [Zhejiang University]

Fixed VSP position in the merge candidate list:

- All the spatial VSP candidate is converted to a DCP candidate;

- Default VSP candidate is not inserted to MCL;

- A VSP candidate is inserted to a fixed position in MCL (only when merge index equals to a certain value N, i.e., N = 3).

### Coding efficiency improvement by removal of redundant merge list candidates

- JCT3V-G0104 CE3 related: Result on clean ups of VSP merging candidates [Zhejiang University]

Test 1 (extra VSP converted to DCP): When checking the VSP flags of spatial neighboring blocks (A1, B1, and B0), only the first candidate with its corresponding block coded by VSP mode is set to a VSP candidate.

Test 2 (VSP pruning): On top of method 1.2, extra VSP candidates are removed from MCL instead of converting to a DCP candidate.

- JCT3V-G0078 3D-CE3 related: Adaptive Disabling Temporal Inter-View Candidates [Samsung]

Test: Adaptive Disabling Temporal Inter-view Motion Candidates:

In the current 3D-HEVC, the illumination compensation is only applied to the coding unit which performs the inter-view prediction. Thus, if the illumination compensation is enabled (i.e. ic_flag is equal to 1), the temporal inter-view motion candidates is proposed to be excluded from the merge list. Although the temporal inter-view motion candidate will be disabled, motion information of the temporal inter-view motion candidate can be used for pruning process.

- JCT3V-G0066 Syntax redundancy removal for illumination compensation [MediaTek]

In the current 3D-HEVC, there is syntax redundancy for illumination compensation (IC) in merge blocks. It is proposed to remove such redundancy without imposing parsing dependency. In the merging candidate construction process, when an ic_flag equal to 1 is received, a candidate that does not employ DCP is considered as unavailable.

- Mandates

  - CE1

The goals of this core experiment are as follows:

- To further investigate the complexity reduction accompanied by a decent worst case complexity analysis

- To further investigate into the coding performance of the removal of redundant merge list candidates

- To further study the coding performance of the remaining one merging candidate in depth coding using a disparity vector and investigate whether it is needed any more.

- Detailed information on the proposals are required to be provided on the following aspects:

- Increase/decrease of worst case number of candidates needed to be inserted into the merge candidate list

- Increase/decrease of memory access

- Increase/decrease of number of condition checks and operations

- Software, Configuration and Evaluation

  - Software

    - CE1

Experiments in CE1 will use the 3D-HTM 10.0 software that is recommended in JCT3V-G1100. Proponents are requested to provide software that can be compiled under Windows and Linux platforms.

  - Test Sequences, Bit Rates and Coding Conditions

The CE will use the test sequences, configuration and conditions that are recommended in JCT3V-G1100. Moreover, proponents and cross checkers are required to provide simulation results for the Random access configuration as specified in JCT3V-G1100.

  - Evaluation of CE Results

- Measure impact on bitrate/PSNR. PSNR shall be calculated for the decoded texture views, relative to original texture views and for the synthesized views relative to uncompressed synthesized views. Use 4-point BD-PSNR and BD-Rate according to common conditions. The anchors will be generated according to common test conditions.

- Measure impact on encoding and decoding runtimes for all test cases relative to the unmodified test model software.

- Report comments of subjective visual quality according to common conditions.

- Timelines

  - CE1

2014/01/24		Finalize CE description

2014/02/10		Release HTM 10.0

2014/03/14		Make source code, simulation results, and draft text available for ALL CE1 participants, not only for the cross-checks.

2014/03/21		Register documents for the 8th JCT-3V meeting

2014/03/21		Upload simulation and cross check results to the JCT-3V document repository

2014/03/29-04/04	Start the 8th JCT-3V meeting

- References

- D. Rusanovksyy, K. Müller, and A. Vetro, " Common Test Conditions of 3DV Core Experiments," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG, JCT3V-G1100, San José, Jan., 2014.