| JJoint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G1103 |
| --- | --- |

| Title: | Description of Core Experiment 3 (CE3) on Entropy Coding Simplifications |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Yu-Lin Chang Hongbin Liu | Tel: Email: Tel: Email: | +886-3-5670766 ext. 25314 yulin.chang@mediatek.com +86-10-57760696 hongbinl@qti.qualcomm.com |
| Source: | CE coordinators |  |  |

_____________________________

- Abstract

This document defines Core Experiment 3 on entropy coding simplifications to be performed for the 8th JCT-3V meeting.

- Introduction

The goal of this Core Experiment (CE) is to investigate the methods on entropy coding simplifications proposed at the 8th JCT-3V meeting. Tools under test will be evaluated according to their impact on compression efficiency and implementation complexity

- Participants

  - CE3

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| MediaTek | Jian-Liang Lin Yi-Wen Chen Kai Zhang Jicheng An Yu-Lin Chang Yu-Wen Huang | jl.lin@mediatek.com yiwen.chen@mediatek.com kai.zhang@mediatek.com jicheng.an@mediatek.com yulin.chang@mediatek.com yuwen.huang@mediatek.com | P/C |
| Qualcomm | Hongbin Liu Li Zhang Ying Chen | hongbinl@qti.qualcomm.com lizhang@qti.qualcomm.com cheny@qti.qualcomm.com | P/C |
| HHI | P. Merkle K. Müller | philipp.merkle@hhi.fraunhofer.de karsten.mueller@hhi.fraunhofer.de | P/C |
| Samsung | Jin Young Lee Min Woo Park | jinyoung79.lee@samsung.com m.w.park@samsung.com | C |
| LGE | Jung Hak Nam | Junghak.nam@lge.com | C |

- Tools under Test

  - CE3

The simplification of entropy coding is to be investigated in CE3. Two categories are listed in the following table: simplification on entropy coding of SDC and DMM. Total five contributions are included.

| Doc. No. | Contributions | Proponent |
| --- | --- | --- |
| Simplification on entropy coding of deltaDC values |  |  |
| JCT3V-G0056 | 3D-CE5: Results on bin reduction for inter SDC residual coding | MediaTek |
| JCT3V-G0060 | 3D-AHG8: Simplification on CABAC contexts for the syntax related to inter SDC, intra SDC and DMM | MediaTek |
| JCT3V-G0124 | CE5 related: Delta DC coding for SDC and DMM modes | Qualcomm |
| Simplification on entropy coding of DMM data |  |  |
| JCT3V-G0062 | 3D-AHG8 related: A simplified method on DMM mode coding in 3D-HEVC | MediaTek |
| JCT3V-G0108 | CE5 related: DMM1 signaling based on intra direction | HHI |

### Test 1: Simplification on entropy coding of deltaDC values

- JCT3V-G0056 3D-CE5: Results on bin reduction for inter SDC residual coding [Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei (MediaTek)]

This contribution proposes three aspects to reduce the bins used for the inter SDC residual coding.

- Binarization and entropy coding of the DC residual: the DC residual is binarized using truncated unary code and only the first bin is coded with context model while the remaining bins are bypass coded.

- Constraint of the DC residual magnitude: constrain the DC residual magnitude to be no larger than 3.

- Constraint of the DC residual magnitude: constrain the DC residual magnitude to be no larger than 5.

 To further study the simplification of the entropy coding and the constraint of deltaDC values

- JCT3V-G0060 3D-AHG8: Simplification on CABAC contexts for the syntax related to inter SDC, intra SDC and DMM [Y.-W. Chen, Y.-L. Chang, J.-L. Lin, Y.-W. Huang, S. Lei (MediaTek)]

This contribution presents the results of simplifications on CABAC for the syntax related to inter SDC, intra SDC and DMM. The proposed simplifications first simplify the parsing process of the flags including depth_dc_flag, inter_sdc_flag and inter_sdc_resi_sign_flag by modifying them from context coding into bypass coding to reduce the number of context coded bins as well as the context models. As for the syntax of absolute residual or DC value (depth_dc_abs and inter_sdc_resi_abs_minus1), in current 3D-HEVC, the prefix codeword bins (13 bins for the worst cases) are context coded by sharing one context. In this contribution, we also propose to simplify the CABAC of depth_dc_abs and inter_sdc_resi_abs_minus1 by only context coding the first bin with single context and bypass coding the remaining bins.

 To further study the simplification of the entropy coding on deltaDC values

- JCT3V-G0124 CE5 related: Delta DC coding for SDC and DMM modes [Q. Yu, H. Liu, Y. Chen, L. Zhang (Qualcomm), S. Ma (PKU)]

This contribution proposes four binarization methods for the delta DC magnitude. Meanwhile, when encoding the delta DC magnitude, bins coded with context model are reduced to be no more than nine.

 To further study the simplification of the entropy coding on deltaDC values

    - Test 2: Simplification on entropy coding of DMM data

- Test 2 is about the tool test on reduction of context coded bins for syntax element wedge_full_tab_idx. Currently the syntax element wedge_full_tab_idx (representing an index in the wedgelet pattern list) is binarized by fixed length coding using 7 to 11 context coded bins depending on the block size. According to proposals JCT3V-G0062 and JCT3V-G0108, the purpose of this tool test is studying possible simplifications by reducing the number of context coded bins.

- JCT3V-G0062 3D-AHG8 related: A simplified method on DMM mode coding in 3D-HEVC [K. Zhang, Y.-W. Chen, J. An, X. Zhang, J.-L. Lin, S. Lei (MediaTek)]

In the current 3D-HEVC, the table index in DMM1 is binarizied as a fixed-length code, and all bins share one context model. This contribution proposed to code these bins in the bypass mode without any context model.

 To further study the simplification of the entropy coding in DMM

- JCT3V-G0108 CE5 related: DMM1 signaling based on intra direction [P. Merkle, K. Müller (HHI)]

Modification of the binarization scheme of wedge_full_tab_idx in the form of a fixed-length prefix with 5 bypass coded bins that represent the direction of the wedgelet and a suffix that represents a refinement index. In JCT3V-G0108 the suffix is fixed-length bypass coded, with the number of bins depending on the prefix and the block size, ranging from 0 to 6. In this tool test the above mentioned modification is studied, including alternative methods for coding the suffix.

 To further study the simplification of the entropy coding in DMM

- Mandates

  - CE3

The goals of this core experiment are as follows:

- To further study the simplification of the entropy coding on deltaDC values [JCT3V-G0056, JCT3V-G0060, JCT3V-G0124]

- To further study the simplification of entropy coding on DMM [JCT3V-G0062, JCT3V-G0108]

- Software, Configuration and Evaluation

  - Software

    - CE3

Experiments in CE3 will use the 3D-HTM 10.0 software that is recommended in JCT3V-G1100 except the experiments with flexible coding order. Proponents are requested to provide software that can be compiled under Windows and Linux platforms.

  - Test Sequences, Bit Rates and Coding Conditions

The CE will use the test sequences, configuration and conditions (CTC and All-Intra) that are recommended in JCT3V-G1100. Moreover, proponents and cross checkers are required to provide simulation results for the Random access configuration as specified in JCT3V-G1100.

  - Evaluation of CE Results

- Measure impact on bitrate/PSNR. PSNR shall be calculated for the decoded texture views, relative to original texture views and for the synthesized views relative to uncompressed synthesized views. Use 4-point BD-PSNR and BD-Rate according to common conditions. The anchors will be generated according to common test conditions.

- Measure impact on encoding and decoding runtimes for all test cases relative to the unmodified test model software.

- Report comments of subjective visual quality according to common conditions.

- Timelines

  - CE3

2014/02/10	Release HTM version 10.0

2014/03/14	Make source code, simulation results and draft text available for all proponents and cross-checkers.

2013/03/21	Register documents for the 8th JCT-3V meeting

2014/03/21	Upload contributions to 8th JCT-3V meeting

2014/03/29-2014/04/04	The 8th JCT-3V meeting

- References

- D. Rusanovksyy, K. Müller, and A. Vetro, "Common Test Conditions of 3DV Core Experiments," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG, JCT3V-G1100, San Jose, Jan., 2014.